| TOWEF0 (Toward Effluent Zero) EVALUATION OF THE EFFECT OF THE IPPC APPLICATION ON THE SUSTAINABLE WASTE MANAGEMENT IN TEXTILE INDUSTRIES |  |  |  | IDENTIFICATION CODE: |  |  | DIS.: |  | Pag.: |  | of pag.: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | IE-302-001 |  |  | PU |  | 1 |  | 9 + An + Att |
|  |  |  |  | PARTNER : |  |  | workpackage: |  |  |  |  |
|  |  |  |  | LARIANA DEPUR (P3) CENTEXBEL (P8) |  |  | WP2 WP8 |  |  |  |  |
|  |  |  |  | External identification code: |  |  |  |  |  |  |  |
|  |  |  |  | Contractual reference: |  |  |  |  |  |  |  |
|  |  |  |  | EVK1-CT-2000-00063 |  |  |  |  |  |  |  |
| TitLE: |  |  |  |  |  |  |  |  |  |  |  |
| "Manual for process identification and data collection in textile industries" |  |  |  |  |  |  |  |  |  |  |  |
| KEY WORDS |  |  |  |  |  |  |  |  |  |  |  |
| Annotations: |  |  |  |  |  |  |  |  |  |  |  |
| This document is in substitution of Deliverable n°4: "Material resources data input of textile industry" |  |  |  |  |  |  |  |  |  |  |  |
| 2 | APR 2002 | Use for LCA highlights |  |  | S. Andreoli I. Devreese |  |  |  |  |  |  |
| 1 | DIC 2001 | Revised with comment of ENEA and JRC |  |  | S. Andreoli I. Devreese |  |  |  |  |  |  |
| 0 | NOV 2001 | Emission |  |  | S. Andreoli I. Devreese |  |  |  |  |  |  |
| REV. | DATE | DESCRIPTION |  |  | EDITING | VALIDATION |  |  |  | APPROVAL. RIF. APP. |  |
| PERIOD COVERED BY REPORT |  |  | SECTION INCLUDED: |  | COORDINETOR NAME: |  |  | PROJECT HOME PAGE: NUMERO: |  |  |  |

Contents:

- Scope.

- General information.

- How to fill "PIDACS".

- Annexes filling schedule.

- Use for LCA activities.

- Annexes list.

Attachments

- Scope

This manual is useful to carry out wp02.02.3 WP02.03.3, WP03.02.8, WP003.03.8 job cards (see PC-100-001), on selected textile industries (WP02.01.3 and WP03.01.8 job cards).

- General Information.

In order to identify textile process and to collect relevant data from textile industries, "PROCESS IDENTIFICATION AND DATA COLLECTION SHEET" (hereinafter called "PIDACS") is the document to be issued, in co-ordination with company contact people.

On "PIDACS", manufacturing information are reported, in mask form, in different sections; moreover, a number of Annexes will be filled and attached.

- How to fill "PIDACS".

(NOTE: all data are non-specific and reported for example only)

1) Notes on data collection.

Collection methodology must be reported before data presentation; a brief summary will describe work  with particular regard to :

    - year to which data refer;

    - collection methodology (interview, meetings,……);

    - type of data collected (measured, calculated, estimated);

    - name of PIDACS compiler and company contact people.

2) General data.

a) Production

Production data, referring to a specific year, shall be reported in terms of processed fabric (yarn, ..), percentage of the total weight, processed linear meters (only fabric) and processed kg. It is very important to indicate weight per linear meter, in order to report production in terms of weight.

Ex:

Reference year: 1999

| Fiber | Type | (%) on total weight | processed linear meter / yr | kg per linear meter | processed kg / yr |
| --- | --- | --- | --- | --- | --- |
| cotton | Fabric | 10 | 12.000 | 0.350 | 4.200 |
| silk | Fabric | 5 | 20.000 | 0.120 | 2.400 |

b) Water use.

Information regarding water are divided in order to define relevant streams (supply, discharge, steam).

b.1) Supplied water

Ex:

Reference year:1999

| Source | Quantity [m3/yr] | Specific Cost [€/m3] | Energy consumption [kWh/ m3] |
| --- | --- | --- | --- |
| Industrial supply | 90.000 | 0,29 |  |
| Own wells | 300 | 2 | 0,100 |
| Potable | 200 | 1,5 |  |

In case of well water supply, energy consumption is reported in order to determine pumping cost; specific cost refers to supply charges.

b.2) Process water and treatment for internal use

Water (treated or not) for internal use (process water) shall be indicated as a "type"; water "type" can be water from one source, or a mix from different sources: it could be a particular discharge stream, that is recovered, treated (or not) and reused, as well. For any type of water, used treatments (one or more) shall be indicate, with quantity and specific cost.

Ex:

Reference year:1999

| Water type | Source | Treatment | Use | Quantity [m3/yr] | Treatment specific cost [€/m3] |
| --- | --- | --- | --- | --- | --- |
| W1 | Industrial | Untreated | Production | 50.000 |  |
| W2 | Industrial | Softening | Production | 25.000 |  |
| W3 | Industrial + wells | Untreated | Production | 45.000 |  |
| W4 | Industrial + wells | Softening | Production | 25.000 |  |
| W5 | Potable | Untreated | Domestic | 5.000 |  |
| W6 | discharge water-D2 (*) | Filtration | Production (printing) | 3.000 |  |

(*): see point b.5) discharged water

b.3) Process water analytic features.

Process water (of any type) is characterised in terms of COD, conductivity, hardness, pH and TSS (other parameters if available).

Ex:

Reference year:1999

| Type | W1 | W2 | W5 |  |  |
| --- | --- | --- | --- | --- | --- |
| T [°C] | 20 | 20 | 23 |  |  |
| Conductivity [mS/cm] | 1500 | 1300 |  |  |  |
| Hardness [°F] | 30 | 2 |  |  |  |
| pH [-] |  |  |  |  |  |
| COD [mg/l] | 30 | 28 |  |  |  |
| TSS [mg/l] |  |  | 0,5 |  |  |
| - - - - - |  |  |  |  |  |
| - - - - - |  |  |  |  |  |
| - - - - - |  |  |  |  |  |

b.4) Steam production.

Water used for steam production shall be reported in terms of type and quantity; also steam Tmax and use (direct or with recovery) have to be reported.

Ex:

Reference year:1999

| Steam type | Water type | Quantity [t/yr] | T max [°C] | Use |
| --- | --- | --- | --- | --- |
| S1 | W1 | 25.000 | 110 | direct |
| S2 | W4 | 20.000 | 130 | with recovery |

b.5) Discharged water.

Discharged water will be indicated as a "type" on the basis of different final destination, even if single streams have different analytical features. If data are available, discharges shall be characterised in terms of temperature, conductivity, hardness, pH, COD, BOD5, TSS, TKN, N-NH4, N-NO3, N-NO2, Ptot, colour (Absorbance 420 nm – 550 nm - 680 nm on 0,01 m optical path) and other parameters as reported in the list below.

Ex:

Reference year:1999

| Type | D1 | D2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Quantity [m3/yr] | 50.000 | 3.000 |  |  |  |
| Final destination | sewer | internal reuse |  |  |  |
| Features: |  |  |  |  |  |
| T [°C] | 28 | 20 |  |  |  |
| Conductivity [mS/cm] | 800 | 250 |  |  |  |
| Hardness [°F] | 12 | 20 |  |  |  |
| pH [-] | 7.2 | 7.0 |  |  |  |
| COD [mg/l] | 2.500 | 100 |  |  |  |
| BOD5 [mg/l] | 100 | 40 |  |  |  |
| TSS [mg/l] | 130 | 30 |  |  |  |
| TKN [mg/l] | 75 | 8 |  |  |  |
| N-NH4 [mg/l] | 60 | 6 |  |  |  |
| N-NO2 [mg/l] | 0 | 0 |  |  |  |
| N-NO3 [mg/l] | 2 | 2 |  |  |  |
| Ptot [mg/l] | 10 | 2 |  |  |  |
| Absorbance 420 nm | 0.850 | 0.080 |  |  |  |
| Absorbance 550 nm | 0.560 | 0.050 |  |  |  |
| Absorbance 680 nm | 0.400 | 0.015 |  |  |  |
| Anionic surf. [mgMBAS/l] | 20 | 3 |  |  |  |
| Non-ionic surf.[mgBiAS/l] | 50 | 6 |  |  |  |
| Cationic surf. [mg/ l] | 5 | 1 |  |  |  |
| Chlorides [mg/l] |  |  |  |  |  |
| Chlorine [mg/l] |  |  |  |  |  |
| AOX [mg/l] |  |  |  |  |  |
| Chrome [mg/l] |  |  |  |  |  |
| Copper [mg/l] |  |  |  |  |  |
| Endocrine activity |  |  |  |  |  |
| Hydrocarbons [mg/l] |  |  |  |  |  |
| Iron [mg/l] |  |  |  |  |  |
| Manganese [mg/l] |  |  |  |  |  |
| Nickel [mg/l] |  |  |  |  |  |
| Zinc [mg/l]ì |  |  |  |  |  |
| Toxic Units (for algae) |  |  |  |  |  |
| Toxic Units (for fish) |  |  |  |  |  |
| Toxic Units (for bacteria) |  |  |  |  |  |
| Toxic Units (for invertebrates) |  |  |  |  |  |

c) Energy consumption.

Ex:

Reference year:1999

| Source | Unit | Use | Quantity | Specific cost [€/ ] |
| --- | --- | --- | --- | --- |
| Methane | Nm3/yr | Heating | 800.000 | 0,20 €/Nm3 |
| Electricity | kWh/yr | Lighting-Production | 2.000.000 | 0,10 €/kWh |

- d) Solid wastes

- Data regarding solid wastes, shall be reported according to company declaration for disposal; wastes are identified with class, with reference to 2001/118/CE decision (L47 dated 16/02/01).

- Ex:

- Reference year:1999

| Type | SW1 | SW2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Description | Packing | Plastic |  |  |  |  |
| Waste class | 150106 | 150102 |  |  |  |  |
| Production [kg/yr] | 6.000.000 | 26.000.000 |  |  |  |  |
| Disposal |  |  |  |  |  |  |
| Disposal cost[€/kg] |  |  |  |  |  |  |

- e) Off-gas emissions.

e1) Identification.

Ex:

Reference year:1999

| Type | Emission source | Flow rate [Nm3/h] | Fumes temperature [°C] | Abatement | Abatement system |
| --- | --- | --- | --- | --- | --- |
| G1 | Torpedo | 350 | 25 | NO |  |
| G2 | Rameuse | 26.000 | 90 | NO |  |

- e2) Analytical features

- If abatement systems are installed, analytical data shall be referred to treated gas emission.

- Ex:

- Reference year:1999

| Type | G1 | G2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NOx [mg/l] |  | 17 |  |  |  |
| SOx [mg/l] |  |  |  |  |  |
| Aldehydes [mg/l] |  |  |  |  |  |
| VOC [mg/l] | 91 |  |  |  |  |
| Ketons [mg/l] |  |  |  |  |  |
| Aromatic hydrocarbons [mg/l] |  |  |  |  |  |
| Chlorine [mg/l] |  |  |  |  |  |
| Ammonia [mg/l] | <0,1 |  |  |  |  |
| Particles [mg/l] |  |  |  |  |  |

f) Departments and working time

In this section, data regarding factory departments shall be reported.

Ex:

Reference year:1999

| Department | Operating days | Daily operating period | Weekly operating period | N° of shifts per days |
| --- | --- | --- | --- | --- |
| General Facilities | 230 | 24 | Mon 6,00-Sat 6,00 | 3 |
| Dyeing | 230 | 24 | Mon 6,00-Sat 6,00 | 3 |

g) Equipment

Each machine or plant used in the process shall be reported, with item number and quantity. Operating mode (continuous or batch) is reported as well.

Installed power, as reported onto the nameplate, is reported in order to determine electricity consumption of each equipment.

Operating years indicate the technology degree used in the factory.

Ex:

Reference year:1999

| Department | Equipment | Item | Quantity | Operating mode | Bath Volume [m3]* | Installed Power [kW] | Absorbed Power [kW] | Operating years |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| General Facilities | Sand filter | SF1 | 1 | continuous |  |  |  |  |
| Preparation | Jigger | J1-2 | 2 | batch | 2 |  |  |  |
| Finishing | Rameuse | R1 | 1 | continuous | 4 |  |  |  |

- volume of bath for batch equipment or volume of starting bath for continuous operating mode.

- Annexes filling schedule

(NOTE: all data are non-specific and reported for example only)

Annexes (hereinafter called An) are a relevant part of the document and it is mandatory to report with "PIDACS".

- An. from 1 to 4 shall be filled after "PIDACS" report is completed:

  - An.1, An.2, An.3 are, respectively, material, energetic, water flow charts (see attached examples).

  - An.4: Production model is the report of textile process; it's a matrix reporting machine operations in terms of run per years (batch process) or operating hours per years (continuous process). It is important to fill column “process” with all the different textile process carried out by the Company (for example: desizing for PET, desizing for viscose, light colour dyeing by reactive dyestuff, dark colour dyeing by reactive dyestuff, light colour dyeing by acid dyestuff, …). For a specific process unit (process carried out by each different equipment), this leads to determine total production in terms of processed fabric for each process unit and for whole process. This document is issued in order to determine relevant processes.

- Each relevant process (An.4) will be evaluated by means of An.5,-a or -b, whether the process unit is batch mode or continuous mode (in this case, one start-up-stop is indicated as "cycle"), respectively: a different sheet is to be filled if any difference in the process regarding water 
and/or reactants quantity or quality is observed, but only one sheet is to be filled if different machines works the same way. Each process is divided into steps relevant features have to be reported (for example: bath temperature-see example attached). For each step, all reactants and products dosage or concentration has to be reported as an input; chemicals are identified by a code, as reported in An.9 (see point d). Water type as well as steam fluxes shall be indicated. Step discharged water have to be reported as an output and characterised by the following parameters: temperature, pH, conductivity, COD, TSS, according to the analytical data of the samples collected (see An.8). Particular process features (i.e: solid wastes production, consumable spare parts, relationships among process parameters) shall be reported in Notes below process description.

- General Facilities department process will be described by means of An.5-c; each process is divided into steps whose relevant features have to be reported: all reactants and products dosage or concentration has to be reported as an input, discharged water as an output. The discharged water has to be characterised by parameters reported in point b).Different output but water shall be reported in different editing. Particular process features (i.e: solid wastes production, consumable spare parts, relationships among process parameters) shall be reported in Notes below process description.

Annexes 5 shall be grouped by department (for example: An.G: Dyeing- Process scheme including 5 pages as An.5-a and 3 pages as An 5-b).

When An.4 report is completed, An.6, An.7, An.8 will be filled (see attached example):

  - An.6 and An.7 are issued in order to focus water consumptions and water discharges referring to kg of processed fabrics (yarns,..). Also, water discharges are characterised in terms of COD (mg/l and kg/yr) for each process unit (see example).

  - An.8 is the report of analytical results for different process unit discharge streams. It will be sampled a number of streams enough to define the features of each streams reported in An.5.

- An.9 is the report of chemicals safety data sheets, that have to be attached: each chemical is here identified by a code (for example: 0019: Acetic acid) that shall be reported in An.5 where the process chemical is used in, is described. Compounds will be also characterized by chemical class according to “Chemical class list” attached (Ex: 0021:…… APEO-containing auxiliary).  Additional information as supplier name, distance from delivery, type of freight shall be reported.

- Use for LCA activities

Data reported on PIDACS can be useful for LCA activities (WP08 and WP09) because LCA selected product (yarn or fabric) is already included in production general description.

A specific production model (An.4) is to be issued for LCA product in order to highlight processes of concern (see example attached).

Where LCA product is included in materials processed by a particular process as described in Annexes 5 (-a or -b), this must be remarked with specific editing and the reference to An.4 reported in Notes below process description.

- Annexes list

  - An.1: Material flow chart;

  - An.2: Energetic flow chart;

  - An.3: Water flow chart;

  - An.4: Production model;

  - An.5a: Batch process scheme

  - An.5b:Continuous process scheme

  - An.5c :General facilities process scheme

  - An.6: Water consumptions

  - An.7: Water discharges

  - An.8: Discharged water analytic data

  - An.9: Chemicals safety data sheets

Attachments

- PIDACS: Process Identification and Data Collection Sheet (model)

- Chemical class list