- TOWEF0

(Toward Effluent Zero)

EVALUATION OF THE EFFECT OF THE IPPC APPLICATION ON THE SUSTAINABLE WASTE MANAGEMENT IN TEXTILE INDUSTRIES

- PROCESS IDENTIFICATION AND

- DATA COLLECTION SHEET

- (WP02.02.3-WP02.03.3)

- Company:   	I02

- N° of pages: 9 + cover

SEPTEMBER 2002

- 1) Notes on data collection.

Reference year: 2000.

Sampling and data collection period: July-September 2002.

Compiler name: Ing. S. Andreoli

Company contact people:

- Dr. Xxxxxx (Production model-Printing-Steaming-Washing-Finishing)

- Sig. Xxxxx (Production model- Printing)

2) General data.

2.a) Production:

Reference year: 2000

| Fibre | Type | %ge on total weight | processed linear meters/yr | kg per linear meter | processed kg/yr |
| --- | --- | --- | --- | --- | --- |
| COTTON (C) | fabric | 16 | 347803 | 0,19 (1) | 66082 |
| SILK (S) | fabric | 16 | 347803 (2) | 0,19 | 66082 |
| SILK (S) | fabric | 1 | 14500 (3) | 0,19 | 2755 |
| WOOL (W) | fabric | 8 | 173901 | 0,19 | 33041 |
| PES (P) | fabric | 20 | 434753 | 0,19 | 82603 |
| POLYACRYLIC (PA) | fabric | 2 | 54344 | 0,19 | 10325 |
| VISCOSE (V) | fabric | 27 | 597785 | 0,19 | 113579 |
| SILK/ACETATE (S/A) | fabric | 10 | 217377 | 0,19 | 41302 |
| TOTAL |  | 100 | 2188266 |  | 415769 |

Notes:

(1):Average weight: 0,190 kg/lm for each fibre (estimation based on different experimental results).

(2):Traditional printing (refer to An. A: Material flow chart).

(3):Ink-jet printing (refer to An. A: Material flow chart).

2.b) Water use:

Reference year: 2000

2.b.1) Supplied water::

| Source | Quantity [m3/yr] | Cost [€/m3] |
| --- | --- | --- |
| Potable water supply | 1500 | 0,41 |
| Own wells | 137575 (1) | 0,27 |
| TOTAL | 139075 |  |

Notes:

(1): Value measured by flow-meter.

2.b.2) Process water and treatment for internal use:

| Water type | Source | Treatment | Use | Quantity [m3/yr] | Treatment specific cost [€/m3] |
| --- | --- | --- | --- | --- | --- |
| W1 | Well water | untreated | Production-Facilities | 131256 (1) |  |
| W2 | Well water | softening | Production-Facilities | 6380 (1) | 1,20 (3) |
| W3 (2) | Potable water | untreated | Domestic use- Fire fighting plant |  |  |

Notes:

(1): Values estimated by mass balances (refer to An. L: Water consumptions);

(2): Not included in mass balances (refer An. C: Water flow chart and An. L: Water consumptions );

(3): Salt consumption cost.

2.b.3) Process water analytic features:

| Source-Type | W1 | W2 | W3 |  |  |
| --- | --- | --- | --- | --- | --- |
| T [°C] | 14 | 14 | 18 |  |  |
| pH [-] | 7,5 | 7,5 | 7,6 |  |  |
| Conductivity [mS/cm] | 0,56 | 0,56 | 0,19 |  |  |
| COD [mg/l] | <10 | <10 | <10 |  |  |
| TSS [mg/l] | 0 | 0 | 0 |  |  |
| Hardness [°F] | 31 | 0,9 |  |  |  |
| Alkalinity [mg/l] | 50,2 | 50,2 |  |  |  |
| Ammonia [mg/l] | <0,05 | <0,05 |  |  |  |
| Nitrite [mg/l] | <0,02 | <0,02 |  |  |  |
| Nitrate [mg/l] | 21,7 | 21,7 |  |  |  |
| Chlorides [mg/l] | 15,1 | 15,1 |  |  |  |
| Sodium [mg/l] | 12 | 12 |  |  |  |
| Potassium [mg/l] | 2 | 2 |  |  |  |
| Calcium [mg/l] | 93,2 | 93,2 |  |  |  |
| Magnesium [mg/l] | 18,5 | 18,5 |  |  |  |
| Sulphates [mg/l] | 50,6 | 50,6 |  |  |  |
| Phosphorous [mg/l] | <0,05 | <0,05 |  |  |  |
| Hexavalent Chrome [mg/l] | <0,005 | <0,005 |  |  |  |
| AOX [mg/l] | <0,001 | <0,001 |  |  |  |
| Oxidability [mg/l] | 0,58 | 0,58 |  |  |  |

Notes:

Source: internal analysis.

b.4) Steam production:

Reference year: 2000

| Steam type | Water type | Quantity [t/yr] | T max [°C] | Use |
| --- | --- | --- | --- | --- |
| S1 | W2 | 2158 (1) | 102 (3) | Direct |
| S2 | W2 | 535 (1) | 165 (3) | Direct |
| S3 | W2 | 225 (2) | 130 (3) | Indirect |

Notes:

(1): Quantity required for fabric steaming

(2): Quantity required for water heating (estimated)

(3): Temperature at the end user.

Evaporator capacity B1: 5000 kg/h.

Evaporator capacity B2: 3000 kg/h.

b.5) Discharged water:

Reference year:2000

| Type | D1(1) | D2 | D3 |  |  |
| --- | --- | --- | --- | --- | --- |
| Quantity [m3/yr] | 132014 |  |  |  |  |
| Final destination | sewer |  |  |  |  |
| Features: |  |  |  |  |  |
| T [°C] |  |  |  |  |  |
| Conductivity [mS/cm] |  |  |  |  |  |
| Hardness [°F] |  |  |  |  |  |
| pH [-] | 9,4 (2) |  |  |  |  |
| COD [mg/l] | 967 (2) |  |  |  |  |
| BOD5 [mg/l] | 212 (2) |  |  |  |  |
| TSS [mg/l] | 301 (2) |  |  |  |  |
| TKN [mg/l] |  |  |  |  |  |
| N-NH4 [mg/l] | 17 (2) |  |  |  |  |
| N-NO2 [mg/l] |  |  |  |  |  |
| N-NO3 [mg/l] |  |  |  |  |  |
| Ptot [mg/l] |  |  |  |  |  |
| Absorbance 420 nm |  |  |  |  |  |
| Absorbance 550 nm |  |  |  |  |  |
| Absorbance 680 nm |  |  |  |  |  |
| Anionic surf. [mgMBAS/l] |  |  |  |  |  |
| Non-ionic surf. [mgBiAS/l] |  |  |  |  |  |
| Cationic surf. [mg/ l] |  |  |  |  |  |
| Chlorides [mg/l] |  |  |  |  |  |
| Chlorine [mg/l] |  |  |  |  |  |
| AOX [mg/l] |  |  |  |  |  |
| Chrome [mg/l] |  |  |  |  |  |
| Copper [mg/l] |  |  |  |  |  |
| Endocrine activity |  |  |  |  |  |
| Hydrocarbons [mg/l] |  |  |  |  |  |
| Iron [mg/l] |  |  |  |  |  |
| Manganese [mg/l] |  |  |  |  |  |
| Nickel [mg/l] |  |  |  |  |  |
| Zinc [mg/l] |  |  |  |  |  |
| Toxic Units (for algae) |  |  |  |  |  |
| Toxic Units (for fish) |  |  |  |  |  |
| Toxic Units (for bacteria) |  |  |  |  |  |
| Toxic Units (for invertebrates) |  |  |  |  |  |

Notes:

(1): Value measured by flow-meter;

(2): Average values (Source: Comodepur SpA)

c) ENERGY CONSUMPTIONS:

Reference year: 2000

| Source | Unit | Use | Quantity | Specific cost [€/ ] |
| --- | --- | --- | --- | --- |
| Methane Gas | m3/yr | Production-Heating | 1421000 | 0,21 €/m3 |
| Electricity | kWh/yr | Production-Facilities | 1587000 | 0,12 €/kWh |

Notes:

d) SOLID WASTES:

Reference year:2000

| Type | SW1 | SW2 | SW3 | SW4 | SW5 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Description | Packing, rags, protection clothes | Plastic packages | Iron and steel | Ionic exchange exhaust resins | Wastes containing printing inks |  |
| Waste class | 150203 | 150102 | 170405 | 190806* | 080308 |  |
| Production [kg/yr] | 34160 | 1850 | 7190 | 1400 | 8 |  |
| Disposal | Incinerator | Reuse | Reuse | Recovery | Incinerator |  |
| Disposal cost[€/kg] | 0,19 | - | - | 0,24 | 2,07 |  |

Notes:

e) OFF-GAS EMISSIONS:

e1) Identification

Reference year:2000

| Type | Emission source | Flow rate [Nm3/h] | Fumes temperature [°C] | Abatement | Abatement system |
| --- | --- | --- | --- | --- | --- |
| G1 | Boiler | 2708 | 176 | NO |  |
| G2 | Washer | 4170 | 32 | NO |  |
| G3 | Thermosetting station n°1 (washing) | 5378 | 98 | NO |  |
| G4 | Rameuse | 10404 | 86 | NO |  |
| G5 | Steamer S1 | 2225 | 99 | NO |  |
| G6 | Thermosetting station n°2 (printing) | 6540 | 110 | NO |  |
| G7 | Steamer S2 | 2073 | 89 | NO |  |
| G8 | Dryer (washing) | 6970 | 30 | NO |  |
| G9 | Thermosetting station n°3 (printing) | 10820 | 141 | NO |  |

Notes:

e2) Analytical features

Reference year:2000

| Type | G1 | G2 | G3 | G4 | G5 | G6 | G7 | G8 | G9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOx [mg/Nm3] | 199 |  | <2 | 4 |  | <2 |  | 4 |  |
| Sox [mg/Nm3] |  |  |  |  |  |  |  |  |  |
| Aldehydes [mg/Nm3] |  | <0,5 | <0,5 | <0,5 | 0,67 | <0,5 | 1,56 | <0,5 |  |
| VOC [mg/m3] |  | <0,1 | <0,3 | <0,1 | 23,6 | 11,4 | 5,2 | <0,1 | <1 |
| Ketons [mg/l] |  |  |  |  |  |  |  |  |  |
| Aromatic hydrocarbons [mg/Nm3] |  |  |  |  |  |  |  |  |  |
| Chlorine [mg/Nm3] |  |  |  |  |  |  |  |  |  |
| Ammonia [mg/Nm3] |  | 0,62 | 2,81 | 1,93 | 6,09 | 1,18 | 19,2 | 0,5 |  |
| Particles [mg/Nm3] |  | 0,13 | <0,1 | <0,1 | 23,1 | <0,1 | 0,62 | 0,21 |  |

Notes:

f) DEPARTMENTS AND WORKING TIME:

Reference year: 2000

| Department | Operating days | Daily operating period | Weekly operating period | N° of shifts per days |
| --- | --- | --- | --- | --- |
| General Facilities | 225 | 18;7 (Sat) | Mon: 05.00 Fri: 23.00 Sat: 05.30-12.30 | - |
| Printing | 225 | 16;8 (Sat) | Mon: 06.00 Fri: 22.00 Sat: 05.30-12.30 | 2;1 |
| Washing | 225 | 16;6 (Sat) | Mon:06.00 - Fri:22.00; Sat:06.00-14.00 | 2;1 |
| Steaming | 225 | 16;6 (Sat) | Mon:06.00 - Fri:22.00; Sat:06.00-14.00 | 2;1 |
| Finishing | 225 | 12 | Mon:07.00-Fri:19.00 | - |

Notes:

Refer to An.A: Material flow chart

g) EQUIPMENT:

Reference year : 2000

| Department | Equipment | Item | Quantity | Operating mode | Bath Volume [m3]* | Installed power [kW] | Absorbed power [kW] | Operating years |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gen. Fac. | Boiler | B1-B2 | 2 | continuous | - | 10 | 9 | 25 |
| Gen. Fac. | I.E. softener | IE1 | 1 | batch | - | 8 | 7,2 | 15 |
| Printing | Mixer | M1-M2 | 2 | batch | - | 3 | 2,7 | 15 |
| Printing | Mixer | M4-M5 | 2 | batch | - | 5,5 | 4,9- | 20 |
| Printing | Automatic repeat table | ART1-ART2-ART3 | 3 | batch | - | 1,5 | 1,35 | 30 |
| Printing | Flat screen printing table | FT1 | 1 | batch | - | 7,5 | 6,75 | 15 |
| Printing | Electronic hand table | ET1-ET2 | 2 | batch | - | 10 | 9 | 20 |
| Printing | Rotary screen printing machine | RM1 | 1 | batch | - | 10 | 9 | 20 |
| Printing | Ink-jet printer | IJP1-IJP2-IJP3 | 3 | batch | - | 1,5 | 1,35 | 3 |
| Printing | Screens washer | SCW1 | 1 | batch | - | 5,5 | 4,95 | 20 |
| Printing | Squeegees washer | SQW1 | 1 | batch | - | 5,5 | 4,95 | 20 |
| Printing | Tubs washer | TW1 | 1 | batch | 0,4 | 3 | 2,7 | 30 |
| Printing | Kids washer | KW1 | 1 | batch | 0,7 |  |  |  |
| Steaming | Steamer | ST1-ST2 | 2 | continuous | - | 22 | 19,8 | 20 |
| Washing | Rope washer | RW1 | 1 | batch | 0,8 | 10 | 9 | 20 |
| Washing | Open width washer | OWW1 | 1 | continuous | 9 | 20 | 18 | 20 |
| Finishing | Rameuse | R1 | 1 | continuous | 0,1 | 33 | 29,7 | 15 |

* volume of bath for batch equipment or volume of starting bath for continuous operating mode

Notes:

Refer to An.B: Energetic flow chart

3) Annexes (all sheets have to be considered as relevant part of the whole document):

- An.A: Material flow chart;

- An.B: Energetic flow chart;

- An.C: Water flow chart;

- An.D: Production model;

- An.E: General Facilities - Process scheme (number of pages: 4);

- An.F: Printing - Process scheme (number of pages: 36);

- An.G : Steaming - Process scheme (number of pages: 3);

- An.H :Washing - Process scheme (number of pages: 16);

- An.I: Finishing - process scheme (number of pages: 3);

- An.L: Water consumptions (number of pages: 6);

- An.M: Water discharges (number of pages: 5);

- An.N: Discharged water analytic data (number of pages: 1);

- An.O: Chemicals safety data sheets (number of pages 1+number of pages 39 as attachments).