- TOWEF0

(Toward Effluent Zero)

EVALUATION OF THE EFFECT OF THE IPPC APPLICATION ON THE SUSTAINABLE WASTE MANAGEMENT IN TEXTILE INDUSTRIES

- PROCESS IDENTIFICATION AND

- DATA COLLECTION SHEET

- (WP02.02.3-WP02.03.3)

- Company:   	I06

- N° of pages: 9 + cover

MARCH 2002

- 1) Notes on data collection.

- Reference year: 2000.

- Sampling and data collection period: January-February 2002.

- Compiler name: Ing. S. Andreoli

- Company contact people:

- Sig. Xxxxxx - Dr. Xxxxxx (Production model-Preparation-Dyeing-Finishing);

- Sig.ra Xxxxxx (General Facilities- Solid wastes-Off-gas emission).

2) General data.

2.a) Production:

Reference year: 2000

| Fibre | Type | %ge on total weight | processed meters | kg per linear meter | processed kg |
| --- | --- | --- | --- | --- | --- |
| FLAX (F) | Fabric | 5,00 | 120000 | 0,215 | 25800 |
| PL (P) | Fabric | 30,00 | 720000 | 0,215 | 154800 |
| CO/PL (CP) | Fabric | 32,50 | 780000 | 0,215 | 167700 |
| FLAX/PL(FP) | Fabric | 16,25 | 390000 | 0,215 | 83850 |
| SILK/PL (SP) | Fabric | 16,25 | 390000 | 0,215 | 83850 |
| TOTAL |  | 100 | 2400000 |  | 516000 |

Notes:

Average weight: 0,215 kg per linear meter.

2.b) Water use:

Reference year: 2000

2.b.1) Supplied water::

| Source | Quantity [m3/yr] | Cost [€/m3] |
| --- | --- | --- |
| Potable water supply | 514 | 1,44 |
| Industrial water supply | 93264 | 0,29 |
| TOTAL | 93778 | 0,30 |

Notes:

2.b.2) Process water and treatment for internal use:

| Water type | Source | Treatment | Use | Quantity [m3/yr] | Treatment specific cost [€/m3] |
| --- | --- | --- | --- | --- | --- |
| W1 | Industrial water | Untreated | Fire fighting plant | - | - |
| W2 | Industrial water | Filtration | Facilities | 93264 |  |
| W3 | Filtered water | Softening | Production | 76251 | 0,045(1) |
| W4 | Softened water | Heating by cooling system | Production |  |  |
| W5 | Potable water | Untreated | Domestic use |  |  |

Notes:

(1): Resins supply; routine upkeep neglectable.

- Refer to An. C: Water flow chart for relevant mass balances.

2.b.3) Process water analytic features:

| Source-Type | W1 | W2 | W3 | W4 | W5 |
| --- | --- | --- | --- | --- | --- |
| T [°C] | 16 | 16 | 18 | 50 | 18 |
| pH [-] | 7,6 | 7,6 | 7,6 | 7,6 | 7,6 |
| Conductivity [mS/cm] | 0,19 | 0,19 | 0,19 | 0,19 | 0,19 |
| COD [mg/l] | <4 | <4 | <4 | 10 | 10 |
| TSS [mg/l] | <5 | 0 | 0 | 0 | 0 |
| Hardness [°F] | 9,8 | 9,8 | <0,4 | - | - |
| Chlorides [mg/l] | 4,1 | 4,1 | 4,1 | - | - |
| Sulphates [mg/l] | 13,3 | 13,3 | 13,3 | - | - |
| Sulphides [mg/l] | <0,1 | <0,1 | <0,1 | - | - |
| Total phosphorous [mg/l] | <0,05 | <0,05 | <0,05 | - | - |
| NO2-N [mg/l] | <0,005 | <0,005 | <0,005 | - | - |
| NO3-N [mg/l] | 1,1 | 1,1 | 1,1 | - | - |
| NH4-N [mg/l] | 0,17 | 0,17 | 0,17 | - | - |
| TKN [mg/l] | 0,33 | 0,33 | 0,33 | - | - |
| Hexavalent chrome[mg/l] | <0,004 | <0,004 | <0,004 | - | - |
| Trivalent chrome [mg/l] | <0,004 | <0,004 | <0,004 | - | - |
| Iron [mg/l] | 0,15 | 0,15 | 0,15 | - | - |
| Copper [mg/l] | <0,005 | <0,005 | <0,005 | - | - |
| Zinc [mg/l] | 0,03 | 0,03 | 0,03 | - | - |
| Lead [mg/l] | <0,03 | <0,03 | <0,03 | - | - |
| Cadmium [mg/l] | <0,002 | <0,002 | <0,002 | - | - |
| MBAS [mg/l] | <0,05 | <0,05 | <0,05 | - | - |
| BiAS [mg/l] | <0,05 | <0,05 | <0,05 | - | - |

Notes:

Source: internal analysis.

2.b.4) Steam production:

| Steam type | Water type | Quantity [t/yr] | T max [°C] | Use |
| --- | --- | --- | --- | --- |
| S1 | W3 | 6050 | 140 | Indirect |

Notes:

Evaporator capacity: 4000 kg/h

2.b.5) Discharged water:

| Type | D1(1) | D2(1) | D3(1) | D4 | D5 |
| --- | --- | --- | --- | --- | --- |
| Quantity [m3/yr] | 85295 | 514 |  |  |  |
| Final destination | Equalization tank | sewer |  |  |  |
| Features (2): | D1 | D2 | D3 | D4 | D5 |
| T [°C] |  |  |  |  |  |
| Conductivity [mS/cm] |  |  |  |  |  |
| Hardness [°F] |  |  |  |  |  |
| pH [-] | 7,06 | 7,6 |  |  |  |
| COD [mg/l] | 931 | 60 |  |  |  |
| BOD5 [mg/l] | 255 | 40 |  |  |  |
| TSS [mg/l] | 140 | 12 |  |  |  |
| TKN [mg/l] | 15 |  |  |  |  |
| N-NH4 [mg/l] | 0 |  |  |  |  |
| N-NO2 [mg/l] |  |  |  |  |  |
| N-NO3 [mg/l] |  |  |  |  |  |
| Ptot [mg/l] |  |  |  |  |  |
| Absorbance 420 nm |  |  |  |  |  |
| Absorbance 550 nm |  |  |  |  |  |
| Absorbance 680 nm |  |  |  |  |  |
| Anionic surf. [mgMBAS/l] |  |  |  |  |  |
| Non-ionic surf. [mgBiAS/l] |  |  |  |  |  |
| Cationic surf. [mg/ l] |  |  |  |  |  |
| Chlorides [mg/l] |  |  |  |  |  |
| Chlorine [mg/l] |  |  |  |  |  |
| AOX [mg/l] |  |  |  |  |  |
| Chrome [mg/l] |  |  |  |  |  |
| Copper [mg/l] |  |  |  |  |  |
| Endocrine activity |  |  |  |  |  |
| Hydrocarbons [mg/l] |  |  |  |  |  |
| Iron [mg/l] |  |  |  |  |  |
| Manganese [mg/l] |  |  |  |  |  |
| Nickel [mg/l] |  |  |  |  |  |
| Zinc [mg/l] |  |  |  |  |  |
| Toxic Units (for algae) |  |  |  |  |  |
| Toxic Units (for fish) |  |  |  |  |  |
| Toxic Units (for bacteria) |  |  |  |  |  |
| Toxic Units (for invertebrates) |  |  |  |  |  |

Notes:

- (1): D1: Industrial discharge;

- D2: Domestic discharge.

- Only D1 is used for mass balances (Refer to An. C: Water flow chart ).

- (2): Year average values (Source: Lariana Depur Spa)

2.c) Energy consumptions:

Reference year: 2000

| Source | Unit | Use | Quantity | Specific cost [€/ ] |
| --- | --- | --- | --- | --- |
| Methane gas | m3/yr | Production-Heating | 923000 | 0,22 €/m3 |
| Electricity | kWh/yr | Services | 1848000 | 0,10 €/kWh |

- Notes:

Refer to An. B: Energetic flow chart.

2.d) Solid wastes:

Reference year: 2000

| Type | SW1 | SW2 | SW3 | SW4 | SW5 | SW6 | SW7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Description | Packing | Iron and steel | Transformers and condensers containing PCB and PCT | Plastic | Exhaust resins | Wastes containing Hg | Electrical disused equipment |
| Waste class | 150106 | 170405 | 160209* | 150102 | 190806* | 200121* | 160214 |
| Production [kg/yr] | 25390 | 10840 | 7910 | 5450 | 1980 | 440 | 860 |
| Disposal | Reuse | Reuse | - | Reuse | - | - | - |
| Disposal cost [€/kg] | 0,13 | - | 0,73 | 0,01 | 0,40 | 1,20 | 0,48 |

- Notes:

- Source: M.U.D declaration

2.e) Off-gas emissions:

2.e.1) Identification:

Reference year: 2000

| Type | Emission source | Flow rate [Nm3/h] | Fumes temperature [°C] | Abatement | Abatement system |
| --- | --- | --- | --- | --- | --- |
| G1 | R161 | 15790 | 110 | NO | - |
| G2 | R163 | 9350 | 125 | NO | - |
| G3 | T151 | 300 | 24 | NO | - |
| G4 | J144 | 70 | 57 | NO | - |
| G5 | T153-154 | 920 | 24 | NO | - |
| G6 | Thermosetting station 175 | 135 | 21 | NO | - |
| G7 | Welding table | 990 | 22 | NO | - |

- Notes:

Source: "Regione Lombardia" emissions authorization request.

2.e.2) Analytical features:

Reference year: 2000

| Type | G1 | G2 | G3 | G4 | G5 | G6 | G7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VOC [mg/Nm3] | 3,4 | 4,9 |  |  |  | <1 |  |
| Aldehydes [mg/Nm3] | <0,1 | <0,1 |  |  |  |  |  |
| Ammonia [mg/Nm3] |  |  | <0,1 | <0,1 | <0,1 |  |  |
| Chloridric acid [mg/Nm3] |  |  | <0,5 | <0,5 | <0,5 |  |  |
| Acetic acid [mg/Nm3] |  |  | <5 |  | <5 |  |  |
| Particles [mg/Nm3] |  |  |  |  |  |  | 1,4 |

- Notes:

Source: "Regione Lombardia" emissions authorization request.

2.f) Departments and working time:

Reference year: 2000

| Department. | Operating days | Daily operating period | Weekly operating period | N° of shifts per days |
| --- | --- | --- | --- | --- |
| General Facilities | 220 | 24 | Mon:6.00-Sat:6,00 | 3 |
| Preparation | 220 | 24 | Mon:6.00-Sat:6,00 | 3 |
| Dyeing | 220 | 24 | Mon:6.00-Sat:6,00 | 3 |
| Finishing | 220 | 24 | Mon:6.00-Sat:6,00 | 3 |

- Notes:

- Refer to An. A: Material flow chart.

2.g) Equipment:

Reference year: 2000

| Department | Equipment | Item | Quantity | Operating mode | Bath Volume [m3]* | Installed power [kW] | Absorbed power [kW] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| General. Facilities | Sand filter | SF1-SF2 | 2 | continuous | - | - | - |
| General. Facilities | I. E. softener | ES1-ES2 | 2 | batch | - | 8,5 | 6,8 |
| General. Facilities | Boiler | B1-B2 | 2 | continuous | - | 9 | 7 |
| General. Facilities | Conditioning group | C1 | 1 | continuous | - | 10,3 | 8,2 |
| General. Facilities | Conditioning group | C2 | 1 | continuous | - | 11,8 | 9,5 |
| Prep/Dyeing | Torpedo | T151 | 1 | batch | 2,5 | 31 | 22 |
| Prep/Dyeing | Torpedo | T153 | 1 | batch | 2,3 | 39 | 27 |
| Prep/Dyeing | Torpedo | T154 | 1 | batch | 0,8 | 14 | 11,2 |
| Prep/Dyeing | Torpedo | T155 | 1 | batch | 2,5 | 39 | 27 |
| Prep/Dyeing | Jigger | J144 | 1 | batch | 2 | - | - |
| Prep/Dyeing | Jigger | J145 | 1 | batch | 0,8 | - | - |
| Prep/Dyeing | Comby | C146 | 1 | batch | 0,6 | 28 | 22,4 |
| Prep/Dyeing | Overflow | O192 | 1 | batch | 4 | 11 | 9,35 |
| Prep/Dyeing | Overflow | O193 | 1 | batch | 1,8 | 11 | 9,35 |
| Prep/Dyeing | Overflow | O194 | 1 | batch | 1,8 | 11 | 9,35 |
| Finishing | Rameuse | R161 | 1 | continuous | 0,1 | - | 50 |
| Finishing | Rameuse | R163 | 1 | continuous | 0,1 | - | 70 |

* volume of bath for batch equipment or volume of starting bath for continuous operating mode

- Notes:

3) Annexes (all sheets have to be considered as relevant part of the whole document):

- An. A: Material flow chart;

- An. B: Energetic flow chart;

- An. C: Water flow chart;

- An. D: Production model;

- An. E: General Facilities - Process scheme (number of pages: 5);

- An. F: Preparation - Process scheme (number of pages: 41);

- An. G : Dyeing - Process scheme (number of pages: 48);

- An. H : Finishing - Process scheme (number of pages: 8);

- An. I: Water consumptions (number of pages: 11);

- An. L: Water discharges (number of pages:11);

- An. M: Discharged water analytic data (number of pages: 2);

- An. N: Chemicals safety data sheets (number of pages:2+number of sheets:51 as attachments).