- TOWEF0 -

(Toward Effluent Zero)

EVALUATION OF THE EFFECT OF THE IPPC APPLICATION ON THE SUSTAINABLE WASTE MANAGEMENT IN TEXTILE INDUSTRIES

PROCESS IDENTIFICATION AND

DATA COLLECTION SHEET

WP02.02.3-WP02.03.3

WP03.02.8-WP03.03.8

Company: I09

Address:   xxxxxxxxxxxxx

xxxxxxxxxxxxx

N° of pages: 10 (including this one)

JULY 2002

1) Notes on data collection.

- Reference year: 2000.

- Sampling and data collection period: April-June 2002

- Compiler name: Ing. S. Andreoli

- Company contact people:

- Sig. Xxxxx (Production model - Preparation - Dyeing - Finishing);

- Ing. Xxxxx (General Facilities - Solid wastes - Off-gas emission).

2) General data.

a) Production:

Reference year:2000

| Fiber | Type | (%) of total weight | processed linear meters/yr | kg per linear meter | processed kg/yr |
| --- | --- | --- | --- | --- | --- |
| SILK (SY) | yarn | 69,5 |  |  | 750792 |
| SILK (SF) | fabric | 24 | 1736797 | 0,15 (1) | 260519 |
| PES (PY) | yarn | 1,3 |  |  | 13172 |
| ACETATE (PY) | yarn | 1,3 |  |  | 13172 |
| NYLON (NY) | yarn | 1,3 |  |  | 13172 |
| SILK/OTHER (SOF) | fabric | 2,7 | 192977 | 0,15 (1) | 28946 |
| TOTAL: |  | 100 | 1929774 |  | 1079773 |

Notes:

(1):Average weight silk-silk/other fabric: 0,15 kg/lm. (determination based on laboratory tests)

b) Water use:

b.1) Supplied water:

Reference year:2000

| Source | Quantity [m3/yr] | Specific Cost [€/m3] | Energy consumption [kWh/m3] |
| --- | --- | --- | --- |
| Potable water supply | 920 | 0,52 |  |
| Industrial water supply | 291568 | 0,29 |  |
| TOTAL: | 292488 |  |  |

Notes:

b.2) Process water and treatment for internal use:

Reference year: 2000

| Water type | Source | Treatment | Use | Quantity [m3/yr] | Treatment specific cost [€/m3] |
| --- | --- | --- | --- | --- | --- |
| W1 | Industrial water | Untreated | Fire fighting plant | - |  |
| W2 | Industrial water | Filtration | Production-Facilities | 291568 (1) | - |
| W3 | Filtered water | Softening | Production-Facilities | 205300 (1) | 0,10 |
| W4 | Softened water | Disinfection | Production - Facilities | 198800 | 0,003 |
| W5 | Disinfected water | Heating by cooling systems | Production |  | - |
| W6 | Filtered water | Untreated | Production - Facilities | 86268 | - |
| W7 | Potable water | Untreated | Domestic use-Fire fighting plant | - |  |

Notes:

(1): Values measured by flow-meter (Refer to An.C: Water flow chart)

b.3) Process water analytic features:

Reference year:2000

| Type | W1 | W2 | W3 | W4 | W5 | W6 | W7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T [°C] | 11 | 11 | 11 | 11 | 25 | 11 | 18 |
| pH [-] | 7,6 | 7,6 | 7,6 | 7,6 | 7,6 | 7,6 | 7,6 |
| Conductivity [mS/cm] | 0,19 | 0,19 | 0,19 | 0,19 | 0,19 | 0,19 | 0,19 |
| COD [mg/l] | <4 | <4 | <4 | <4 | <4 | <4 | 10 |
| TSS [mg/l] | <5 | 0 | 0 | 0 | 0 | 0 | 0 |
| Hardness [°F] | 9,8 | 9,8 | 0,5 | 0,5 | 0,5 | 9,8 | 18 |
| Chlorides [mg/l] | 4,1 | 4,1 | 4,1 | 4,1 | 4,1 | 4,1 |  |
| Sulphates [mg/l] | 13,3 | 13,3 | 13,3 | 13,3 | 13,3 | 13,3 |  |
| Sulphides [mg/l] | <0,1 | <0,1 | <0,1 | <0,1 | <0,1 | <0,1 |  |
| Total phosphorous [mg/l] | <0,05 | <0,05 | <0,05 | <0,05 | <0,05 | <0,05 |  |
| NO2-N [mg/l] | <0,005 | <0,005 | <0,005 | <0,005 | <0,005 | <0,005 |  |
| NO3-N [mg/l] | 1,1 | 1,1 | 1,1 | 1,1 | 1,1 | 1,1 |  |
| NH4-N [mg/l] | 0,17 | 0,17 | 0,17 | 0,17 | 0,17 | 0,17 |  |
| TKN [mg/l] | 0,33 | 0,33 | 0,33 | 0,33 | 0,33 | 0,33 |  |
| Hexavalent chrome[mg/l] | <0,004 | <0,004 | <0,004 | <0,004 | <0,004 | <0,004 |  |
| Trivalent chrome [mg/l] | <0,004 | <0,004 | <0,004 | <0,004 | <0,004 | <0,004 |  |
| Iron [mg/l] | 0,15 | 0,15 | 0,15 | 0,15 | 0,15 | 0,15 |  |
| Copper [mg/l] | <0,005 | <0,005 | <0,005 | <0,005 | <0,005 | <0,005 |  |
| Zinc [mg/l] | 0,03 | 0,03 | 0,03 | 0,03 | 0,03 | 0,03 |  |
| Lead [mg/l] | <0,03 | <0,03 | <0,03 | <0,03 | <0,03 | <0,03 |  |
| Cadmium [mg/l] | <0,002 | <0,002 | <0,002 | <0,002 | <0,002 | <0,002 |  |
| MBAS [mg/l] | <0,05 | <0,05 | <0,05 | <0,05 | <0,05 | <0,05 |  |
| BiAS [mg/l] | <0,05 | <0,05 | <0,05 | <0,05 | <0,05 | <0,05 |  |

Notes:

b.4) Steam production:

Reference year:2000

| Steam type | Water type | Quantity [t/yr] | T max [°C] | Use |
| --- | --- | --- | --- | --- |
| S1 | W4 | 14890 | 280 (1) | Indirect |

Notes:

(1): Temperature at the end of the evaporator

Evaporator capacity: B1=B2= 15000 kg/h

b.5) Discharged water:

Reference year:2000

| Type | D1 (1) | D2(2) | D3(2) | D4(2) | D5(2) | D6(2) (3) |
| --- | --- | --- | --- | --- | --- | --- |
| Quantity [m3/yr] | 267619 | 89151 | 5628 | 4922 | 171991 | 703 |
| Final destination | Sewer | Water heating system | Membrane ultra filtration | Screening | Screening | Sericin recovery system |
| Features: |  |  |  |  |  |  |
| T [°C] | 35 (4) | 85 | 100 | 80 | 21 |  |
| Conductivity [mS/cm] |  |  | 0,883 | 0,702 |  |  |
| Hardness [°F] |  |  |  |  |  |  |
| pH [-] | 5,5 (4) |  | 7,26 | 6,81 |  |  |
| COD [mg/l] | 1600 (4) | 3210 | 17880 | 8400 | 930 |  |
| BOD5 [mg/l] | 667 (4) |  |  |  |  |  |
| TSS [mg/l] | 127 (4) |  |  |  |  |  |
| TKN [mg/l] | 75,6 (4) |  |  |  |  |  |
| N-NH4 [mg/l] |  |  |  |  |  |  |
| N-NO2 [mg/l] |  |  |  |  |  |  |
| N-NO3 [mg/l] |  |  |  |  |  |  |
| Ptot [mg/l] |  |  |  |  |  |  |
| Absorbance 420 nm |  |  |  |  |  |  |
| Absorbance 550 nm |  |  |  |  |  |  |
| Absorbance 680 nm |  |  |  |  |  |  |
| Anionic surf. [mgMBAS/l] |  |  |  |  |  |  |
| Non-ionic surf. [mgBiAS/l] |  |  |  |  |  |  |
| Cationic surf. [mg/ l] |  |  |  |  |  |  |
| Chlorides [mg/l] |  |  |  |  |  |  |
| Chlorine [mg/l] |  |  |  |  |  |  |
| AOX [mg/l] |  |  |  |  |  |  |
| Chrome [mg/l] |  |  |  |  |  |  |
| Copper [mg/l] |  |  |  |  |  |  |
| Endocrine activity |  |  |  |  |  |  |
| Hydrocarbons [mg/l] | 9,8 |  |  |  |  |  |
| Iron [mg/l] |  |  |  |  |  |  |
| Manganese [mg/l] |  |  |  |  |  |  |
| Nickel [mg/l] |  |  |  |  |  |  |
| Zinc [mg/l] | 0,52 |  |  |  |  |  |
| Toxic Units (for algae) |  |  |  |  |  |  |
| Toxic Units (for fish) |  |  |  |  |  |  |
| Toxic Units (for bacteria) |  |  |  |  |  |  |
| Toxic Units (for invertebrates) |  |  |  |  |  |  |

Notes:

(1): Value measured by flow-meter including domestic water discharges (W7)-(refer to An.C: Water flow chart): industrial water discharge =D1-W7 = D2+D4+D5

(2): Values extimated by mass balances (refer to An.L: Water discharges)

(3): Membrane concentrate (not included in mass balances and not included in solid wastes declaration).

(4): Average values  (Source: Comodepur SpA)

c) ENERGY CONSUMPTIONS:

Reference year:2000

| Source | Unit | Use | Quantity | Specific cost [€/ ] |
| --- | --- | --- | --- | --- |
| Methane Gas | m3/yr | Production-Heating | 1320000 | 0,23 €/m3 |
| Electricity | kWh/yr | Facilities | 2819000 | 0,1 €/kWh |

Notes:

d) SOLID WASTES:

Reference year:2000

| Type | SW1 | SW2 | SW3 | SW4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Description | Packing | Paper | Iron-materials | Iron |  |  |
| Waste class | 150106 | 150101 | 120102 | 160117 |  |  |
| Production [kg/yr] | 71800 | 44200 | 4900 | 3300 |  |  |
| Disposal | Incinerator | Recovery | Recovery | Recovery |  |  |
| Disposal cost[€/kg] | 0,135 | 0,035 | - | - |  |  |

Notes:

e) OFF-GAS EMISSIONS:

e1) Identification

Reference year:2000

| Type | Emission source | Flow rate [Nm3/h] | Fumes temperature [°C] | Abatement | Abatement system |
| --- | --- | --- | --- | --- | --- |
| G1 | Boiler B1 | - | 158 | NO |  |
| G2 | Colour kitchen | 1160 | 31 | NO |  |
| G3 | STALAM dryer | 3560 | 23 | NO |  |
| G4 | Drying chambers | 3700 | 40 | NO |  |
| G5 | KRANTZ Rameuse | 5800 | 68 | NO |  |
| G6 | KRANTZ dryer | 6140 | 100 | NO |  |
| G7 | FAMATEX Rameuse | 4700 | 90 | NO |  |
| G8 | Turbang | 860 | 80 | NO |  |
| G9 | Airo1000 | 1700 | 24 | NO |  |

Notes:

e2) Analytical features

Reference year:2000

| Type | G1 | G2 | G3 | G4 | G5 | G6 | G7 | G8 | G9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOx [mg/Nm3] | 180 |  |  |  |  |  |  |  |  |
| CO [mg/Nm3] | 21,9 |  |  |  |  |  |  |  |  |
| Aldehydes [mg/Nm3] |  |  |  |  | 0,5 |  | <0,1 |  |  |
| VOC [mg/Nm3] |  |  |  |  | <1,0 | <2 | <1 |  |  |
| Acetic acid [mg/Nm3] |  |  | <5,0 | <5,0 |  |  |  |  |  |
| Formic acid [mg/Nm3] |  |  | <5,0 | <5,0 |  |  |  |  |  |
| Ammonia [mg/Nm3] |  |  |  |  | <0,1 |  |  |  |  |
| Particles [mg/l] |  | 0,7 | 1,2 | <0,5 | 1,4 | 2,7 |  | <0,5 | <0,5 |

Notes:

f) DEPARTMENTS AND WORKING TIME:

Reference year: 2000

| Department | Operating days | Daily operating period | Weekly operating period | N° of shifts per days |
| --- | --- | --- | --- | --- |
| General facilities | 220 | 24-16 | Mon 06.00- Sat 06.00 | - |
| Preparation | 220 | 24-16 | Mon 06.00- Sat 06.00 | 3 (yarn)- 2 (fabric) |
| Dyeing | 220 | 24-16 | Mon 06.00- Sat 06.00 | 3 (yarn)- 2 (fabric) |
| Finishing | 220 | 24-16 | Mon 06.00- Sat 06.00 | 3 (yarn)- 2 (fabric) |

Notes:

Refer to An.A: material flow chart.

g) EQUIPMENT:

Reference year: 2000

| Department | Equipment | Item | Quantity | Operating mode | Bath Volume [m3]* | Installed power [kW] | Absorbed power [kW] | Operating years |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gen. Fac | Sand filter | SF1-SF2 | 2 | continuous | - | 7,5 | 6 | 4 |
| Gen. Fac | I. E softener | IE1-IE2-IE3 | 3 | batch | - | - | - | 16 |
| Gen. Fac | Disinfection | DS1 | 1 | continuous | - | 1,2 | 1,14 | 2 |
| Gen. Fac | Boiler | B1-B2 | 2 | continuous | - | 50 | 45 | 2 |
| Gen. Fac | Ultra-filtration | UF1 | 1 | batch | - | 7,5 | 5 | 4 |
| Prep/Dyeing/Finishing | BT176 autoclave | 102-103 | 2 | batch | 1 | - | 4 | 0,33 |
| Prep/Dyeing/Finishing | BT176 autoclave | 104-105-106-107 | 4 | batch | 1,05 | - | 4 | 4 |
| Prep/Dyeing/Finishing | BT352 autoclave | 203-204 | 2 | batch | 2 | - | 4 | 33-44 |
| Prep/Dyeing/Finishing | BT600 autoclave | 301-302 | 2 | batch | 4 | - | 20 | 41 |
| Prep/Dyeing/Finishing | BT64 autoclave | 31-32-33-34 | 4 | batch | 0,45 | - | 4 | 32 |
| Prep/Dyeing/Finishing | BT112 autoclave | 51-52 | 2 | batch | 0,65 | - | 5 | 32 |
| Prep/Dyeing/Finishing | BT1040 autoclave | 601 | 1 | batch | 6 | - | 14 | 16 |
| Prep/Dyeing/Finishing | BT16 autoclave | A-B-C | 3 | batch | 0,18 | - | 5 | 41 |
| Preparation | Stella | STP1-STP2 | 2 | batch | 0,6 | 7,5 | 6,75 | 15 |
| Preparation | Stella | STM1-STM2 | 2 | batch | 2 | 7,5 | 6,75 | 15 |
| Preparation | Stella | STG1 | 1 | batch | 3 | 7,5 | 6,75 | 15 |
| Prep/Dyeing | Jigger | JG01-JG05-JG09 | 3 | batch | 0,4 | - | 5 | 30-14-4 |
| Prep/Dyeing | Jigger | JG03 | 1 | batch | 0,3 | - | 2 | 30 |
| Prep/Dyeing | Mezzera | VBM1 | 1 | continuous | 40 | - | 18 | 17 |
| Prep/Dyeing | Overflow | MCS1 | 1 | batch | 2,4 | - | 3 | 2 |
| Prep/Dyeing | Overflow | MCS2 | 1 | batch | 1,6 | - | 2 | 4 |
| Prep/Dyeing | Overflow | MCS3 | 1 | batch | 0,8 | - | 2 | 10 |
| Prep/Dyeing | Overflow | MCS5 | 1 | batch | 0,4 | - | 2 | 10 |
| Prep/Dyeing | Barca | NB02 | 1 | batch | 0,38 | - | 0,2 | 20 |
| Prep/Dyeing | Barca | NB05 | 1 | batch | 0,9 | - | 0,2 | 20 |
| Prep/Dyeing | Barca | NB06 | 1 | batch | 2,1 | - | 0,8 | 20 |
| Finishing | Stalam | STA | 1 | continuous | - | - | 128 | 3 |
| Finishing | Centrifugal machine | DET | 1 | continuous | - | - | 3,2 | 1 |
| Finishing | Rameuse | FM01 | 1 | continuous | 0,1 | - | 31 | 30 |
| Finishing | Rameuse | FM02 | 1 | continuous | 0,1 | - | 44 | 5 |

* volume of bath for batch equipment or volume of starting bath for continuous operating mode

Notes:

Refer to An.B: energetic flow chart.

3) Annexes (all sheets have to be considered as relevant part of the whole document):

- An.A: Material flow chart;

- An.B: Energetic flow chart;

- An.C: Water flow chart;

- An.D: Production model;

- An.E: General Facilities - Process scheme (number of pages: 7);

- An.F: Preparation - Process scheme (number of pages: 25);

- An.G : Dyeing - Process scheme (number of pages: 93);

- An.H : Finishing - Process scheme (number of pages:8);

- An.I: Water consumptions (number of pages: 21);

- An.L: Water discharges (number of pages: 20);

- An.M: Discharged water analytic data (number of pages:2);

- An.N: Chemicals safety data sheets (number of pages 2.+number of pages 49 as attachments).