An.E: General Facilities- Process scheme.

- E.1: Softening.

- E.2: Steam production.

E.1: Softening.

| Department | General Facilities |
| --- | --- |
| Process | Softening |
| Operating | batch |
| Equipment | Ion exchange softener |
| Item | IE1 |
| Number of run/yr | 75 |
| Run time (h) | 48 |
| Regeneration cycle time (h) | 1,5 |

| Water type W1: 85,07 m3 | Softening | Water type W2: 85,07 m3 |  |
| --- | --- | --- | --- |
| Water type W2: 5610 l Sodium chloride ()= 990 kg. | Regeneration t=75 min | Discharge type D2: 5610 l T [°C]=14; pH [-]=9,2; Conductivity [mS/cm]= 4; COD [mg/l]=50; TSS [mg/l]=0. |  |
| Water type W2: 3625 l | Washing t=15 min | Discharge type D2: 3625 l T [°C]=13; pH [-]=7,9; Conductivity [mS/cm]=1; COD [mg/l]=50; TSS [mg/l]=0. |  |

Notes:

E.2: Steam production.

| Department | General Facilities |
| --- | --- |
| Process | Steam production |
| Operating | continuous |
| Equipment | boiler |
| Item | B1 |
| Number of cycles/yr | - |
| Cycle time (h) | - |
| Working hours (h) | 4950 |

| Water type W2: 455 l/h Deoxygenating agent (S01)=3 g/l; Conditioning agent (S02)= 5 g/l. | Evaporator | Steam type S1: 336 kg/h Steam type S2: 83 kg/h Steam type S3: 35 kg/h |  |
| --- | --- | --- | --- |
|  | Steam losses Evaporation 879 l/h (1) |  |  |

Notes:

(1): Total evaporation losses

| Department | General Facilities |
| --- | --- |
| Process | Steam production |
| Operating | continuous |
| Equipment | boiler |
| Item | B2 |
| Number of cycles/yr | - |
| Cycle time (h) | - |
| Working hours (h) | 4950 |

| Water type W2: 135 l/h Deoxygenating agent (S01)= 3 g/l; Conditioning agent (S02)= 5 g/l. | Evaporator | Steam type S1: 100 kg/h Steam type S2: 25 kg/h Steam type S3: 10 kg/h |  |
| --- | --- | --- | --- |
|  | Steam losses Evaporation 261 l/h (1) |  |  |

Notes:

(1): Total evaporation losses