An.E: General Facilities- Process scheme.

- E.1: Filtration.

- E.2: Softening.

- E.3: Steam production.

- E.4: Disinfection.

- E.5: Membrane ultrafiltration.

- E.6: Domestic use.

E.1:Filtration.

| Department | General Facilities |
| --- | --- |
| Process | Filtration |
| Operating (continuous/batch) | continuous |
| Equipment | Sand filter |
| Item | SF1-SF2 |
| Number of run/yr | - |
| Run time (h) | - |
| Number of cycles/yr | 15-15 |
| Cycle time (h) | 360-360 |
| Backwashing time (h) | 2 |

| Water type W1: 27,6 m3/h | Filtration | Water type W2: 27,6 m3/h |  |
| --- | --- | --- | --- |
| Water type W6: 9530,5 l | Bed expansion t=35 min |  |  |
| Compressed air | Backwashing t=105 min | Discharge type D5: 9530,5 l. T [°C]=15; pH [-]=7.6; Conductivity [mS/cm]=0,19; COD [mg/l]=10; TSS [mg/l]=10. |  |
| Water type W6: 4084,5 l | Rinsing t=15 min | Discharge type D5: 4084,5 l. T [°C]=12; pH [-]=7,6; Conductivity [mS/cm]=0,19; COD [mg/l]=10; TSS [mg/l]=0. |  |

Notes:

E.2: Softening.

| Department | General Facilities |
| --- | --- |
| Process | Softening |
| Operating (continuous/batch) | batch |
| Equipment | Ion exchange softener |
| Item | ES1-ES2-ES3 |
| Number of run/yr | 98-98-98 |
| Run time (h) | 18-18-18 |
| Regeneration cycle time (h) | 1,5 |

| Water type W2: 700 m3 | Softening | Water type W3: 700 m3 |  |
| --- | --- | --- | --- |
| Water type W3: 10000 l | Bed expansion t=20 min | Discharge type D5: 10000 l T [°C]=13; pH [-]=7,6; Conductivity [mS/cm]=0,19; COD [mg/l]=10; TSS [mg/l]=0. |  |
| Water type W3: 4100 l Sodium chloride = 817 kg | Regeneration t= 40 min | Discharge type D5: 4100 l T [°C]=13; pH [-]=8; Conductivity [mS/cm]=4; COD [mg/l]=10; TSS [mg/l]=0. |  |
| Water type W3: 8000 l | Washing t=30 min | Discharge type D5: 8000 l T [°C]=15; pH [-]=7,9; Conductivity [mS/cm]=1; COD [mg/l]=10; TSS [mg/l]=0. |  |

Notes:

E.3:Steam production.

| Department | General Facilities |
| --- | --- |
| Process | Steam production |
| Operating (continuous/batch) | continuous |
| Equipment | boiler |
| Item | B1-B2 |
| Number of cycles/yr | - |
| Cycle time (h) | - |
| Working hours (h) | 2640-2640 |

| Water type W4: 2820 l/h Deoxygenating agent (S01): 117 mg/l Deoxygenating agent (S02): 40 mg/l | Evaporator | Steam type S1: 2820 kg/h |  |
| --- | --- | --- | --- |
|  | Steam losses Evaporation 4495 l/h (1) |  |  |

Notes:

(1):Evaporation losses  (water lost in atmosphere) included

E.4: Disinfection.

| Department | General Facilities |
| --- | --- |
| Process | Disinfection |
| Operating (continuous/batch) | continuous |
| Equipment | UV lamp |
| Item | UV1 |
| Number of cycles/yr | - |
| Cycle time (h) | - |
| Working hours (h) | 5280 |

| Water type W3: 37,65 m3/h | Disinfection | Water type W4: 37,65 m3/h |
| --- | --- | --- |

Notes:

E.5: Membrane ultra-filtration.

| Department | General Facilities |
| --- | --- |
| Process | Membrane ultra-filtration |
| Operating (continuous/batch) | batch |
| Equipment | Membrane system |
| Item | MS1 |
| Number of run/yr | 375 |
| Run time (h) | 13,5 |
| Backwashing time (h) | 2,00 |
| Number of backwashing/yr | 125 |

| Discharge type D3: 15000 l | Ultra-filtration | Concentrate D6: 1875 l Discharge type D4: 13125 l T [°C]= 80; pH [-]=6,81; Conductivity [mS/cm]=0,702; COD [mg/l]=8227; TSS [mg/l]=0. |  |
| --- | --- | --- | --- |
| Water type W4: 500 l Cationic detergent = 0,5 % vol.; Caustic soda (10016) = 2 g/l. | Backwashing t=120 min | Discharge type D5: 500 l T [°C]=30; pH [-]=8,7; Conductivity [mS/cm]=0,103; COD [mg/l]=500; TSS [mg/l]=150. |  |

Notes:

Concentrate (not included in mass balances) re-used in sericin recovery system (see An.C: water flow chart)

E.6: Domestic use.

| Department | General Facilities |
| --- | --- |
| Process | Domestic use |
| Operating (continuous/batch) | continuous |
| Equipment | - |
| Item | - |
| Operating hours/yr | 5280 |

| Water type W6: 174 l/h | Use | Discharge type D6: 174 l/h T [°C]=20; pH [-]=7,6; Conductivity [mS/cm]=0,19; COD [mg/l]=60; TSS [mg/l]=10. |
| --- | --- | --- |