TOWEF0

(Toward Effluent Zero)

EVALUATION OF THE EFFECT OF THE IPPC APPLICATION ON THE SUSTAINABLE WASTE MANAGEMENT IN TEXTILE INDUSTRIES

PROCESS IDENTIFICATION AND

DATA COLLECTION SHEET

WP02.02.3-WP02.03.3

WP03.02.8-WP03.03.8

Company:   	B02

Address:

Contacts:

July 2002

a) Production:

Reference year: 2000

| Fiber | Type | (%) of total weight | processed m | kg per linear meter | processed kg |
| --- | --- | --- | --- | --- | --- |
| cotton | yarn or fabric |  |  |  |  |
| PES/CO | yarn |  |  |  |  |
| lycra woven in cotton |  |  |  |  |  |
| tencel |  |  |  |  |  |
| TOTAL: |  |  |  |  | 16 777 000 |

Notes:

Part of the cotton is first dyed as a yarn, afterwards it is woven and part of that fabric is dyed again in another colour shade, this process is called: to top.

B02 dyes predominantly jeans, which is a living fabric, i.e. the colour shade changes a bit after every household washing. You can only have this effect when the same fabric is dyed different times and under different forms (as yarn and as fabric).

b) Water use:

Reference year: 2000

b.1) Supplied water:

energy requirement for pumping groundwater: 0.5 kWh/m³

| Source | Quantity [m3/yr] | Cost [€/m3] |
| --- | --- | --- |
| potable water | 5 746 | 0.37 |
| groundwater | 396 510 | 0.12 |
| recuperated water (cooling water) | 49 345 | 0 |
| TOTAL: |  |  |

Notes:

b.2) Process water and treatment for internal use:

| Water type | Source | Treatment | Use | Quantity [m3/yr] | Treatment specific cost [€/m3] |
| --- | --- | --- | --- | --- | --- |
| 1 | potable | - | domestic | 5 746 | - |
| 2 | own wells | de-ironing, degassing, softening | production | 396 510 | 0.124 |
| 3 | recuperated water | - | mixed with pre-treated groundwater and used in production | 49 345 | - |

Notes:

10% loss of water in the pre-treatment of groundwater

total amount of process water is: (396 510 –10%) + 49 345 = 406 204 m³

b.3) Process water analytic features:

| Source- Type | groundwater (before pretreatment) | groundwater (after pretreatment) |
| --- | --- | --- |
| T [°C] |  |  |
| Conductivity [mS/cm] | 0.841 | 0.483 |
| Hardness [°F] | 34.5 | < 5 |
| pH [-] | 7.4 | 7.8-8 |
| COD [mg/l] | 18 |  |
| TSS [mg/l] | <20 |  |
| Fe (mg/l) | 2.8 | < 0.1 |
| Mn (mg/l) | 0.1 | < 0.1 |
| - - - - - |  |  |

Notes:

b.4) Steam production:

| Source- Type | Quantity [m3/yr] | T max [°C] | Use |
| --- | --- | --- | --- |
| 2 | 28 468 | 180 (9 bar) | drying indirect heating of dye baths and rinsing water (only part of the steam is used for direct heating) |

Notes:

b.5) Discharged water:

| Type | effluent before treatment | effluent after physical-chemical and biological treatment |
| --- | --- | --- |
| Quantity [m3/yr] | 309 270 | 294 060 |
| Final destination | treatment plant | sewer |
| Features: |  |  |
| T [°C] | 20-25 | 20-25 |
| Conductivity [mS/cm] |  |  |
| Hardness [°F] |  |  |
| pH [-] |  |  |
| COD [mg/l] | 4000-7000 | 958 |
| BOD5 [mg/l] | 2000-3500 | 131 |
| SST [mg/l] |  | 410 |
| Total N [mg/l] |  | 52.2 |
| N-NH4 [mg/l] |  |  |
| N-NO2 [mg/l] |  |  |
| N-NO3 [mg/l] |  |  |
| Ptot [mg/l] |  | 7.16 |
| Absorbance 420 nm |  |  |
| Absorbance 550 nm |  |  |
| Absorbance 680 nm |  |  |
| Anionic surf. [mgMBAS/l] |  |  |
| Non-ionic surf. [mgBiAS/l] |  |  |
| Cationic surf. [mg/ l] |  |  |
| Chlorides [mg/l] | 600-1600 |  |
| Chlorine [mg/l] |  |  |
| AOX [mg/l] |  |  |
| Chrome [mg/l] |  | 0.0578 |
| Copper [mg/l] |  | 0.079 |
| Endocrine activity |  |  |
| Hydrocarbons [mg/l] |  |  |
| Iron [mg/l] |  |  |
| Manganese [mg/l] |  |  |
| Nickel [mg/l] |  | 0 |
| Zinc [mg/l]ì |  | 0.1042 |
| Toxic Units (for algae) |  |  |
| Toxic Units (for fish) |  |  |
| Toxic Units (for bacteria) |  |  |
| Toxic Units (for invertebrates) |  |  |

Notes:

c) ENERGY CONSUMPTIONS:

| Source | Unit | Use | Quantity | Specific cost [€/ ] |
| --- | --- | --- | --- | --- |
| gas | gigajoule/yr | production | 195 494 |  2.5 EUR/gigajoule |
| electricity | kWh/yr | production | 39 592 851 | 0.05 |

Notes:

d) DEPARTMENTS AND WORKING TIME:

| Department. | Operating days | Daily operating period | Weekly operating period | N° of shifts per days |
| --- | --- | --- | --- | --- |
| yarn dyeing | 338 | 24/24 | 7/7 | 3 shift/d in the week 2 shifts/d in the weekend |
| piece dyeing | 338 | 24/24 | 7/7 | 3 shift/d in the week 2 shifts/d in the weekend |
| finishing | 225 | 24/24 | 5/7 | 3 shift/d in the week |

Notes:

e) EQUIPMENT:

| Department. | Equipment | Item | Quantity | Operating | Bath Volume [m3]* |
| --- | --- | --- | --- | --- | --- |
| piece dyeing | pretreatment line | PT1 | 1 | continuous | 4,6 |
|  | pad-steam | PS1 | 1 | continuous | 1.6 |
|  | sanfor | SF1-2 | 2 | continuous | 0.1 |
| yarn dyeing | warp dyeing line | WDL1-4 | 4 | continuous | 10.2 |

* volume of bath for batch equipment or volume of starting bath for continuos equipment

Notes:

The yarn dyeing department consists of 4 parallel lines. In practice, the pad-dry and the rinsing line is combined into 1 line. I split them up to be able to use the correct terminology and to be more clear where the water use is situated.

Annexes (all sheets have to be considered as relevant part of the whole document):

- An.A: Material flow chart;

- An.B: Energetic flow chart;

- An.C: Water flow chart;

- An.D: Production model;

- An.E: General Facilities - Process scheme (number of pages:….);

- An.F: Preparation - Process scheme (number of pages:….);

- An.G : Dyeing - Process scheme (number of pages:….);

- An.H : Xxxx (*) - Process scheme (number of pages:….);

- An.I: Water consumptions (number of pages:……);

- An.L: Water discharges (number of pages:….);

- An.M: Discharged water analytic data (number of pages:……);

- An.N: Chemicals safety data sheets (number of pages…..+number of pages ……as attachments).

(*):Xxxx: name of department