TOWEF0

(Toward Effluent Zero)

EVALUATION OF THE EFFECT OF THE IPPC APPLICATION ON THE SUSTAINABLE WASTE MANAGEMENT IN TEXTILE INDUSTRIES

PROCESS IDENTIFICATION AND

DATA COLLECTION SHEET

WP02.02.3-WP02.03.3

WP03.02.8-WP03.03.8

Company:   	B04

Address:

Contacts:

MAY 2003

a) Production:

Reference year: 2000

| Fiber | Type | (%) of total weight | processed m | kg per linear meter | processed kg |
| --- | --- | --- | --- | --- | --- |
| cotton | yarn | 20 |  |  | 1 463 200 |
| cotton | fabric | 80 |  |  | 5 852 800 |
| TOTAL: |  |  |  |  | 7 316 000 |

Notes:

b) Water use:

Reference year: 2000

b.1) Supplied water:

energy requirement for pumping ground water: 0.36 kWh/m³ and 0.46 kWh/m³ (2 different pumping sites)

| Source | Quantity [m3/yr] | Cost [€/m3] |
| --- | --- | --- |
| potable water | 6 242 | 2 |
| ground water | 553 073 | 0.05 |
| rain water | 35 271 | - |
| TOTAL: |  |  |

Notes:

b.2) Process water and treatment for internal use:

| Water type | Source | Treatment | Use | Quantity [m3/yr] | Treatment specific cost [€/m3] |
| --- | --- | --- | --- | --- | --- |
| 1 (potable water) | potable water | - | domestic/climatisation | 6 242 | - |
| 2 (ground water) | own wells | de-ironing, softening | production | 553 073 | 0.32 |
| 3 (rain water) | rain | filtration | steam production | 35 271 | - |

Notes:

b.3) Process water analytic features:

| Source- Type | groundwater (before pretreatment) | groundwater (after pretreatment) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| T [°C] |  |  |  |  |  |
| Conductivity [mS/cm] | 1.109 |  |  |  |  |
| Hardness [°F] | 53 | 2 |  |  |  |
| pH [-] | 7.2 |  |  |  |  |
| COD [mg/l] | 27 |  |  |  |  |
| TSS [mg/l] |  |  |  |  |  |
| Fe (mg/l) | 4.6 | < 0.1 |  |  |  |

Notes:

b.4) Steam production:

| Source- Type | Quantity [m3/yr] | T max [°C] | Use |
| --- | --- | --- | --- |
| 3 | 35 271 | 140-150 | heating of water and contact drying |

Notes:

b.5) Discharged water:

| Type | effluent before treatment | effluent after biological treatment |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Quantity [m3/yr] | 514 120 | 489 638 |  |  |  |
| Final destination | treatment plant | sewer |  |  |  |
| Features: |  |  |  |  |  |
| T [°C] |  |  |  |  |  |
| Conductivity [mS/cm] |  |  |  |  |  |
| Hardness [°F] |  |  |  |  |  |
| pH [-] |  |  |  |  |  |
| COD [mg/l] | 2000-2500 | 610 |  |  |  |
| BOD5 [mg/l] |  | 12 |  |  |  |
| SST [mg/l] |  | 42 |  |  |  |
| total N [mg/l] | 58-88 | 27 |  |  |  |
| N-NH4 [mg/l] |  |  |  |  |  |
| N-NO2 [mg/l] |  |  |  |  |  |
| N-NO3 [mg/l] |  |  |  |  |  |
| Ptot [mg/l] | 20 | 16 |  |  |  |
| Absorbance 420 nm |  |  |  |  |  |
| Absorbance 550 nm |  |  |  |  |  |
| Absorbance 680 nm |  |  |  |  |  |
| Anionic surf. [mgMBAS/l] |  |  |  |  |  |
| Non-ionic surf. [mgBiAS/l] |  |  |  |  |  |
| Cationic surf. [mg/ l] |  |  |  |  |  |
| Chlorides [mg/l] |  | 1240 |  |  |  |
| Chlorine [mg/l] |  |  |  |  |  |
| AOX [mg/l] |  |  |  |  |  |
| Chrome [mg/l] |  | 0.001 |  |  |  |
| Copper [mg/l] |  | 0.1 |  |  |  |
| Endocrine activity |  |  |  |  |  |
| Hydrocarbons [mg/l] |  |  |  |  |  |
| Iron [mg/l] |  |  |  |  |  |
| Manganese [mg/l] |  |  |  |  |  |
| Nickel [mg/l] |  |  |  |  |  |
| Zinc [mg/l]ì |  | 0.1 |  |  |  |
| Toxic Units (for algae) |  |  |  |  |  |
| Toxic Units (for fish) |  |  |  |  |  |
| Toxic Units (for bacteria) |  |  |  |  |  |
| Toxic Units (for invertebrates) |  |  |  |  |  |

Notes:

c) ENERGY CONSUMPTIONS:

| Source | Unit | Use | Quantity | Specific cost [€/ ] |
| --- | --- | --- | --- | --- |
| gas | m³/yr | production | 5 651 066 | 0.18 |
| electricity | kWh/yr | production | 24 212 399 | 0.05 |

Notes:

d) DEPARTMENTS AND WORKING TIME:

| Department. | Operating days | Daily operating period | Weekly operating period | N° of shifts per days |
| --- | --- | --- | --- | --- |
| pre-treatment | 225 | 24/24 | Mon 5 am – Sat 5 am | 3 |
| piece dyeing | 225 | 24/24 | Mon 5 am – Sat 5 am | 3 |
| yarn dyeing | 90 | 24/24 | 2days/week | 3 |

Notes:

e) EQUIPMENT:

| Department. | Equipment | Item | Quantity | Operating | Bath Volume [m3]* | Installed power (kW) | Absorbed power (kWh) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| pretreatment | pretreatment line | PT1 | 1 | continuous | 9.4 | 240.85 | 52.5 |
| piece dyeing | pad-steam | PS1 | 1 | continuous | 9.1 | 398.8 | 26 |
| yarn dyeing | bobine autoclave | AU1-11 | 11 | batch | 3.3 | 42 | 23 |

* volume of bath for batch equipment or volume of starting bath for continuous equipment

Notes:

Annexes (all sheets have to be considered as relevant part of the whole document):

- An.A: Material flow chart;

- An.B: Energetic flow chart;

- An.C: Water flow chart;

- An.D: Production model;

- An.E: General Facilities - Process scheme (number of pages:….);

- An.F: Preparation - Process scheme (number of pages:….);

- An.G : Dyeing - Process scheme (number of pages:….);

- An.H : Xxxx (*) - Process scheme (number of pages:….);

- An.I: Water consumptions (number of pages:……);

- An.L: Water discharges (number of pages:….);

- An.M: Discharged water analytic data (number of pages:……);

- An.N: Chemicals safety data sheets (number of pages…..+number of pages ……as attachments).