TOWEF0

(Toward Effluent Zero)

EVALUATION OF THE EFFECT OF THE IPPC APPLICATION ON THE SUSTAINABLE WASTE MANAGEMENT IN TEXTILE INDUSTRIES

PROCESS IDENTIFICATION AND

DATA COLLECTION SHEET

WP02.02.3-WP02.03.3

WP03.02.8-WP03.03.8

Company:   	B05

Address:

Contacts:

MAY 2003

a) Production:

Reference year: 2000

| Fiber | Type | (%) of total weight | processed m | kg per linear meter | processed kg |
| --- | --- | --- | --- | --- | --- |
| natural fibres: cotton and flax | fabric |  |  |  | 3 150 000 |
| kermel (fire free fibre) | fabric |  |  |  | 315 000 |
| mixed (CO/PES, CO/PA, CO/flax, CO/wool, CO/viscose) | fabric |  |  |  | 2 835 000 |
| TOTAL: |  |  |  |  | 6 300 000 |

Notes:

b) Water use:

Reference year: 2000

b.1) Supplied water:

| Source | Quantity [m3/yr] | Cost [€/m3] |
| --- | --- | --- |
| potable water | 5 000 | 1 |
| ground water | 500 000 | 0.5 |
| TOTAL: |  |  |

Notes:

b.2) Process water and treatment for internal use:

| Water type | Source | Treatment | Use | Quantity [m3/yr] | Treatment specific cost [€/m3] |
| --- | --- | --- | --- | --- | --- |
| 1 | potable | - | domestic | 5 000 | - |
| 2 | own wells | de-ironing, softening | production | 500 000 | 0.25 |

Notes:

b.3) Process water analytic features:

| Source- Type | ground water (before pretreatment) | ground water (after pretreatment) |
| --- | --- | --- |
| T [°C] |  |  |
| Conductivity [mS/cm] |  |  |
| Hardness [°F] | 50 | 1 |
| pH [-] | 6.8-7 | 7.2 |
| COD [mg/l] |  |  |
| TSS [mg/l] |  |  |
| Fe (mg/l) | 8 | 0 |
| - - - - - |  |  |
| - - - - - |  |  |

Notes:

b.4) Steam production:

| Source- Type | Quantity [m3/yr] | T max [°C] | Use |
| --- | --- | --- | --- |
| 2 (+ extra demineralisation) | 50 000 | 180°C (10 bar) | heating of baths (indirect for the new machinery and direct heating for the old ones) fixation of vat dyes in steamer |

Notes:

b.5) Discharged water:

| Type |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Quantity [m3/yr] | 450 000 |  |  |  |  |
| Final destination | sewer |  |  |  |  |
| Features: |  |  |  |  |  |
| T [°C] | 40 |  |  |  |  |
| Conductivity [mS/cm] |  |  |  |  |  |
| Hardness [°F] |  |  |  |  |  |
| pH [-] | 12 |  |  |  |  |
| COD [mg/l] | 2000 |  |  |  |  |
| BOD5 [mg/l] | 900 |  |  |  |  |
| SST [mg/l] | 50 |  |  |  |  |
| Total N [mg/l] | 40 |  |  |  |  |
| N-NH4 [mg/l] |  |  |  |  |  |
| N-NO2 [mg/l] |  |  |  |  |  |
| N-NO3 [mg/l] |  |  |  |  |  |
| Ptot [mg/l] | 5 |  |  |  |  |
| Absorbance 420 nm |  |  |  |  |  |
| Absorbance 550 nm |  |  |  |  |  |
| Absorbance 680 nm |  |  |  |  |  |
| Anionic surf. [mgMBAS/l] |  |  |  |  |  |
| Non-ionic surf. [mgBiAS/l] |  |  |  |  |  |
| Cationic surf. [mg/ l] |  |  |  |  |  |
| Chlorides [mg/l] |  |  |  |  |  |
| Chlorine [mg/l] |  |  |  |  |  |
| AOX [mg/l] |  |  |  |  |  |
| Chrome [mg/l] | 0.1 |  |  |  |  |
| Copper [mg/l] | 0.1 |  |  |  |  |
| Endocrine activity |  |  |  |  |  |
| Hydrocarbons [mg/l] |  |  |  |  |  |
| Iron [mg/l] |  |  |  |  |  |
| Manganese [mg/l] |  |  |  |  |  |
| Nickel [mg/l] | 0.1 |  |  |  |  |
| Zinc [mg/l]ì | 0.2 |  |  |  |  |
| Toxic Units (for algae) |  |  |  |  |  |
| Toxic Units (for fish) |  |  |  |  |  |
| Toxic Units (for bacteria) |  |  |  |  |  |
| Toxic Units (for invertebrates) |  |  |  |  |  |

Notes:

c) ENERGY CONSUMPTIONS:

| Source | Unit | Use | Quantity | Specific cost [€/ ] |
| --- | --- | --- | --- | --- |
| gas | m³/yr | production | 7 500 000 | 0.2 |
| electricity | kWh/yr | production | 8 500 000 | 0.05 |

d) DEPARTMENTS AND WORKING TIME:

| Department. | Operating days | Daily operating period | Weekly operating period | N° of shifts per days |
| --- | --- | --- | --- | --- |
| pretreatment | 285 | 24/24 | Mon 5 am – Sat 10 pm | 3 shift/d in the week 2 shifts/d in the weekend |
| dyeing | 285 | 24/24 | Mon 5 am – Sat 10 pm | 3 shift/d in the week 2 shifts/d in the weekend |

e) EQUIPMENT:

| Department. | Equipment | Item | Quantity | Operating | Bath Volume [m3]* | Installed power [kW] | Absorbed power ([kWh/month] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| pretreatment | pad-batch | PB1 | 1 | semi-continuous | 0.8 | 21 | 10 kW per hour |
|  | rakojet | RJ1 | 1 | continuous | 6.4 | 121,1 | 9 000 |
|  | mercerising line | ML1 | 1 | continuous | 6.8 | 50 | 12 605 |
| dyeing | hot flue | HF1-2 | 2 | continuous | 0.04 | 27.2 | 2 708 |
|  | pad-steam | PS1 | 1 | continuous | 11.32 | 100.8 | 1 318 |
|  | pad-steam | PS2 | 1 | continuous | 8.86 | 92.7 | 13 500 |
|  | pad-batch | PB2 | 1 | semi-continuous | 0.024 | 21 | 10 kW per hour |

* volume of bath for batch equipment or volume of starting bath for continuos equipment

Notes:

The absorbed power is a monthly average value.

Annexes (all sheets have to be considered as relevant part of the whole document):

- An.A: Material flow chart;

- An.B: Energetic flow chart;

- An.C: Water flow chart;

- An.D: Production model;

- An.E: General Facilities - Process scheme (number of pages:….);

- An.F: Preparation - Process scheme (number of pages:….);

- An.G : Dyeing - Process scheme (number of pages:….);

- An.H : Xxxx (*) - Process scheme (number of pages:….);

- An.I: Water consumptions (number of pages:……);

- An.L: Water discharges (number of pages:….);

- An.M: Discharged water analytic data (number of pages:……);

- An.N: Chemicals safety data sheets (number of pages…..+number of pages ……as attachments).

(*):Xxxx: name of department