| TOWEFO (Toward Effluent Zero) EVALUATION OF THE EFFECT OF THE IPPC APPLICATION ON THE SUSTAINABLE WASTE MANAGEMENT IN TEXTILE INDUSTRIES |  |  |  | IDENTIFICATION CODE: |  |  | DIS.: |  | Pag.: |  | of pag.: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | RE |  | 1 |  | 2 |
|  |  |  |  | PARTNER: |  |  | workpackage: |  |  |  |  |
|  |  |  |  | IPTS/JRC/EC |  |  | WP4 |  |  |  |  |
|  |  |  |  | EXTERNAL IDENTIFICATION CODE: |  |  |  |  |  |  |  |
|  |  |  |  | ContracT NUMBER: |  |  |  |  |  |  |  |
| TitLE: |  |  |  |  |  |  |  |  |  |  |  |
| Draft D8-“Data source on relaxation possibilities of process inlet water quality” |  |  |  |  |  |  |  |  |  |  |  |
| KEY WORDS |  |  |  |  |  |  |  |  |  |  |  |
| Annotations: |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |
| Rev. | DatE | DescriPTION |  |  | Editing | Validation |  |  |  | Approval |  |
| PERIOD COVERED BY REPORT |  |  | sections included: |  | CO-ORDINaTOR NAME: |  |  | PROJECT HOME PAGE: Numero: |  |  |  |
|  |  |  |  |  |  |  |  | http://spring.bologna.enea.it/towefo/ |  |  |  |

The identification of reuse/recycle options of process effluents, possibly after regeneration, implies a knowledge of the maximum tolerated relaxation of the inlet water quality. The table below gives a non-exhaustive, qualitative list of possible relaxation –of the inlet water qualities- and reuse for different steps in a textile finishing company. Some quantitative ranges are given as well. This data is based on literature data and discussions with partner 8 (Centexbel).

Table 1: Discussion of different process steps

| Process | Effluent | Relaxation |
| --- | --- | --- |
| Desizing | Possible product recovery. Permeate recycable. | Low quality water accepted. |
| Prewashing | Possible regeneration. | Low quality water accepted |
| Bleaching | Possible reuse of rinsing waters. Regeneration could be required. | High quality required (COD<60mg/l and S.S.<5mg/l) |
| Mercerising | Recovery of NaOH |  |
| Dyeing | Possible reuse of rinsing waters. Regeneration could be required. | High quality required (COD<60mg/l and S.S.<5mg/l) |
| Printing | Possible reuse of rinsing waters. Regeneration could be required. | High quality required (COD<60mg/l and S.S.<5mg/l) |

Due to the vast variation in textile processes and related effluents it is very hard to generalize the required inlet concentrations of the different steps. Based on experience, partner 8 (Centexbel) uses the values of Table 2 as a general requirement for process water in the Belgian textile industry.

As the required quality is, of course, subjected to for example the product colour (light or dark), the type of  rinsing (first or final), the kind of dyestuff that is being used, … extensive testing is still required for every possible new case.

Table 2: Requirements for process water

| pH | 6.5 – 9 |
| --- | --- |
| Hardness (°F) | < 5 |
| Conductivity (µS/cm) | < 1500 |
| COD (mg/l) | < 60 |
| Fe (µg/l) | < 100 |
| Mn (µg/l) | < 100 |
| Coulour | No visible colour or suspended solids |