FINAL REPORT ANNEX 4

Project TOWEF0

EC Contract EVK1-CT-2000-00063

Bologna January 16th , 2004

###### Minutes of the group meeting

#### List of participants

| Contractor | Participant |
| --- | --- |
| ENEA | Davide Mattioli |
|  | Loredana De Florio |
|  | Simona Scalbi |
|  | Mario Tarantini |
|  | Simona Verità |
| VITO | Wim Schiettecatte |
| ECOBILAN | Capucine Vayn |

The Meeting was held in Bologna on the January the 16th 2004, with the aim of identifying water saving strategies within selected textile companies and defining the data needed by LCA methodology and Ecobilan web based software to model the improved water management scenarios.

The meeting had  the following indicative agenda:

- presentation of tests results of membrane treatment of textile effluents  by ENEA;

- presentation of Water Pinch analysis results and assumptions behind them (VITO);

- request of data to define the future scenarios of LCA for the Italian Companies (ENEA, Ecobilan).

Davide Mattioli (ENEA) presented the results of pilot scale experiments  carried out using ultrafiltration, nanofiltration and reverse osmosis membrane plants.

The results of nanofiltration showed that only less polluted process effluents can be effectively treated to produce permeates suitable for reuse. The results of reverse osmosis showed that it is technically possible to treat all type of water to produce permeates suitable for reuse, but the economical evaluations led to very high costs.

Wim Schiettecatte (VITO) presented a flowchart of the entire water system of I06 company, showing all processes where water is used, including utility services, and where waste water is generated.

Different water re-use strategies were evaluated simulating their effects on the final WWTP effluent characteristics.

It was presented the design of an optimal water re-use scheme that reduces costs related to the water usage and discharge.

Water Pinch optimised scenarios proposed are based on the nanofiltration treatment and reuse of the more diluted effluents, according to the membrane treatment results. Treatment and reuse is department based (all preparation (or dyeing) effluents are treated in the same plant and reused in the preparation (or dyeing) processes .

Mario Tarantini (ENEA) and Capucine Vayn (Ecobilan) specified the data needs of the LCA web based software in order to analyse the improved water management scenarios. To allow a comparison with the “reference” scenarios, it was decided to focus on the products already analysed by LCA studies conducted by ENEA in the Italian companies. After an in-depth discussion the following scenarios were selected:

| Company | System | Processes | Reusable Water | Waste water treatment | Notes |
| --- | --- | --- | --- | --- | --- |
| I06 | B- Stuck Flax-Pes fabric dyed with dark colors | F.3.2 Desizing&Scouring | 81% of 4 m3 | Ultrafiltration and Nanofiltration | Reuse only discharge water and filling discharge water |
|  |  | G.3.1 Dyeing PES | 81% of 2,5 m3 | Ultrafiltration and Nanofiltration | Reuse only discharge water |
|  |  | G.8.1 Dyeing Flax |  |  | No reuse of water because its high conductivity |
|  |  | H.4 Antistatic Finishing |  |  | No water reuse |
| I02 | A-electronic table reactive printing | F.4.3 Printing | 80% of | Reverse Osmosis and ultrafiltration | Using reverse osmosis (best environmental friendly scenario) |
|  |  | H.1.3 Fabric washing | 80% of 4 m3 | Reverse Osmosis and ultrafiltration | Using reverse osmosis (best environmental friendly scenario) |
|  |  | I.1 Water finishing |  |  | No water reuse |
| I04 | A | F.1.1 Continuos scouring in mezzera | 80% of 365 m3 | Ultrafiltration and Nanofiltration | Reuse only filling and rinsing water |
|  |  | G.3.2 Dark direct dyeing in jigger | 80% of 3,7 m3 | Ultrafiltration and Nanofiltration | Reuse only water from continuous rinsing |
|  |  | G.13.1 Soaping in pad-steam | 80% of 14,45 m3 | Ultrafiltration and Nanofiltration | Reuse only rinsing, discharge, neutralisation water |
|  |  | H.2.2 Softener 2 finishing |  |  | No water reuse |
| I09 | A – silk yarn dyed with dark colors | F.1.3 HT scouring | 80% of 4 m3 | Ultrafiltration and Nanofiltration | Reuse only discharge washing water |
|  |  | G.6.2 Silk dark acid dyeing | 80% of 1.3m3 | Ultrafiltration and Nanofiltration | Reuse only discharge washing water |
|  |  | H.1.1 Softener finishing |  |  | No water reuse |
| I15 | A | F.1.1 Silk continuous scouring | 80% of 330m3 | Ultrafiltration and Nanofiltration | Reuse only rinsing water |
|  |  | G.3.1 Silk dark acid dyeing | 80% of 1m3 | Ultrafiltration and Nanofiltration | Reuse only continuous washing water |