| TOWEFO (Toward Effluent Zero) EVALUATION OF THE EFFECT OF THE IPPC APPLICATION ON THE SUSTAINABLE WASTE MANAGEMENT IN TEXTILE INDUSTRIES |  |  |  | IDENTIFICATION CODE: |  | DIS.: |  |  | Pag.: |  | of pag.: |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TM-607-002 |  | PU |  |  | 1 |  | 40 |
|  |  |  |  | PARTNER: |  | workpackage: |  |  |  |  |  |
|  |  |  |  | IES-JRC and VITO |  | WP7 |  |  |  |  |  |
|  |  |  |  | EXTERNAL IDENTIFICATION CODE: |  |  |  |  |  |  |  |
|  |  |  |  | ContracT NUMBER: |  |  |  |  |  |  |  |
|  |  |  |  | EVK1-CT-2000-00063 |  |  |  |  |  |  |  |
| TitLE: |  |  |  |  |  |  |  |  |  |  |  |
| D 19 - Final Report on Effluent Characterization in Selected Textile Finishing Industries |  |  |  |  |  |  |  |  |  |  |  |
| KEY WORDS |  |  |  |  |  |  |  |  |  |  |  |
| Annotations: |  |  |  |  |  |  |  |  |  |  |  |
| Authors: R. Loos, G. Hanke, S.J. Eisenreich IES-JRC Institute for Environment and Sustainability (IES), Inland and Marine Waters Unit, Joint Research Centre (JRC), European Commission, TP 290, I-21020 Ispra, Italy H. Witters, P. Berckmans, C. Vangenechten VITO Flemish Institute for Technological Research (Vito), B-2400 Mol, Belgium |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 04.2004 | Emission of the final version |  |  |  |  |  |  |  |  |  |
| Rev. | DatE | DescriPTION |  |  | Editing |  | Validation |  |  | Approval |  |
| PERIOD COVERED BY REPORT |  |  | sections included: |  | CO-ORDINaTOR NAME: |  |  | PROJECT HOME PAGE: Numero: |  |  |  |
|  |  |  |  |  | enea |  |  | http://spring.bologna.enea.it/towefo/ |  |  |  |

Table of content

1.	Introduction	3

1.1.	Chemical Analysis of organic environmental pollutants (IES-JRC)	3

1.2.	Ecotoxicological characterizations (VITO)	4

1.3.	Biological assays to screen for the presence of chemicals with endocrine disrupting activity (VITO)	5

2.	Experimental section	5

2.1.	Water Monitoring	5

2.2.	Solid-phase extraction (SPE) (IES-JRC)	6

2.3.	Liquid chromatography-mass spectrometry (LC-MS) (IES-JRC)	6

2.4.	LC triple-quadrupole tandem MS-MS (IES-JRC)	6

2.5.	Physical and sum parameters (IES-JRC)	6

3.	Results and discussion	7

3.1.	Chemical Analysis of organic environmental pollutants (IES-JRC)	7

3.1.1.	Liquid chromatography-mass spectrometry (IES-JRC)	7

3.1.2.	Solid-phase extraction (IES-JRC)	7

3.1.3.	Sequential solid-phase extraction (IES-JRC)	8

3.1.4.	Tandem LC-MS-MS (IES-JRC)	8

3.2.	Ecotoxicological characterisation (VITO)	8

3.2.1.	Acute toxicity of Belgian textile industry (VITO)	8

3.2.2.	Acute toxicity of Belgian WWTP (VITO)	9

3.2.3.	Acute toxicity of effluents of Italian waste water treatment plants (VITO)	10

3.2.4.	Summary of acute toxicity of Belgian and Italian samples (VITO).	11

3.2.5.	Chronic toxicity of effluents of Italian waste water treatment plants (VITO)	11

3.3.	Water monitoring sampling campaigns, simultaneously performed for integrated analysis (IES-JRC + VITO).	14

3.3.1.	Physical-chemical characterizations (IES-JRC)	14

3.3.2.	Sampling campaign June 2002: LC-MS analysis	15

3.3.3.	Sampling campaigns 2003: LC-MS-MS analysis (IES-JRC)	17

3.3.4.	Measurements of estrogenic activity in water samples in Belgian and Italian waters(VITO)	18

4.	Conclusions, Policy implications and Outlook	21

5.	ANNEXES	23

# Introduction

The objective of the IPPC Directive 96/61/EC is to prevent or reduce industrial emissions into the environment. Article 14 demands the monitoring of releases affecting the environment.

In support of the IPPC Directive, chemical analysis methods for organic micropollutants in water samples were developed, optimised, and validated (IES-JRC), and ecotoxicological tests to assess hazardous effects developed. Moreover, biological screening methods for endocrine disrupting activity were applied (VITO).

Biological methodologies ascertain the endocrine-disrupting activity of the sample, whilst chemical techniques identify known chemicals and quantify the concentration within that sample. Linking cause and effect by identifying the chemicals responsible for the estrogenic activity relies on a combination of both approaches. Both approaches are complementary and are needed to study the relevance of environmental pollutants in the water cycle.

## Chemical Analysis of organic environmental pollutants (IES-JRC)

The objective of this work was the establishment of analysis methods for the monitoring of organic micropollutants in the European textile industry and the corresponding receiving waters to access the impact of the industries on the environment. The knowledge of the priority pollutants present in the effluents of textile plants or the corresponding waste water treatment plants (WWTPs) is the first step in the development and improvement of water treatment processes.

Generally, for the assessment of the environmental impact, the treated effluents of the textile industries or the WWTPs treating the appropriate effluents were analysed, and not the untreated or internal waste water fluxes.

Environmental trace analysis of organic pollutants usually is performed by gas chromatography-mass spectrometry (GC-MS) or liquid chromatography-mass spectrometry (LC-MS) techniques after extraction of the samples by liquid-liquid- (LLE) or solid-phase extraction (SPE).

Of special interest are polar hydrophilic environmental pollutants because they are readily soluble in water and therefore pose several problems for fresh water systems and the supply of clean drinking water. They are difficult to remove during water treatment processes. In contrast, the more nonpolar water components are usually well removed in WWTPs due to their adsorption to soil organic matter.

Emphasis was placed on LC-MS analysis techniques because LC-MS is the method of choice for the analysis of polar water-soluble contaminants. Prior to the instrumental analysis, the same attention has to be paid to the sample preparation, extraction and clean-up procedures. Nowadays, SPE is the preferred extraction procedure in environmental analytical chemistry.

In the first two project years a multi-component analysis method for polar organic environmental water pollutants was developed, based on sequential solid-phase extraction (SSPE) followed by single-stage LC-MS. In the third project year emphasis was placed on the analysis of the organic micropollutants with a new triple-quadrupole tandem LC-MS-MS instrument. The objective of these methods was the analysis of as many as possible organic environmental pollutants in water samples.

Target analytes comprised phenols, carboxylic acids, aromatic sulfonates, surfactants, dyes (colorants), pharmaceuticals, and pesticides. Special emphasis was placed on alkylphenolic endocrine disrupting compounds (EDCs), namely nonylphenol ethoxylate surfactants (NPEOs), and their metabolites, nonylphenol (NP) and carboxylated NPECs.

The analysis of dyes resulted to be difficult due to the vast amount of different compounds used in textile industry. In addition, they are not of special environmental and toxicological importance.

An important part of the work was the production of four peer-reviewed publications. Analytical details and literature references can be found there.

1. R. Loos, G. Hanke, S.J. Eisenreich: Multi-Component Analysis of Polar Water Pollutants Using Sequential Solid-Phase Extraction Followed by LC-ESI-MS, J. Environ. Monit. 5 (2003) 384-394.

2. R. Loos, G. Hanke, S.J. Eisenreich: Comparison of Sequential Solid Phase Extraction (SSPE) with single SPE and Liquid Chromatography Single-Stage Mass Spectrometry (LC-MS) with Triple Quadrupole LC-MS-MS for Multi-Residue Analysis of Organic Water Pollutants.

3. R. Loos, G. Hanke, S.J. Eisenreich: Analysis of Nonylphenol Ethoxylate Surfactants (NPEOs), Octyl- and Nonylphenols, Carboxylated NPEC Metabolites and Bisphenol A in Water by Solid-Phase Extraction and Liquid Chromatography Single-Stage and Triple- Quadrupole Mass Spectrometry.

4. H. Witters, R. Loos, P. Berckmans, C. Vangenechten, G. Hanke, S.J. Eisenreich: Correlations between organic micropollutants and estrogenic activity in textile and waste water treatment plant effluents and receiving waters in Belgium and Italy.

## Ecotoxicological characterizations (VITO)

Complementary to chemical characterisation, ecotoxicological methods have been selected which allow to evaluate the environmental impact of textile effluents on receiving waters. A battery of ecological relevant tests has been selected, as they cover biological responses of 4 organisms (bacteria, green algae, invertebrate, fish) which represent different levels in the aquatic trophic chain. The selected test methods, either for acute toxicity (test duration maximum 4 days) or eventual chronic toxicity (test duration between 7 and 21 days) can be performed according to international guidelines (OECD, ISO,..). Methods for acute toxicity are preferably used for effluent characterisation, while chronic test are more appropriate to evaluate environmental impact of discharges in the receiving waters. With regard to the application of these methods for effluent characterisation, specific guidelines with regard to sample handling and boundary conditions have been defined. Moreover a methodology (score system) has been established which allows to rank waste water samples according to the measured acute toxicity, expressed as toxic units, which are derived from dose-response curves for each of the test organisms. Methods are explained in detail in deliverable 18.

## Biological assays to screen for the presence of chemicals with endocrine disrupting activity (VITO)

In addition, biological screening methods should be applied for the measurement of estrogenic activity caused by contaminants present in textile effluents and released into the environment. Two in vitro estrogen-receptor-based assays were made available at Vito for these applications. These methods, the MVLN assay (human breast cancer cell line) and the YES-assay (yeast cells) have been optimised and validated to detect the estrogenicity of pure chemical compounds (eg. pesticides, alkylphenolic compounds), which are likely to occur in textile effluents. In addition these cellular methods were used to detect estrogenic activity in complex waste water effluents and in surface waters. A solid phase extraction procedure has been developed and is validated, which does allow to simplify the matrix prior to exposure in the cellular systems. These biological screening methods are cost-effective methods which do allow to estimate the potential impact of chemicals and environmental samples on reproductive ability of natural fish populations in rivers. Methods have been described in detail in deliverable 18.

# Experimental section

## Water Monitoring

Belgian and Italian waters were investigated. In the 3rd project year two big water monitoring campaigns were jointly performed by IES-JRC and VITO in Belgium and Italy to investigate the environmental impact of textile industry.

Belgium: A water sampling campaign was performed on the 17-18 June 2003 in the textile industry region in Belgium south of Ghent. Samples were taken at three different times at 3 textile companies, or the corresponding WWTPs, and the receiving rivers (River Schelde, Creek Molenbeek and Creek Gaverbeek)

The textile companies are named CB1, CB4, and CB5. Company CB4 treats its discharge water in its own WWTP; the effluent of this plant was also analysed (sample B1). This effluent is transported in the sewer to a municipal WWTP. The effluent of this WWTP (B2) flows into the River Schelde (samples B3 and B4, after and before the effluent). Textile company CB5 discharges its water (B5) untreated into the Creek Molenbeek, and company CB1 into the sewer, which is treated in a municipal WWTP (B8). The effluents of the WWTPs and the receiving rivers and creeks before and after the effluents were investigated.

Italy: The Italian sampling campaign was performed on 19 November 2003 in the textile industry region south of the city Como. Samples: WWTP effluent Fino Mornasco (I1); receiving River Seveso (I2, I3); WWTP effluent Bulgarograsso (I4); the receiving creek Lura was dry; I5: River Seveso down-stream before entering Milan. In addition, in the North of Milan the River Lambro was investigated; I6: before the WWTP Monza / Brugherio; I7 after this WWTP in San Maurizio al Lambro; I8: River Lambro more down-stream in the Parco Lambro (Milan).

Other campaigns were performed before mainly for analytical process optimisations or for determination of acute and/or chronic toxicity. The following Table 1 shows all sampling campaigns performed during the project.

Table 1: Water monitoring sampling campaigns performed.

| No. | Date | Location | IES | Vito |
| --- | --- | --- | --- | --- |
| 1 | July 19, 2001 | At a textile WWTP in Fino Mornasco, N-Italy. | X |  |
| 2 | April 2001- June 2002 | Belgium, effluents textile, waste water treatment plant and receiving water |  | X (estrog. act.) |
| 3 | October 23, 2001 | At different rivers (Seveso, Olano, Lura, Guisa, Pudiga) north-west and south of Milan, N-Italy. | X |  |
| 4 | June-October 2002 | Belgium, effluents of 5 textile companies |  | X (acute ecotox) |
| 5 | February 12, 2002 | Along the River Seveso which runs in the direction north to south from the area of Como to Milan, N-Italy. | X |  |
| 6 | June 12, 2002 | At three textile WWTPs in the area south of Como, Fino Mornasco, N-Italy. | X | X (acute tox + estrog. act.) |
| 7 | July 16, 2002 | At the Lake Maggiore, Ispra, N-Italy (after rainfalls). | X |  |
| 8 | October 2002 | At three textile WWTPs in the area south of Como, Fino Mornasco, N-Italy. |  | X (chronic tox.+ estrog. act.) |
| 9 | December 16, 2002 | In the area east of Milan at the River Lambro. | X |  |
| 10 | June 17-18, 2003 | Belgium: Oudenaarde, Ronse, Waregem; River Schelde, creeks Molenbeek and Gaverbeek | X | X ((estrog. act.) |
| 11 | November 19, 2003 | Italy: Fino Mornasco, Bulgarograsso, Milan ; Rivers Seveso, Lura and Lambro | X | X (estrog. act.) |

## Solid-phase extraction (SPE) (IES-JRC)

The sequential SPE procedure can be found in publication (1), and the single-stage SPE method in publications (2) and (3).

## Liquid chromatography-mass spectrometry (LC-MS) (IES-JRC)

Publication (1).

## LC triple-quadrupole tandem MS-MS (IES-JRC)

Publications (2) and (3).

## Physical and sum parameters (IES-JRC)

The pH, conductivity, turbidity, dissolved oxygen, and the temperature were measured with a multi-parameter water probe U-22 (Horiba, Sulzbach, Germany).

The Dissolved Organic Carbon (DOC) and Total Organic Carbon (TOC) were measured with a TOC-5000 A Total Organic Carbon Analyzer (Shimadzu, Milan, Italy) using the European-DIN norm 1484. These parameters can be used as sum-monitoring parameters of organic water pollution.

The spectral absorption coefficient SAC (254 nm) and the color at 436 nm were measured with a Cadas 200 UV/VIS- spectrophotometer (Dr. Lange, Duesseldorf, Germany).

# Results and discussion

## Chemical Analysis of organic environmental pollutants (IES-JRC)

### Liquid chromatography-mass spectrometry (IES-JRC)

Publications (1), (2) and (3).

### Solid-phase extraction (IES-JRC)

Precise analytical results are dependent on a successful combination of sample preparation, chromatographic separation, and analytical detection. Interfering matrix compounds make a thorough sample preparation essential for reliable quantitative results.

In the first year of the project, SPE was thoroughly studied for the isolation of phenols, aromatic sulfonates, dyes, pesticides, aromatic amines, NPEOs, NPECs, and alkylphenols (APs). For the recovery experiments different water types (tap, Milli-Q, and lake water) were used, different SPE cartridges were tested, various organic solvents examined for the elution of the analytes, the elution volume optimized, and the evaporation of the analytes investigated. Moreover, the recoveries were evaluated at different spike concentrations.

C18 adsorbs most of the compounds at neutral pH, however very polar substances are only extracted by polymeric sorbents. The importance of the choice of the right elution solvent was pointed out. Ethylacetate was the best suited organic solvent for the elution of nonpolar compounds like APs. In contrast, methanol and acetone are well suited for the elution of polar compounds.

The results for the different compound classes are summarized in the following:

	The best sorbents for tracking polar compounds are Oasis HLB and LiChrolut 	EN.

	Elution of polar substances like sulfonates or dyes is best performed with polar 	solvents (acetone, methanol), not with ethylacetate.

	Ion-pair SPE is a well suited method for the extraction of aromatic sulfonates. Triethyl-	amine (TEA) or tributylamine (TBA) were used as ion-pairing reagent. Higher recoveries 	were obtained by further acidification with formic acid (pH 3).

	Due to the nonpolar character of NP and OP reversed-phase materials (C18) as well as 	polymeric styrol-divinylbenzene (SDB) are well suited sorbents for their extraction.

	Adsorption of APs with a lipophilic side-chain is easy, the bigger problem is the 	desorption of the compounds from the cartridge. For the extraction of APs the use of 	C18 adsorbent is recommended.

	The choice of the best suited organic solvent for elution of the compounds is 	crucial. 	Ethylacetate has a higher eluting power for the lipophilic APs than polar solvents 	methanol or acetone.

	Also mixtures of acetone or methanol with ethylacetate, hexane, or dichloromethane 	can be used. However, the disadvantage of hexane and dichloromethane is that these 	nonpolar solvents are not compatible with LC. Nonpolar solvents are strongly 	recommended for the elution of APs.

The fact that the choice of the right elution solvent is very important for successful SPE methods was also shown for dyes (colorants). Quantitative recoveries were obtained with C18 and Oasis HLB using methanol-acetone elution. With methanol-acetone-ethylacetate the recoveries were significant less. C18 did not adsorb the two most polar compounds, acid red 1 and acid orange 6.

### Sequential solid-phase extraction (IES-JRC)

For the polar to medium-polar analytes, sample clean-up and group separation were performed with a newly developed sequential SPE procedure using automated SPE with C18 and polymeric sorbent materials. The objective of this method was to analyse as many as possible compounds. The compounds are divided by this procedure in three different polarity classes. More then 90 compounds of environmental interest - comprising the most important chemical and substance classes: phenols, carboxylic acids, aromatic sulfonates, aromatic amines, pharmaceuticals, surfactants, dyes, pesticides, endocrine disrupters - have been chosen for the experiments. The extraction recovery values were determined, they were for most of the compounds in the range of 50-100 %. Analyses were performed by negative and positive electrospray ionisation (ESI) LC-MS using a single-quadrupole system.

Publication (1).

### Tandem LC-MS-MS (IES-JRC)

In the third project year emphasis was placed on the analysis of the organic micropollutants with a new triple-quadrupole tandem LC-MS-MS instrument. Cone voltage and collision energy for the MS-MS precursor-fragment-transitions were optimized for maximum sensitivity for each target analyte.

LC-MS-MS is advantageous in terms of selectivity, sensitivity, reliability and rapidity in analysis. The main advantage of LC-MS-MS is its high degree of certainty in detection. LC-MS-MS allows for a much faster and more reliable multi-residue analysis of polar contaminants with lower detection limits.

Publication (2) and (3).

## Ecotoxicological characterisation (VITO)

### Acute toxicity of Belgian textile industry (VITO)

For a sampling campaign of Belgian effluents of textile industry, acute toxicity end of pipe was determined (figure 1). As only, for company B04 a full scale waste water treatment plant was in operation, it appeared that lowest acute toxicity was seen at the level of this plant. For company B05, all 4 organisms did show a toxic response which amounted between 10 and 30 toxic units. This resulted also in a high cumulative effect score for this company (figure 4). There was no indication of one organism which appeared most sensitive for effluents of textile industry. Effects were most pronounced for algae and bacteria.

Figure 1: comparison of acute toxicity with 4 test organisms for industrial effluents of textile companies.

### Acute toxicity of Belgian WWTP (VITO)

Sampling at influents and effluents of Belgian waste water treatments plants demonstrated very low acute toxicity (< 4 TU). In all cases no acute toxicity was seen for waterflea. For influents bacteria, algae and fish showed minor effects at Oudenaarde, while only algae and fish responded for toxicity in the Waregem influent. At Waregem WWTP all measured acute toxicity was fully eliminated at the effluent side. This was not the case for Oudenaarde where acute toxicity remained for algae and fish. It is likely that effluents of Oudenaarde WWTP do have impact (chronic toxicity, not measured) on the receiving waters.

Figure 2: comparison of acute toxicity with 4 test organisms for influents and effluents of Belgian municipal waste water treatment plants which receive textile effluents.

### Acute toxicity of effluents of Italian waste water treatment plants (VITO)

Figure 3: comparison of acute toxicity with 4 test organisms for industrial effluents of textile companies.

### Summary of acute toxicity of Belgian and Italian samples (VITO).

Based on the classification for acute toxicity of the most sensitive organism (colour) and cumulative effect score based on the response of all organisms, results of all samples can be compared (details see description in deliverable 18). As expected, highest toxicity is observed for industrial effluents, followed by influents of WWTP, and in effluents of WWTP, except for Oudenaarde, no acute toxicity was seen (figure 4). Effluents of Italian textile companies were not available for acute toxicity evaluation in this project, and therefore no straightforward comparison with Belgian textile effluents is not possible.

In addition to acute toxicity assessment, which appeared absent for effluents of Italian WWTP, it was decided to estimate chronic toxicity. (see below).

### Chronic toxicity of effluents of Italian waste water treatment plants (VITO)

For chronic toxicity evaluation, which gives best prediction of environmental impact; 3 tests were used: algae growth, daphnid reproduction and fish early life stage test. For practical reasons, and availability of sample volume, only daphnid reproduction was evaluated for the receiving waters, while effects on algae and fish development were only assessed for the effluents of Italian waste water treatment plants (figure 5, 6, 7).

Observations on algae growth did show increase of growth compared to control conditions. This is not unexpected for waste waters of municipal treatment plants were high content of nutrients can occur. This evaluation does indicate that these 3 effluents will have a eutrophication effect, rather than a toxic effect due to chemicals present.

Figure 4: ranking of Belgian and Italian waste waters acording to acute toxicity.

Figure 5: results of algae growth test: stimulation of growth compared to control condition is shown instead of growth inhibition caused by toxic substances.

This effect of eutrophication was also confirmed by the tests with the invertebrate species “Daphnia magna”.

It is clearly illustrated in figure 6 that the number of young organisms which are produced by adult animals is higher for those exposed to effluents of WWTP compared to control conditions. Previous studies at Vito for effluents from municipal WWTP did gave similar results. The presence of nutrients does increase reproduction succes. However it can not be excluded that no chemicals, with potential toxic effects are present. Indeed, toxic potential can be masked by high nutrient content of the waste water. The latter might be the case for the effluent of Seveso WWTP. The eutrophication of the receiving waters is even more pronounced. Especially, rivers receiving the water from Bulgarograsso and Seveso plant do show increased numbers of young daphnids. The latter is again a confirmation of eutrophic conditions.

With regard to chronic toxicity of WWTP effluents to fish, it again appears that there is no chronic toxicity. There is minor fish mortality in effluents of Livescia and Seveso. These differences are not signifcant compared to control.

In summary, test results from chronic tests do give no indication for the presence of chemical pollutants which excert long-term toxicity for the selected samples and test organism.

Figure 6: results of waterflea reproduction test: stimulation of reproduction success as the ratio of number of young organisms compared to control is > 1 (except effluent Seveso).

Figure 7: results of fish early life stage test: survival of fish larvae is not signficantly disturbed compared to control group.

## Water monitoring sampling campaigns, simultaneously performed for integrated analysis (IES-JRC + VITO).

Textile industry effluents, effluents from municipal and industrial WWTPs treating the textile industries discharges, and the corresponding receiving rivers were simultaneously analysed in Belgium and Italy with the developed chemical and estrogenic analysis methods.

Urban and industrial pollution was observed in rivers and streams in N-Italy (Como, Milan) and in the Belgian textile industry region south of Ghent (Oudenaarde, Ronse, Waregem).

### Physical-chemical characterizations (IES-JRC)

Physical-chemical characterizations were performed in Belgium with a multi-parameter in-situ water probe. The following parameters were measured in the effluent and receiving waters: pH, conductivity, turbidity, dissolved oxygen, temperature, color (436 nm) and the standard absorption coefficient SAC (254 nm). Moreover, DOC and TOC were measured in the year 2002.

In Belgium, in the direct textile plant WWTP effluent of company B4 and the untreated textile plant effluent of company B5 significant higher values were found for the physical-chemical parameters conductivity, temperature, color (436 nm) and SAC (254 nm) (see the following Table). Especially the high temperature of B5 plant effluent is worrying. These results show the environmental impact of the textile industries in terms of “physical-chemical pollution”.

In the Belgian textile plant effluents physical-chemical pollution was observed for the parameters conductivity, temperature, color (436 nm) and SAC (254 nm).

Table 2. Physical-chemical parameters for the Belgian textile companies CB4 and CB5 and the corresponding receiving waters. Medium values from three measurements. Sampling campaign 17-18 June 2003.

| Place | Conductivity [S/m] | Temp. [ºC] | Color 436 nm [m-1] | SAC 245 nm [m-1] |
| --- | --- | --- | --- | --- |
| B4 WWTP effluent | 1.4 | 28.0 | 8.6 | 64.2 |
| WWTP Oudenarde | 0.2 | 19.8 | 0.6 | 6.5 |
| River Schelde before WWTP | 0.1 | 24.7 | 0.7 | 3.1 |
| River Schelde after WWTP | 0.1 | 24.8 | 0.9 | 3.6 |
| B5 plant effluent | 0.9 | 46.9 | 10.4 | 106.7 |
| Creek Molenbeek before effluent | 0.2 | 18.1 | 1.7 | 10.8 |
| Creek Molenbeek after effluent | 0.2 | 23.6 | 6.0 | 38.5 |

Also in the effluents of the Italian textile WWTPs (Livescia, Bulgarograsso, Seveso) physical-chemical pollution was observed for the parameters conductivity, turbidity, temperature, TOC, DOC, SAC, and color in comparison to the receiving rivers.

Table 3. Physical-chemical parameters for three Italian textile WWTPs and the corresponding receiving rivers. Sampling campaign 12 June 2002.

|  | Livescia |  |  | Bulgarograsso |  |  | Seveso |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TC2 before | TC1 Eff. | TC3 After | TC4 Before | TC5 Eff. | TC6 After | TC8 Before | TC7 Eff. | TC9 after |
| Conductivity [S/m] | 0.07 | 0.12 | 0.10 | 0.04 | 0.20 | 0.16 | 0.05 | 0.19 | 0.16 |
| Turbidity [NTU] | 10.8 | 145 | 19 | 20.6 | 47 | 22.5 | 10.8 | 109 | 179 |
| Temperature [ºC] | 16.2 | 19.1 | 18.5 | 15.3 | 22.1 | 21.1 | 16.2 | 21.4 | 20.2 |
| TOC [mg/l] | 6.3 | 12.5 | 11.2 | 2.2 | 13.6 | 20.9 | 2.0 | 16.1 | 14.8 |
| DOC [mg/l] | 5.8 | 11.0 | 8.9 | 2.0 | 17.2 | 16.3 | 3.1 | 13.1 | 10.7 |
| SAC [m-1] 254 nm | 2.8 | 6.6 | 6.4 | 1.6 | 11.0 | 11.9 | 2.0 | 6.7 | 2.1 |
| Color [m-1] 436 nm | 1.1 | 2.9 | 3.0 | 0.9 | 2.7 | 3.8 | 0.9 | 1.8 | 0.9 |

### Sampling campaign June 2002: LC-MS analysis

In June 2002 a big sampling campaign was conducted together with Vito (Belgium) and Lariana Depur (Italy) at three textile industry WWTPs in the area south of Como. In summary, the following pollutant types were identified in the water samples: phenols (nitrophenols, bisphenol A, NP), alkylphenolethoxylate (APEO) surfactants and their metabolites with endocrine disrupting potential (NPE1C, NPE1O, NPE2O, OPE2O, etc.), different aromatic sulfonates, LAS surfactants, dyes (colorants), pesticides, and pharmaceuticals.

The Lariana Depur WWTP “Alto Seveso” in Fino Mornasco (Italy) showed good treatment performance. In the effluents of the older WWTPs Livescia and Bulgarograsso slightly higher organic pollutant concentrations were detected.

Special attention was attached to the detection of endocrine disrupting compounds (EDCs): short-chain APEOs, NP, OP, BPA, and carboxylated APEO metabolites (alkylphenolethoxy-carboxylates, APECs). NP and NPE1C were found in all water samples spanning a concentration range of 0.46 g/L to 12.5 g/L for NP, and 0.58 g/L to 14.8 g/L for NPE1C. NPECs are relatively water soluble, so that their concentrations in river water are typically higher than that of the short-chain ethoxylated NPEO metabolites or NP, which have lower water solubilities. However, especially high concentrations of NP were found in the Rivers Livescia (5.3 g/L) and Lura (12.5 g/L, WWTP Bulgarograsso). NP was found in the effluents of all WWTPs (0.54 – 0.88 g/L). Besides NPE1C, also NPE2C, and NPE3C were found in all water samples, also the effluents of the WWTPs. In Livescia, higher NP and NPEC levels were found in the receiving river than in the WWTP effluent. In Bulgarograsso and Seveso however, the concentrations were higher in the effluents. Dicarboxylated NPEO metabolites were found in all investigated rivers, but not in the effluents of the plants.

Linear alkylbenzene sulfonates (LAS) are commonly used domestic surfactants; C10-13-LAS were found in all water samples. LAS are usually well removed in WWTPs by adsorption to the sludge as well as biodegradation. Only small quantities were found for these compounds in the effluents of the plants (0.8 – 2.3 g/L). However, surprisingly high concentrations were detected in the Rivers Seveso, Lura, and Livescia (22.8 – 171.8 g/L), and even higher concentrations in the rivers of the sampling campaign 6 at the River Lambro (16 December 2002).

Four dye compounds (colorants) were detected in the water samples: disperse blue 14, disperse red 1 (not in the effluents), acid red 1, and sulforhodamine B. The dye occurring most frequently and in the highest concentrations was sulforhodamine B. In the receiving water before the WWTP Livescia 4.7 g/L sulforhodamine B was found.

Table 4. Dyes in the effluents of the WWTPs; concentrations in [g/L].

| Compound | Livescia | Bulgarograsso | Seveso |
| --- | --- | --- | --- |
| Disperse Blue 14 | 2.3 | 0.02 | n.d. |
| Acid Red 1 | 1.2 | n.d. | n.d. |
| Sulforhodamine B | 0.71 | 0.60 | 1.1 |

Aromatic sulfonates were mostly found in the rivers, their concentrations in the WWTP effluents were low.

Pharmaceuticals such as benzafibrate, gemfibrozil, diclofenac, ibuprofen, and carbamazepine were regularly detected in Italian rivers in the low ng/l concentration range.

### Sampling campaigns 2003: LC-MS-MS analysis (IES-JRC)

In the third project year two big water monitoring campaigns were jointly performed in Belgium and Italy to investigate the environmental impact of textile industry. The chemical analysis results of these campaigns are given in the Annexes.

In summary, the following pollutant types were identified in the water samples: phenols (nitrophenols, bisphenol A, nonylphenol (NP), octylphenol (OP)), nonylphenolethoxylate (NPEO) surfactants and their metabolites with endocrine disrupting potential (NPECs, NPE1O, NPE2O, OPE2O, etc.), different aromatic sulfonates, LAS surfactants, dyes (colorants), pesticides, and pharmaceuticals.

Five dye compounds were detected in the water samples: Uniblue A, acid red 1 (up to 4 g/L in Belgium), disperse blue 14, and sulforhodamine B and G. In the WWTP effluent Waregem, the dye acid red 1 was detected in all three cases (1.3 – 4.0 g/L), which shows that this polar compound passes the waste water treatment process.

Table 5. Dyes in Belgian and Italian waters; concentrations in [g/L].

|  | Belgium |  | Italy |  |
| --- | --- | --- | --- | --- |
| Uniblue A | 1.5 | Effluent CB4 | 0.015 | River Lambro (I7) |
| Acid Red 1 | 4.0 | WWTP Waregem (B8) | n.d. |  |
| Disperse Blue 14 | 0.013 | Creek Gaverbeek (B16) | n.d. |  |
| Sulforhodamine B | n.d. |  | 0.75 | River Seveso (I5) |
| Sulforhodamine G | n.d. |  | 0.064 | River Seveso (I5) |

Pharmaceuticals such as benzafibrate, gemfibrozil, diclofenac, ibuprofen, and carbamazepine were regularly detected in both the Belgian and Italian rivers in the ng/L concentration range, up to 0.4 g/L for carbamazepine in Belgium and 0.47 g/L for diclofenac in Italy.

Sewage effluents from textile industry are a major source for endocrine disrupting compounds (EDCs) in the aquatic environment, the elimination of these substances in WWTPs is therefore of special interest.

In Belgium, among the EDCs investigated, NP and NPE1C had the highest concentrations in the water samples. The NP concentration was between 0.25 g/L (untreated plant effluent CB5) and 2.51 g/L (River Schelde after WWTP effluent Oudenaarde); the carboxylated metabolite NPE1C concentration was between 0.01 g/L (B5 plant effluent) and 2.36 g/L (WWTP Oudenaarde effluent). The highest NP levels were found in the River Schelde, the medium concentration was 2.2 g/L before the WWTP effluent Oudenaarde and 2.5 g/L after this effluent. The medium concentration in the WWTP effluent was 0.73 g/L, and in the WWTP effluent of the textile company (B1) 0.26 g/L. The highest NPEC concentrations were detected for NPE1C in the WWTP effluent Oudenaarde (2.3 g/L), River Schelde (1.5 g/L), Creek Molenbeek (1.9 g/L), and Creek Gaverbeek (1.1 g/L).

The same trend for NP was observed in Waregem, also here a lower (medium) concentration was measured in the WWTP effluent (0.58 g/L) compared to the receiving Creek Gaverbeek, 1.22 g/L before and 0.76 g/L after the WWTP effluent.

The lowest overall NP and NPEC concentrations were detected in the direct textile plant effluents of company B4 and B5 (0.26 g/L and 0.25 g/L for NP, 0.23 g/L and 0.01 g/L for NPE1C). For NPE2O however, the highest concentration levels were detected in the textile plant effluent B5 (2.1 g/L).

Also in Italy, the NPE1C concentration was higher in the WWTP effluent Fino Mornasco (2.8 g/L)  than in the River Seveso (0.3 g/L). For NP however a higher concentration in the receiving River Seveso (0.5 g/L) was observed compared to the effluent (0.37 g/L).

The highest NPE1C concentration (4.5 g/L) was found in the WWTP effluent in Bulgarograsso.

In conclusion, the highest NP levels were found in the receiving waters, the highest NPEC concentrations however in the WWTP effluents. In addition, worrying high NPEO concentrations (NPE2-17O) for all oligomers were found in the Belgian textile industry effluent B5 (sum up to 30.4 g/L), but also in the creek Molenbeek (5.7 g/L) and in Italy in the River Lambro in Milan (22.6 g/L). The highest NPE2O levels were found in textile plant effluents and surface waters (up to 3.6 g/L), suggesting direct industrial discharges. NP and NPECs are formed from the NPEO surfactants in the WWTPs and receiving waters.

Figure 8 shows that NP, the carboxylated NPEC metabolites and the short-chain NPEOs are released from WWTPs; they are difficult to remove.

Figure 8. APEO metabolites detected in WWTP effluents. I1: Fino Mornasco, ozone treatment; B8: Waregem, sand filter treatment; B2: Oudenaarde, no sand filter.

### Measurements of estrogenic activity in water samples in Belgian and Italian waters(VITO)

Monitoring campaigns in Italy and Belgium both did demonstrate high levels of estrogenic activity up to 12 ng/L E2 equivalent. Highest levels were observed in rivers and creeks. The level of 5 ng/L E2 equivalent, which is likely to cause hazardous effects to normal reproduction of fish populations was exceeded a few times in Italy and Belgium. Former studies at Vito did demonstrate significant increased levels of vitellogenin in zebrafish which were exposed for 3 weeks to river water with levels of estrogenic activity higher than 5 ng/L E2 equivalents (not published yet). Vitellogenin, a biomarker for estrogenic exposure is shown to be related to disturbed sexual development of fish larvae, and decreased egg production in female fish (Van den Belt et al., 2002, 2003). WWTP effluents in Italy did show levels of estrogenic activity below detection limit.

Figure 9: measured estrogenic activity with YES, and theoretical estimate based on measurement of analytes, multiplied with relative potency for Belgian waters.

Figure 10: measured estrogenic activity with YES, and theoretical estimate based on measurement of analytes, multiplied with relative potency for Italian waters.

Estrogenic activity in surface waters, being higher than those measured in industrial textile effluents and effluents of WWTPs might be due to the occurrence of NP. This metabolite of the nonylphenol ethoxylates, surfactant chemicals which are known to be used in textile industry, is formed in the effluents and receiving waters. The higher biological potency of NP compared to NPEO and NPEC compounds, might explain the elevated levels of estrogenic activity in surface waters.

Based on knowledge of relative potency (REP) of the EDCs BPA, NP, OP, NPE1C, NPE2C, NPE1O and NPE2O and the measured concentrations, an estimate could be made of the contribution of these chemicals to the estrogenic activity in the sample.  The correlation between the estrogenic activity and the chemicals detected was better for Italian than for Belgian waters. For surface waters, in most cases less than 10% of estrogenic activity could be predicted by chemical analysis. This does indicate that both chemical and biological approaches are complementary, and that other chemical compounds do occur in surface waters which have a significant contribution to the measured estrogenic activity. It is suggested, in comparison to literature data, that major compounds might be the natural and synthetic estrogenic hormones (17-oestradiol, oestrone, oestriol and 17-ethinyloestradiol), which are the result of diffuse pollution from household and agricultural run-off.

As a result of the simultaneous monitoring and analysis of Belgian and Italian waters by both partners with either chemical or biological methods, results could be compared. For each of the measured chemical pollutants, which are suspect for endocrine disrupting activity, the relation with the measured estrogenic activity in the bioassay was investigated. A highly significant correlation between the estrogenic signal and concentrations of bisphenol A in Belgian waters was demonstrated, which was less pronounced for Italian surface waters and WWTPs. Other significant correlations with estrogenic activity were found for NPE1C in Belgian industrial effluents and surface waters, for NPEOs in Italian WWTPs and surface waters, and for NPECs in Italian surface waters. When all the available data on estrogenic activity in Belgian and Italian waters were pooled, only for NP a significant correlation was found (Figure 11).

Figure 11: measured estrogenic activity with YES, comared to measured NP concentrations for Italian and Belgian watersamples do show significant correlation.

# Conclusions, Policy implications and Outlook

In support of the IPPC Directive, analysis methods for organic micropollutants in water samples were developed, based on solid-phase extraction (SPE) followed by liquid chromatography – mass spectrometry (LC-MS). These methods are suitable instruments for water monitoring sampling programmes of polar organic environmental water pollutants. They were successfully applied to the chemical analysis of textile industry effluents, waste water treatment plants (WWTPs), and the corresponding receiving waters. They contribute to the harmonization of analytical methods in order to provide internationally agreed methods, and to the implementation of the Water Framework and IPPC Directives. Moreover, the monitoring results from the field campaigns give useful environmental information to the public as well as policy-makers, considering the lack of reliable monitoring data in Europe. (IES-JRC).

A methodology for ecotoxicological characterisation has been developed which might be implemented by government as an effect-based instrument to assess waste water quality and quality of receiving waters. Moreover in vitro screening assays have shown to be a suitable instrument for detection of hazardous compounds with endocrine disrupting activity (VITO).

Method selection and validation for both acute and chronic ecotoxicity, and for detection of estrogenic compounds was finished and applied for Flemish and Italian textile industry. A successful characterisation of environmental samples in Italy has been performed through 2 monitoring campaigns in 2002. It is demonstrated that discharges of Italian waste water plants had minor environmental impact with regard to the release of toxic substances. On the other hand some evidence was given for eutrophication and the release of substances with estrogenic activity (VITO).

Emphasis was placed on alkylphenolic endocrine disrupting compounds (EDCs). Nonylphenol ethoxylate surfactants (NPEOs), nonylphenol (NP), and carboxylated NPECs are released into the environment from textile industry WWTPs in considerable amounts. NP and NPECs are formed from the NPEO surfactants in WWTPs and the receiving waters, they are difficult to remove in WWTPs. These results show that NPEOs are still used in industry. The predicted no-effect concentration (PNEC) for NP of 0.33 g/L for water species was frequently exceeded in the waters (up to 2.5 g/L), suggesting potential adverse effects to the aquatic environment. The concentrations for the non-regulated NPEOs and NPECs were even higher. This shows the need to improve water quality by cessation of toxic emission into the environment. The use of NPEO surfactants should be phased out. In addition to these alkylphenolic EDCs, in Belgium especially high and worrying pesticide concentrations were found in the water samples (IES-JRC). Through comparison of biological and chemical measurements a significant correlation was demonstrated between NP levels and estrogenic activity for pooled data from Italian and Belgian monitoring campaigns. In addition, for selected groups of samples, correlations were found between estrogenic activity and other EDCs such as bisphenol A, and some of the NPEOs and NPECs (VITO).

Measurement data did show that the NP quality standard was regularly exceeded and that estrogenic activity in Italian and Belgian surface waters was of that order of magnitude that normal fish reproduction could be hampered. It is confirmed that the European strategy with regard to EDC should be of high priority and that chemicals with known hormone disrupting activity should better be phase out, and be part of monitoring programmes as a follow-up.

Acknowledgements:

The water sampling campaigns were performed in cooperation with Lariana Depur (Italy) and Centexbel (Belgium). They are thanked for their kind cooperation.

We would like to acknowledge the assistance of E. Canuti, G. Zanarella, R. Tilio, and J. Wollgast. The physical-chemical parameters were measured by E. Canutti.

At Vito, the technical support by G. Jacobs, G. Geukens, E. Janssen, J. Baeken are highly appreciated.

# ANNEXES

Annex 1. Sampling campaign to three Italian textile WWTPs. Sum parameters. 12 June 2002. TC10: River Lura.

|  | Livescia |  |  | Bulgarograsso |  |  | Seveso |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TC2 before | TC1 Eff. | TC3 after | TC4 Before | TC5 Eff. | TC6 After | TC8 before | TC7 Eff. | TC9 after | TC10 |
| PH | 7.8 | 7.1 | 7.6 | 8.2 | 7.5 | 7.8 | 8.2 | 7.9 | 8.0 | 8.3 |
| Conductivity [S/m] | 0.07 | 0.12 | 0.10 | 0.04 | 0.20 | 0.16 | 0.05 | 0.19 | 0.16 | 0.09 |
| Turbidity [NTU] | 10.8 | 145 | 19 | 20.6 | 47 | 22.5 | 10.8 | 109 | 179 | 13.5 |
| Dissolved oxygen [%] | 100 | 100 | sat. | sat. | sat. | sat. | sat. | sat. | sat. | 100 |
| Temperature [ºC] | 16.2 | 19.1 | 18.5 | 15.3 | 22.1 | 21.1 | 16.2 | 21.4 | 20.2 | 19.7 |
| TOC [mg/l] | 6.3 | 12.5 | 11.2 | 2.2 | 13.6 | 20.9 | 2.0 | 16.1 | 14.8 | 10.1 |
| DOC [mg/l] | 5.8 | 11.0 | 8.9 | 2.0 | 17.2 | 16.3 | 3.1 | 13.1 | 10.7 | 6.2 |
| SAK [m-1] 254 nm | 2.8 | 6.6 | 6.4 | 1.6 | 11.0 | 11.9 | 2.0 | 6.7 | 2.1 | 5.4 |
| Color [m-1] 436 nm | 1.1 | 2.9 | 3.0 | 0.9 | 2.7 | 3.8 | 0.9 | 1.8 | 0.9 | 1.7 |

Annex 2. Sampling campaign to three WWTPs. Polar organic compounds, [μg/l]. 12 June 2002. TC10: River Lura.

|  |  | Livescia |  |  | Bulgarograsso |  |  | Seveso |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Compound |  | TC2 before | TC5 Eff. | TC3 after | TC4 before | TC5 Eff. | TC6 after | TC8 Before | TC7 Eff. | TC9 after | TC10 |
| PCP |  | 0.070 | - | 0.037 | - | - | - | - | - | - | - |
| Mononitrophenol |  | - | 0.030 | 0.026 | 0.025 | 0.14 | 0.12 | 0.031 | 0.16 | 0.13 | 0.14 |
| 2,4-Dinitrophenol |  | - | 0.028 | 0.022 | 0.041 | - | - | - | 0.022 | - | 0.088 |
| Bisphenol A |  | 0.043 | - | - | - | - | - | - | - | - | - |
| Nonylphenol |  | 5.3 | 0.66 | 1.3 | 1.9 | 0.88 | 12.5 | 0.46 | 0.54 | 0.63 | 2.3 |
| NPE1C |  | 12.9 | 2.4 | 4.4 | 0.64 | 0.65 | 3.9 | 0.53 | 3.8 | 4.6 | 2.3 |
| NPE2C |  | 7.2 | 3.8 | 4.6 | 0.21 | 0.57 | 1.4 | 0.22 | 2.0 | 2.5 | 1.5 |
| NPE3C |  | 4.0 | 3.8 | 4.1 | 0.12 | 2.7 | 3.1 | 0.15 | 2.0 | 2.6 | 2.4 |
| Atrazine |  | - | 0.041 | 0.033 | - | - | - | - | - | - | - |
| Diuron |  | - | - | - | 0.012 | 0.17 | 0.15 | - | - | - | - |
| Trifluralin |  | 0.44 | 0.48 | 0.43 | 0.60 | 0.48 | 0.35 | 0.63 | 0.33 | 0.49 | 0.54 |
| Benzenesulfonate |  | 0.39 | - | - | - | - | 0.42 | - | 0.37 | - | 0.13 |
| 2-Amino-1-naphthalenes. |  | 0.18 | - | 0.082 | - | - | - | 0.031 | - |  |  |
| p-Toluenesulfonate |  | 0.17 | - | - | 0.44 | - | 3.20 | - | 0.32 | - | - |
| Naphthalenesulfonate |  | 0.71 | - | 0.16 | 2.60 | - | 16.3 | 0.21 | - | 0.05 | - |
| Naphthalene-1,5-disulfonate |  | 2.4 | - | 1.6 | - | 15.5 | 14.9 | - | - | - | 7.2 |
| LAS (Dodecylbenzenesulfonate) | C10-LAS C11-LAS C12-LAS C13-LAS Σ LAS | 80.1 42.5 24.7 24.5 171.8 | 0.59 0.46 0.42 0.80 2.3 | 9.7 4.6 1.7 1.6 17.6 | 46.9 34.3 31.3 42.9 155.4 | 0.16 0.18 0.23 0.60 1.2 | 210.2 165.6 164.4 268.7 808.9 | 10.1 5.1 3.3 4.3 22.8 | 0.16 0.12 0.19 0.31 0.8 | 20. 1.1 0.82 1.0 4.9 | 37.2 26.4 24.3 34.7 122.6 |
| NPE1O |  | 0.65 | - | - | 0.13 | 0.16 | 0.83 | - | - | - | - |
| NPE2O |  | 4.0 | 0.74 | 1.1 | 0.86 | 0.36 | 5.6 | 0.07 | 0.17 | 0.22 | 0.67 |
| OPE2O |  | - | - | - | 0.08 | - | 0.35 | - | - | - | - |
| NPE9O |  | 0.59 | - | 0.14 | 1.7 | - | 7.5 | 0.16 | - | - | 0.16 |
| Disperse Blue 14 |  | 0.078 | 2.3 | 1.7 | 0.040 | 0.021 | 0.125 | 0.041 | - | - | 0.072 |
| Disperse Red 1 |  | - | - | - | 0.054 | - | 0.207 | - | - | - | - |
| Acid Red 1 |  | - | 1.19 | 0.80 | - | - | - | - | - | - | - |
| Sulforhodamine B | Pos. | 8.2 | 0.71 | 1.6 | 0.31 | 0.60 | 0.76 | 0.09 | 1.7 | 0.62 | 0.71 |
|  | IPC | 4.7 | 0.81 | 1.3 | 0.13 | 0.91 | 1.4 | 0.06 | 0.71 | 0.66 | 0.87 |
| Carbamazepine |  | - | 0.026 | 0.020 | 0.006 | 0.023 | 0.020 | 0.008 | - | - | 0.020 |
| 3,3’-Dichlorobenzidine |  | - | - | - | 0.121 | - | - | - | - | - | - |

OPE2O – OPE7O were found  in Bulgarograsso in the river before and in Bulgarograsso in the river after the WWTP. Also NPE1O – NPE15O were found in the water samples.

Annex 3. Sampling campaign in Belgium (B). 17-18 June 2003; Oudenaarde.

| Sample Name | Location | Date / Time | CPS coordinates Lat. Long. |  | Comments |
| --- | --- | --- | --- | --- | --- |
| B1 | Oudenaarde effluent (CB4) | 17.06, 10.57 h | 50.5143 | 3.3719 | Plant is stopped (no electricity), color dark purple |
| B2 | WWTP effluent Oudenarde | 11:16 h | 50.5180 | 3.3751 | WWTP is not working (no electricity) |
| B3 | Oudenaarde, River Schelde, after WWTP | 11:49 h | 50.5221 | 3.3768 | Sample taken at the edge of big river |
| B4 | Oudenaarde, River Schelde, before WWTP | 12:10 h | 50.5145 | 3.3756 | Sample taken at the edge of big river |
| B11 | Oudenaarde effluent (CB4) | 18.06, 8:52 h | 50.5143 | 3.3719 | Foam, color red/brown |
| B12 | WWTP Oudenarde | 8:58 h | 50.5180 | 3.3751 | water opaque and light brown |
| B13 | Oudenaarde, River Schelde, after WWTP | 9:15 h | 50.5221 | 3.3768 |  |
| B14 | Oudenaarde, River Schelde, before WWTP | 9:27 h | 50.5145 | 3.3756 |  |
| B21 | Oudenaarde effluent (CB4) | 18.06, 13:20 h | 50.5143 | 3.3719 | Foam, color dark purple |
| B22 | WWTP Oudenarde | 13:30 h | 50.5180 | 3.3751 | Water opaque and light brown |
| B23 | Oudenaarde, River Schelde, after WWTP | 13:40 h | 50.5221 | 3.3768 |  |
| B24 | Oudenaarde, River Schelde, before WWTP | 14:00 h | 50.5145 | 3.3756 |  |

Annex 4. Sampling campaign in Belgium. Physical-chemical parameters, Oudenaarde.

| Sample Name | pH | Conductivity [S/m] | Turbidity [NTU] | Dissolved Oxygen [%] | Temp. [ºC] | Color [m-1] 436 nm | SAC [m-1] 254 nm |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B1 | 8.43 | 1.41 | 200.0 | 98.48 | 26.6 | 9.22 | 62.4 |
| B2 | 7.68 | 0.19 | 127.0 | Sat | 19.8 | 0.41 | 4.67 |
| B3 | 7.38 | 0.09 | 174.0 | 68.93 | 25.2 | 1.11 | 3.83 |
| B4 | 7.50 | 0.09 | 66.6 | 69.39 | 24.6 | 0.62 | 3.14 |
| B11 | 8.44 | 1.32 | 245.5 | Sat | 28.3 | 7.96 | 61.2 |
| B12 | 7.74 | 0.25 | 87.1 | Sat | 19.4 | 0.75 | 7.28 |
| B13 | 7.54 | 0.10 | 253.0 | 56.30 | 24.0 | 0.69 | 3.18 |
| B14 | 7.76 | 0.09 | 240.0 | 98.89 | 25.1 | 0.68 | 2.99 |
| B21 | 8.40 | 1.32 | 661.0 | Sat | 29.1 | 8.5 | 69.2 |
| B22 | 7.63 | 0.28 | 200.0 | Sat | 20.1 | 0.74 | 7.64 |
| B23 | 7.55 | 0.11 | 665.0 | 58.57 | 25.1 | 1.03 | 3.64 |
| B24 | 7.56 | 0.09 | 220.0 | 98.66 | 24.3 | 0.84 | 3.25 |

Annex 5. Sampling campaign in Belgium (B). 17-18 June 2003; Ronse.

| Sample Name | Location | Date / Time | CPS coordinates Lat. Long. |  | Comments |
| --- | --- | --- | --- | --- | --- |
| B5 | Ronse plant effluent (CB5) | 17.06, 14:07 h | 50.4488 | 3.3535 | Deep blue color |
| B6 | Creek Molenbeek before plant | 14:35 h | 50.4490 | 3.3543 | Light blue color |
| B7 | Creek Molenbeek after plant | 14:49 h | 50.4485 | 3.3516 | Blue color |
| B18 | Ronse plant effluent (CB5) | 11:15 h | 50.4488 | 3.3535 | Deep blue color and textile particles, white foam |
| B19 | Creek Molenbeek before plant | 11:30 h | 50.4490 | 3.3543 | Brown color |
| B20 | Creek Molenbeek after plant | 11:50 h | 50.4485 | 3.3516 | Dark blue color and blue textile particles |
| B28 | Ronse plant effluent (CB5) | 16:00 h | 50.4488 | 3.3535 | Deep blue color and textile particles, white foam |
| B29 | Creek Molenbeek before plant | 16:15 h | 50.4490 | 3.3543 | Light brown color |
| B30 | Creek Molenbeek after plant | 16:20 h | 50.4485 | 3.3516 | Blue textile particles suspended in the water |

Annex 6. Sampling campaign in Belgium. Physical-chemical parameters; Ronse.

| Sample Name | pH | Conductivity [S/m] | Turbidity [NTU] | Dissolved Oxygen [%] | Temp. [ºC] | Color [m-1] 436 nm | SAC [m-1] 254 nm |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B5 | 5.20 | 0.29 | 357.0 | Sat | 36.7 | 6.67 | 55.7 |
| B6 | 7.64 | 0.15 | 177.5 | 69.01 | 18.2 | 1.4 | 8.41 |
| B7 | 6.98 | 0.25 | 422.5 | Sat | 22.0 | 6.35 | 57.1 |
| B18 | 12.03 | 1.94 | 106.8 | Sat | 53.8 | 7.5 | 197.2 |
| B19 | 8.31 | 0.14 | 173.5 | Sat | 17.7 | 1.7 | 12.8 |
| B20 | 9.37 | 0.19 | 570.0 | Sat | 23.6 | 5.32 | 32.9 |
| B28 | 6.60 | 0.52 | 370.0 | 86.91 | 50.0 | 17.1 | 67.3 |
| B29 | 7.05 | 0.17 | 240.0 | Sat | 18.5 | 1.85 | 11.3 |
| B30 | 6.76 | 0.21 | 375.0 | sat | 25.3 | 6.4 | 25.6 |

Annex 7. Sampling campaign in Belgium (B). 17-18 June 2003; Waregem.

| Sample Name | Location | Date / Time | CPS coordinates Lat. Long. |  | Comments |
| --- | --- | --- | --- | --- | --- |
| B8 | WWTP effluent Waregem (CB1) | 17.06, 15.30 h | 50.5385 | 3.2571 | Color light reddish |
| B9 | Creek Gaverbeek before plant | 15:48 h | 50.5387 | 3.2572 | Water opaque |
| B10 | Creek Gaverbeek after plant | 16:05 h | 50.5387 | 3.2563 | Water opaque, reddish foam on the surface |
| B15 | WWTP effluent Waregem | 18.06, 10:15 h | 50.5385 | 3.2571 | Color light rose, rainwater from the night |
| B16 | Creek Gaverbeek before plant | 10:20 h | 50.5387 | 3.2572 | Water opaque |
| B17 | Creek Gaverbeek after plant | 10:30 h | 50.5387 | 3.2563 | Reddish foam on the surface, brown color |
| B25 | WWTP effluent Waregem | 14:30 h | 50.5385 | 3.2571 | Color light rose, rainwater from the night |
| B26 | Creek Gaverbeek before plant | 14:50 h | 50.5387 | 3.2572 | Water opaque |
| B27 | Creek Gaverbeek after plant | 15:00 h | 50.5387 | 3.2563 | Water opaque, reddish foam on the surface |

Annex 8. Sampling campaign in Belgium. Physical-chemical parameters; Waregem.

| Sample Name | PH | Conductivity [S/m] | Turbidity [NTU] | Dissolved Oxygen [%] | Temp. [ºC] | Color [m-1] 436 nm | SAC [m-1] 254 nm |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B8 | 7.56 | 0.15 | 323.0 | 87.65 | 21.6 | 1.21 | 10.3 |
| B9 | 7.72 | 0.11 | 48.8 | sat | 21.6 | 0.94 | 7.41 |
| B10 | 7.84 | 0.14 | 30.10 | sat | 21.5 | 1.19 | 9.95 |
| B15 | 7.60 | 0.13 | 109.0 | 71.37 | 20.9 | 1.39 | 11 |
| B16 | 7.60 | 0.05 | 136.0 | 59.67 | 18.5 | 0.21 | 6.18 |
| B17 | 7.70 | 0.12 | 136.0 | sat | 20.6 | 1.46 | 10.6 |
| B25 | 7.69 | 0.12 | n.a. | sat | 21.9 | 1.33 | 10.7 |
| B26 | 7.76 | 0.05 | n.a. | sat | 20.1 | 0.82 | 6.84 |
| B27 | 7.83 | 0.12 | n.a. | sat | 21.7 | 1.29 | 9.87 |

Annex 9. Sampling campaign in Belgium (B). Polar organic compounds. 17-18 June 2003, [μg/l]. SSPE, 2nd fr., negative.

|  |  | Nitrophenol | 2,4-Dinitro-phenol | BPA | Uniblue A | Sulfor-hodamine B | Benzafibrate | Diclofenac | Ibuprofen | Gemfibrozil | NPE1C | NP (1st fr.) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB4 | B1 | - | 0.0017 | 0.0410 | 1.8578 | 0.0136 | 0.0036 | - | - | 0.0011 | 0.8305 | 1.0293 |
| Plant effluent | B11 | 0.0022 | 0.0017 | 0.0248 | - | 0.0202 | 0.0041 | - | 0.0012 | 0.0009 | 0.4527 | 0.6132 |
| WWTP | B21 | 0.0006 | 0.0064 | - | 1.9711 | 0.0227 | 0.0101 | 0.0083 | 0.0028 | 0.0022 | 0.5274 | 0.7148 |
| WWTP | B12 | 0.0015 | 0.0019 | 0.0347 | 0.0471 | 0.0461 | 0.1281 | 0.2441 | 0.0432 | 0.0011 | 3.4286 | 7.5754 |
| Oudenaarde | B22 | 0.0020 | 0.0030 | 0.0435 | 0.1069 | 0.0428 | 0.1100 | 0.2097 | 0.0325 | - | 3.6287 | 7.8567 |
| River Schelde | B3 | 0.0032 | - | 0.1237 | - | 0.0398 | 0.1113 | - | 0.1005 | 0.0165 | 2.4410 | 1.5346 |
| after WWTP | B13 | - | - | 0.0265 | - | - | 0.0018 | - | 0.0997 | - | 1.9929 | 2.4859 |
|  | B23 | 0.0046 | 0.0032 | 0.1410 | - | 0.0376 | 0.1341 | 0.0219 | 0.1439 | 0.0242 | 2.2705 | 3.0854 |
| River Schelde | B4 | 0.0033 | - | 0.0821 | - | 0.0485 | 0.1026 | - | 0.0906 | 0.0134 | 2.6103 | 1.6225 |
| before WWTP | B14 | 0.0019 | 0.0020 | 0.1400 | - | 0.0343 | 0.1488 | 0.0200 | 0.1652 | 0.0230 | 2.1841 | 2.9691 |
|  | B24 | 0.0018 | - | 0.0748 | - | 0.0401 | 0.1054 | - | 0.1228 | 0.0228 | 1.7188 | 1.1485 |
| CB5 effluent | B5 | 0.0033 | - | 0.0313 | - | 0.0118 | - | - | 0.0554 | - | 0.2205 | 0.4002 |
|  | B18 | 0.0020 | 0.0042 | - | - | 0.0076 | 0.0030 | - | 0.0017 | 0.0010 | - | 0.0050 |
|  | B28 | 0.0060 | 0.0072 | - | - | 0.0110 | 0.0023 | - | - | - | 0.0107 | 0.0241 |
| Creek Molen- | B6 | 0.0013 | - | - | - | 0.1355 | - | - | 0.1935 | - | 2.9720 | 4.5401 |
| beek before | B19 | 0.0020 | - | 0.0342 | - | 0.1092 | 0.0001 | - | 0.1118 | - | 2.2371 | 3.6034 |
| Plant | B29 | 0.0016 | - | 0.0720 | - | 0.1163 | 0.0094 | 0.0707 | 0.1740 | - | 3.9355 | 6.5343 |
| Creek Molen- | B7 | 0.0006 | - | 0.0350 | - | 0.0618 | - | 0.0404 | 0.1204 | - | 2.0306 | 3.0710 |
| beek after | B20 | 0.0016 | 0.0061 | - | - | 0.1033 | 0.0048 | 0.0138 | 0.0305 | - | 1.0365 | 2.4353 |
| Plant | B30 | 0.0028 | 0.0054 | - | - | 0.0690 | 0.0134 | 0.0275 | 0.0535 | - | 0.9533 | 1.9905 |
| WWTP | B8 | 0.0043 | - | 0.0777 | - | 0.0709 | 0.0127 | - | 0.0012 | - | 0.1936 | 0.9528 |
| Waregem | B15 | 0.0044 | 0.0016 | 0.0997 | - | 0.0812 | 0.0263 | 0.0810 | 0.0123 | 0.0032 | 1.1360 | 3.8544 |
|  | B25 | 0.0034 | 0.0022 | 0.1018 | - | 0.0720 | 0.0180 | 0.0759 | 0.0088 | 0.0031 | 0.9732 | 3.4381 |
| Creek Gaver- | B9 | 0.0016 | 0.0011 | 0.0333 | - | 0.0183 | 0.0198 | - | 0.0765 | 0.0004 | 1.1525 | 1.5812 |
| beek before | B16 | 0.0066 | 0.0036 | 0.1177 | - | 0.0191 | 0.0184 | 0.0375 | 0.0890 | - | 1.0616 | 1.5103 |
| Plant | B26 | 0.0094 | 0.0033 | 0.0866 | - | 0.0220 | 0.0155 | 0.0217 | 0.0877 | - | 1.0994 | 1.5296 |
| Creek Gaver- | B10 | 0.0038 | - | 0.1025 | - | 0.0768 | 0.0173 | 0.0636 | 0.0327 | - | 0.7746 | 2.0624 |
| beek after | B17 | 0.0044 | 0.0022 | 0.0883 | - | 0.0541 | 0.0220 | 0.0676 | 0.0284 | 0.0033 | 1.1529 | 3.2337 |
| Plant | B27 | 0.0070 | 0.0043 | 0.0904 | - | 0.0624 | 0.0203 | 0.0625 | 0.0198 | 0.0031 | 1.1670 | 3.3646 |

Table continued.

|  |  | Simazine | Desethyl-atrazine | Atrazine | Isoproturon | Diuron | Linuron | Terbutyl-azine | Chlorven-finphos | Carbaryl | NPE2O | Carbama-zepin |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB4 | B1 | - | 0.1544 | 0.0270 | 0.0038 | 0.3155 | 0.0233 | 0.0611 | - | 0.1083 | 1.4417 | 0.1245 |
| Plant effluent | B11 | - | 0.0078 | 0.0241 | 0.0013 | - | - | 0.0037 | - | - | 0.0278 | 0.0066 |
| WWTP | B21 | - | - | 0.0218 | 0.0017 | - | - | 0.0039 | - | - | 0.0399 | 0.0068 |
| WWTP | B12 | 2.6425 | 0.3541 | 2.0758 | 0.0124 | 2.4696 | 0.0499 | 0.2346 | - | 0.0441 | 0.6154 | 0.3243 |
| Oudenaarde | B22 | 2.6771 | 0.2666 | 2.3098 | 0.0158 | 2.5412 | 0.0203 | 0.2117 | - | - | 0.4024 | 0.3508 |
| River Schelde | B3 | 0.4236 | 0.2079 | 1.4140 | 0.1451 | 0.8652 | 0.0152 | 0.0639 | - | 0.0043 | 0.1848 | 0.0714 |
| after WWTP | B13 | 0.8091 | 0.4546 | 2.9696 | 0.1617 | 0.9519 | 0.1003 | 0.0864 | - | 0.0071 | 0.2115 | 0.1530 |
|  | B23 | 0.5558 | 0.2788 | 2.3275 | 0.0911 | 0.5989 | 0.0169 | 0.0474 | - | - | 0.2121 | 0.1283 |
| River Schelde | B4 | 0.4270 | 0.1707 | 1.7196 | 0.1333 | 0.8074 | 0.0095 | 0.0581 | - | - | 0.1890 | 0.0849 |
| before WWTP | B14 | 0.4478 | 0.2950 | 1.7877 | 0.0980 | 0.7048 | 0.0407 | 0.0499 | - | - | 0.3943 | 0.0837 |
|  | B24 | 0.3204 | 0.1325 | 1.0127 | 0.0766 | 0.4253 | 0.0041 | 0.0208 | - | - | 0.2303 | 0.0564 |
| CB5 effluent | B5 | 0.0095 | - | 0.0129 | - | - | - | 0.0029 | - | 0.0150 | 2.9951 | 0.0835 |
|  | B18 | - | - | 0.0090 | - | - | - | - | - | - | 0.0019 | 0.0053 |
|  | B28 | - | - | - | - | - | - | - | - | 0.0291 | 0.0473 | 0.0053 |
| Creek Molen- | B6 | 0.7574 | 0.0935 | 0.2370 | - | 0.1799 | 0.0338 | 0.0143 | - | 0.1484 | 0.4112 | 0.0585 |
| beek before | B19 | 0.6202 | 0.1604 | 0.4611 | - | 0.2229 | 0.0083 | 0.1117 | - | 0.0492 | 1.7897 | 0.0786 |
| Plant | B29 | 0.5032 | 0.2561 | 2.0848 | 0.1183 | 0.6686 | 0.0182 | 0.0613 | - | - | 0.4456 | 0.1271 |
| Creek Molen- | B7 | 0.7255 | 0.0570 | 0.1753 | 0.0007 | 0.1312 | 0.0131 | 0.0165 | - | 0.0570 | 1.3573 | 0.0623 |
| beek after | B20 | 0.8774 | 0.1264 | 0.7629 | 0.0027 | 0.1132 | 0.0048 | 0.0931 | - | 0.0752 | 0.0772 | 0.1430 |
| Plant | B30 | 0.5054 | 0.0907 | 0.3696 | - | 0.0723 | 0.0029 | 0.0309 | - | 0.0706 | 0.0519 | 0.0966 |
| WWTP | B8 | 0.9337 | 0.0961 | 0.2589 | - | 1.6614 | 0.0050 | 0.0030 | - | - | 0.0112 | 0.1820 |
| Waregem | B15 | 1.3011 | 0.1519 | 0.6533 | 0.0009 | 1.5675 | 0.0082 | 0.0087 | - | 0.0280 | 0.0350 | 0.3154 |
|  | B25 | 1.7038 | 0.2213 | 0.7423 | - | 1.9675 | 0.0230 | 0.0041 | - | 0.0484 | 0.0367 | 0.3147 |
| Creek Gaver- | B9 | 1.4114 | 0.5264 | 8.0951 | 0.0599 | 1.4632 | 0.0493 | 0.0097 | 0.0016 | - | 0.2955 | 0.0318 |
| beek before | B16 | 3.4758 | 0.6497 | 4.2243 | 0.0128 | 1.8176 | 0.0663 | 0.0086 | 0.0010 | 0.0014 | 0.1273 | 0.0539 |
| Plant | B26 | 5.4230 | 1.2307 | 5.8168 | 0.0137 | 1.7942 | 0.1006 | 0.0110 | 0.0015 | 0.0329 | 0.5355 | 0.0634 |
| Creek Gaver- | B10 | 1.9156 | 0.4287 | 4.9035 | 0.0257 | 2.4034 | 0.0761 | 0.0079 | - | 0.0300 | 0.0456 | 0.2533 |
| beek after | B17 | 1.7691 | 0.2851 | 1.2320 | 0.0029 | 1.7952 | 0.0212 | 0.0045 | - | 0.0530 | 0.0586 | 0.2351 |
| Plant | B27 | 2.3622 | 0.3628 | 1.8714 | 0.0048 | 1.7976 | 0.0168 | 0.0074 | - | 0.0346 | 0.0994 | 0.2763 |

Annex 10. Sampling campaign in Belgium (B). Polar organic compounds. 17-18 June 2003, [μg/l], SSPE, 3rd fraction, IPC, negative.

|  |  | Nitrophenol | 2,4-Dinitro-phenol | Benzene-sulfonate | 4-Chloro-bzsulfonate | p-Toluene-sulfonate | 2-Amino-1-nasulfonate | NS | Anthra-quinon-1,5-disulfonate | 1,5-Na-disulfonate | Anthra-quinon-2-sulfonate | C12-LAS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB4 | B1 | - | - | 0.0645 | 0.0136 | 0.0572 | 0.0360 | - | - | 0.5534 | 0.0086 | 0.1741 |
| Plant effluent | B11 | - | - | 0.0067 | - | 0.5976 | 0.6379 | 39.146 | - | 10.102 | 0.0410 | 3.3560 |
| WWTP | B21 | - | 0.0096 | - | - | - | - | 3.9051 | - | - | 0.0036 | 1.5129 |
| WWTP | B12 | 0.0365 | 0.0510 | 0.1303 | - | 1.0448 | 0.1240 | 0.6764 | - | 8.5403 | 0.0082 | 0.3699 |
| Oudenaarde | B22 | 0.0232 | 0.0414 | 0.1500 | - | 1.5781 | 0.0731 | 0.1583 | - | 7.1472 | 0.0057 | 0.2585 |
| River Schelde | B3 | 0.0367 | 0.0203 | 0.0264 | 0.0086 | 0.9574 | - | 1.1168 | - | 1.0337 | 0.0038 | 2.6117 |
| after WWTP | B13 | 0.0325 | 0.0149 | 0.0337 | - | 0.6665 | - | - | - | 1.8213 | 0.0015 | 0.1998 |
|  | B23 | - | - | 0.0639 | 0.0157 | 0.1050 | 0.0415 | 0.1012 | - | 0.7680 | 0.0056 | 0.1207 |
| River Schelde | B4 | 0.0417 | 0.0160 | 0.0237 | 0.0086 | 0.5590 | - | 1.2231 | - | 0.8582 | 0.0024 | 4.1031 |
| before WWTP | B14 | 0.0631 | 0.0176 | 0.0566 | - | 0.3288 | 0.0919 | 0.5786 | - | 1.3750 | 0.0041 | 0.1012 |
|  | B24 | 0.0334 | 0.0181 | 0.0826 | 0.0081 | 0.4848 | - | 2.9697 | - | 1.5852 | 0.0031 | 2.5229 |
| CB5 effluent | B5 | - | 0.0110 | 0.0959 | - | 2.2770 | 0.1513 | 82.813 | - | 122.41 | 0.0314 | 11.533 |
|  | B18 | - | - | 0.0486 | 0.0081 | 0.1293 | 0.0995 | 5.7685 | - | 1.5829 | 0.0086 | 0.6060 |
|  | B28 | - | 0.0058 | - | - | 0.1975 | - | 70.589 | - | - | - | 0.5226 |
| Creek Molen- | B6 | 0.0274 | 0.0060 | 0.1314 | 0.0163 | 6.3051 | 0.5360 | - | - | 20.613 | 0.0687 | 13.985 |
| beek before | B19 | 0.0446 | 0.0262 | 0.2403 | - | 2.6414 | 0.3350 | - | - | 18.476 | 0.0438 | 5.7038 |
| Plant | B29 | 0.0383 | 0.0195 | 0.1738 | 0.0227 | 7.3287 | 0.5880 | - | - | 18.749 | 0.0654 | 8.4583 |
| Creek Molen- | B7 | - | 0.0008 | - | 0.0066 | 0.5914 | 0.7376 | 54.152 | - | 9.9139 | 0.0516 | 1.9073 |
| beek after | B20 | 0.1090 | 0.0253 | 0.8778 | 0.0227 | 26.707 | 0.1847 | 103.20 | - | 108.31 | 0.0415 | 9.1840 |
| Plant | B30 | - | 0.0299 | 0.1349 | - | 0.9110 | - | 0.5398 | - | 23.634 | 0.0039 | 0.9351 |
| WWTP | B8 | - | - | 0.0296 | - | 0.2888 | - | - | - | 42.053 | 0.0021 | 0.2969 |
| Waregem | B15 | - | 0.0130 | 0.2652 | - | 0.8201 | - | - | - | 37.336 | - | 0.2927 |
|  | B25 | 0.0678 | 0.1397 | 0.5994 | 0.0094 | 4.8254 | - | 5.0485 | - | 3.4690 | 0.0030 | 2.2297 |
| Creek Gaver- | B9 | 0.0154 | 0.0210 | 1.8119 | 0.0113 | 9.5037 | - | 0.4086 | - | 10.297 | 0.0044 | 4.6079 |
| beek before | B16 | 0.0626 | 0.1268 | 0.5160 | - | 3.7913 | - | 4.6292 | 0.0822 | 2.7781 | - | 4.7764 |
| Plant | B26 | - | 0.0146 | 0.1348 | - | 1.5386 | - | - | - | 39.975 | 0.0026 | 5.8737 |
| Creek Gaver- | B10 | - | 0.0082 | 0.4262 | 0.0168 | 10.389 | - | 0.1630 | - | 50.018 | 0.0035 | 0.5948 |
| beek after | B17 | - | 0.0328 | 0.3352 | - | 0.7140 | - | 0.5574 | - | 33.087 | 0.0027 | 1.1948 |
| Plant | B27 | - | 0.0282 | - | - | 0.3893 | - | 0.7583 | - | 2.7111 | 0.0028 | 0.1858 |

Table continued.

|  |  | MCPA | 2,4-D | Mecoprop | Bentazone | Dichlorprop | 2,4,5-T | Acid Red 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB4 | B1 | 0.0086 | - | - | 0.0085 | - | - | - |
| Plant effluent | B11 | 0.0123 | 0.0139 | 0.0106 | 0.0032 | - | 0.0158 | - |
| WWTP | B21 | - | - | - | - | - | - | - |
| WWTP | B12 | 0.2357 | 0.2025 | 0.0875 | 0.0605 | 0.0169 | - | - |
| Oudenaarde | B22 | 0.1791 | 0.2104 | 0.0858 | 0.0591 | 0.0140 | - | - |
| River Schelde | B3 | 0.1626 | 0.1039 | 0.0433 | 0.1259 | 0.0667 | - | - |
| after WWTP | B13 | 0.0962 | 0.0404 | 0.0320 | 0.1026 | 0.0560 | - | - |
|  | B23 | 0.0110 | 0.0061 | - | 0.0081 | - | - | - |
| River Schelde | B4 | 0.1391 | 0.0897 | 0.0422 | 0.1574 | 0.0661 | - | - |
| before WWTP | B14 | 0.1247 | 0.0638 | 0.0396 | 0.1335 | 0.0740 | - | - |
|  | B24 | 0.0824 | 0.0368 | 0.0312 | 0.0791 | 0.0440 | 0.0032 | - |
| CB5 effluent | B5 | 0.0165 | 0.0070 | 0.0133 | 0.0066 | 0.0064 | 0.0205 | - |
|  | B18 | 0.0056 | 0.0050 | 0.0042 | 0.0051 | - | - | - |
|  | B28 | - | 0.0032 | 0.0063 | 0.0051 | 0.0036 | - | - |
| Creek Molen- | B6 | 0.0415 | 0.0375 | 0.0231 | 0.0072 | - | 0.0828 | - |
| beek before | B19 | 0.0469 | 0.0212 | 0.0308 | 0.0082 | 0.0091 | 0.0470 | - |
| Plant | B29 | 0.0792 | 0.0354 | 0.0400 | 0.0137 | 0.0207 | 0.1071 | - |
| Creek Molen- | B7 | 0.0001 | 0.0067 | 0.0078 | 0.0064 | 0.0044 | - | - |
| beek after | B20 | 0.0340 | 0.0172 | 0.0267 | - | - | 0.0283 | - |
| Plant | B30 | 0.0691 | 0.0306 | 0.0233 | 0.0342 | 0.0092 | - | 2.9902 |
| WWTP | B8 | - | - | 0.0233 | 0.0193 | 0.0074 | - | 4.4607 |
| Waregem | B15 | 0.0244 | 0.0202 | 0.0271 | 0.0213 | 0.0074 | - | 3.3652 |
|  | B25 | 0.4066 | 0.1480 | 0.0288 | 0.1085 | 0.0226 | - | 0.4893 |
| Creek Gaver- | B9 | 1.7308 | 0.0711 | 0.0302 | 0.3222 | 0.0542 | - | - |
| beek before | B16 | 0.1750 | 0.0403 | 0.0250 | 0.0707 | 0.0149 | 0.0047 | 0.3813 |
| Plant | B26 | 0.0149 | 0.0156 | 0.0298 | 0.0234 | - | - | 2.9688 |
| Creek Gaver- | B10 | 0.7624 | 0.0478 | 0.0494 | 0.2044 | 0.0389 | - | 5.5603 |
| beek after | B17 | 0.0497 | 0.0273 | 0.0272 | 0.0299 | 0.0088 | - | 2.6063 |
| Plant | B27 | 0.0797 | 0.0397 | 0.0274 | 0.0787 | 0.0491 | - | - |

Annex 11. Sampling campaign in Belgium (B). Polar organic compounds. 17-18 June 2003, [μg/l], SPE-LC-MS-MS.

|  |  | Nitrophenol | 2,4-Dinitro-phenol | Propanil | Uniblue A | Sulforhod-amine B | Benzafibrate | Diclofenac | Ibuprofen | Gemfibrozil |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB4 | B1 | 0.0553 | 0.0674 | - | 0.0393 | 0.0457 | 0.0092 | 0.0739 | 0.1876 | - |
| Plant effluent | B11 | 0.1245 | 0.0238 | - | - | 0.0777 | 0.0237 | - | - | - |
| WWTP | B21 | 0.0870 | 0.0186 | - | 1.4816 | 0.0653 | 0.0240 | - | - | 0.0095 |
| WWTP | B12 | 0.0369 | 0.1386 | - | - | 0.0310 | 0.0956 | 0.1964 | 0.0306 | - |
| Oudenaarde | B22 | 0.0235 | 0.1139 | - | 0.0448 | 0.0270 | 0.0642 | 0.1714 | 0.0180 | 0.0010 |
| River Schelde | B3 | 0.0432 | 0.0597 | - | - | 0.0255 | 0.0736 | 0.0137 | 0.0819 | 0.0124 |
| after WWTP | B13 | 0.0422 | 0.0426 | - | - | 0.0273 | 0.1010 | 0.0182 | 0.1004 | 0.0172 |
|  | B23 | 0.0465 | 0.0797 | - | - | 0.0257 | 0.0906 | 0.0190 | 0.1293 | 0.0200 |
| River Schelde | B4 | 0.0505 | 0.0575 | - | - | 0.0316 | 0.0697 | 0.0118 | 0.0829 | 0.0159 |
| before WWTP | B14 | 0.0490 | 0.0408 | - | - | 0.0244 | 0.1244 | 0.0231 | 0.1293 | 0.0213 |
|  | B24 | 0.0430 | 0.0461 | - | - | 0.0226 | 0.1107 | 0.0182 | 0.1175 | 0.0233 |
| CB5 effluent | B5 | 0.0518 | 0.0364 | - | - | 0.0941 | 0.0214 | - | 0.1826 | - |
|  | B18 | 0.0047 | 0.0107 | - | - | - | 0.0021 | - | 0.0013 | - |
|  | B28 | 0.3568 | 0.0508 | - | - | 0.0787 | 0.0427 | - | 0.0138 | 0.0106 |
| Creek Molen- | B6 | 0.0313 | 0.0326 | - | - | 0.0960 | 0.0038 | 0.1095 | 0.3247 | - |
| beek before | B19 | 0.0725 | 0.1487 | - | - | 0.0807 | 0.0028 | 0.0562 | 0.1700 | - |
| Plant | B29 | 0.0450 | 0.0385 | - | - | 0.0196 | 0.1053 | 0.0199 | 0.1448 | 0.0259 |
| Creek Molen- | B7 | 0.0140 | 0.0040 | - | - | 0.0282 | 0.0018 | 0.0355 | 0.0819 | - |
| beek after | B20 | 0.1195 | 0.1512 | - | - | 0.0608 | 0.0054 | 0.0275 | 0.1380 | - |
| Plant | B30 | 0.0432 | 0.0290 | - | - | 0.0373 | 0.0163 | 0.0221 | 0.0513 | - |
| WWTP | B8 | - | 0.0127 | - | - | 0.0550 | 0.0112 | 0.0592 | - | 0.0007 |
| Waregem | B15 | 0.0444 | 0.0157 | - | - | 0.0212 | 0.0034 | - | - | - |
|  | B25 | - | 0.0846 | 0.0022 | - | 0.0355 | 0.0159 | 0.0617 | - | 0.0023 |
| Creek Gaver- | B9 | 0.0466 | 0.1296 | - | - | 0.0084 | 0.0186 | 0.0163 | 0.0988 | - |
| beek before | B16 | 0.0289 | 0.4545 | 0.0024 | - | 0.0073 | 0.0319 | 0.0452 | 0.1072 | 0.0007 |
| Plant | B26 | 0.0392 | 0.4470 | - | - | 0.0091 | 0.0193 | 0.0347 | 0.0688 | - |
| Creek Gaver- | B10 | 0.0263 | 0.0555 | - | - | 0.0420 | 0.0149 | 0.0549 | 0.0276 | - |
| beek after | B17 | 0.0962 | 0.1617 | - | - | 0.0410 | 0.0197 | 0.0634 | 0.0245 | 0.0029 |
| Plant | B27 | 0.1007 | 0.1379 | - | - | 0.0356 | 0.0151 | 0.0559 | 0.0174 | 0.0022 |

Table continued.

|  |  | NP | NPE1C | NPE2C | NPE3C | OPE2C | BPA | PCP | NPE1O | NPE2O | OPE1C | OPE2O |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB4 | B1 | 0.1689 | 0.5386 | 0.0486 | 0.2554 | 0.0216 | - | 0.0374 | - | 0.0137 | - | 0.011 |
| Plant effluent | B11 | 0.4821 | 0.0696 | - | 0.0246 | - | - | - | - | 0.061 | - | 0.004 |
| WWTP | B21 | 0.1159 | 0.0788 | - | 0.0261 | - | - | - | - | 0.0678 | - | 0.004 |
| WWTP | B12 | 0.8901 | 2.4542 | 0.7738 | 0.9566 | 0.2335 | - | 0.0363 | 0.218 | 0.423 | - | 0.043 |
| Oudenaarde | B22 | 0.5626 | 2.2604 | 0.7578 | 0.9863 | 0.125 | - | 0.0247 | 0.165 | 0.3354 | - | 0.044 |
| River Schelde | B3 | 1.0408 | 1.1105 | 0.1079 | 0.0904 | 0.0202 | 0.0326 | - | 0.189 | 0.1283 | - | - |
| after WWTP | B13 | 3.0334 | 1.7429 | 0.1752 | 0.1523 | 0.0285 | 0.0446 | - | - | 0.2462 | - | 0.002 |
|  | B23 | 2.4615 | 1.5978 | 0.1408 | 0.1207 | 0.0178 | 0.0875 | 0.0078 | 0.436 | 0.1356 | - | 0.012 |
| River Schelde | B4 | 2.8966 | 1.8566 | 0.2182 | 0.1368 | 0.033 | 0.0325 | - | 0.122 | 0.2275 | - | 0.009 |
| before WWTP | B14 | 2.6868 | 1.8457 | 0.2025 | 0.1748 | 0.0295 | 0.0449 | - | 0.492 | 0.198 | - | 0.014 |
|  | B24 | 1.9382 | 1.3174 | 0.1845 | 0.1435 | 0.0267 | 0.0362 | 0.0057 | - | 0.5297 | 0.0051 | 0.017 |
| CB5 effluent | B5 | 0.3494 | 0.0293 | - | - | - | 0.0171 | - | - | 5.2764 | - | 0.128 |
|  | B18 | 0.0165 | 0.0001 | - | - | - | - | - | - | 0.005 | - | - |
|  | B28 | 0.3873 | - | 0.0027 | - | - | - | - | - | 1.0858 | - | - |
| Creek Molen- | B6 | 0.8537 | 1.7201 | 0.566 | 0.3078 | 0.1443 | 0.0249 | 0.055 | - | 1.0465 | - | 0.016 |
| beek before | B19 | 1.0516 | 1.6395 | 0.6063 | 0.2809 | 0.1532 | - | 0.1065 | - | 1.7185 | - | 0.027 |
| Plant | B29 | 1.0602 | 2.3052 | 0.6987 | 0.2723 | 0.109 | 0.0591 | 0.0731 | 1.016 | 0.8944 | - | 0.035 |
| Creek Molen- | B7 | 0.3918 | 0.9127 | 0.3079 | 0.1521 | 0.072 | 0.0219 | 0.0797 | - | 0.5992 | - | 0.055 |
| beek after | B20 | 0.3886 | 0.2148 | 0.0586 | 0.0129 | 0.0222 | 0.0146 | - | - | 0.1626 | - | - |
| Plant | B30 | 0.1926 | 0.0883 | 0.0747 | 0.0137 | 0.0126 | 0.0118 | - | - | 0.0691 | - | - |
| WWTP | B8 | 0.3218 | 0.1471 | 0.1252 | 0.9836 | 0.0078 | - | 0.0456 | - | 0.0156 | 0.0042 | 0.006 |
| Waregem | B15 | 0.5854 | 0.7795 | 0.4108 | 1.1039 | 0.0191 | - | 0.043 | - | 0.0308 | - | 0.01 |
|  | B25 | 0.8373 | 0.6463 | 0.4254 | 0.95 | 0.0238 | - | 0.0467 | - | 0.0219 | - | 0.004 |
| Creek Gaver- | B9 | 0.8784 | 1.5397 | 0.2833 | 0.1103 | - | 0.016 | 0.0142 | 0.183 | 0.2646 | - | 0.012 |
| beek before | B16 | 1.5249 | 1.0367 | 0.2086 | 0.0768 | 0.0199 | 0.0314 | 0.0175 | 0.458 | 0.3691 | - | 0.026 |
| Plant | B26 | 1.2625 | 0.8184 | 0.1646 | 0.0599 | 0.0173 | 0.0358 | 0.0196 | 0.458 | 0.2644 | - | 0.021 |
| Creek Gaver- | B10 | 0.6146 | 0.5137 | 0.1579 | 0.8325 | 0.0111 | - | 0.0294 | - | 0.0476 | - | 0.005 |
| beek after | B17 | 0.7499 | 0.8948 | 0.4536 | 0.76 | 0.031 | 0.0095 | 0.0339 | - | 0.0634 | - | 0.01 |
| Plant | B27 | 0.9116 | 0.738 | 0.2947 | 0.7887 | 0.0143 | - | 0.0457 | - | 0.0688 | - | 0.013 |

Table continued.

|  |  | Simazine | Desethyl-atrazine | Atrazine | Isoproturon | Diuron | Linuron | Terbutyl-azine | Alachlor | Carbama-zepin |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB4 | B1 | - | - | 0.0394 | 0.0088 | 0.0042 | - | 0.0121 | - | 0.0184 |
| Plant effluent | B11 | - | - | 0.1667 | 0.0501 | 0.0019 | - | 0.0785 | - | 0.1050 |
| WWTP | B21 | - | - | 0.0636 | - | - | - | 0.0732 | - | 0.0996 |
| WWTP | B12 | 2.7374 | 0.3057 | 1.9759 | 0.0143 | 2.1799 | 0.0172 | 0.3977 | - | 0.3994 |
| Oudenaarde | B22 | 2.4015 | 0.2763 | 2.6598 | 0.0199 | 2.4633 | 0.0185 | 0.2125 | 0.0121 | 0.3076 |
| River Schelde | B3 | 0.6200 | 0.2182 | 1.8385 | 0.1510 | 0.9041 | 0.0217 | 0.0710 | - | 0.1108 |
| after WWTP | B13 | 0.5246 | 0.2490 | 1.7576 | 0.0547 | 0.4588 | 0.0140 | 0.0468 | 0.0155 | 0.1215 |
|  | B23 | 0.4605 | 0.3024 | 2.0294 | 0.0896 | 0.5712 | 0.0220 | 0.0495 | - | 0.1153 |
| River Schelde | B4 | 0.5707 | 0.2355 | 2.3029 | 0.1418 | 0.8637 | 0.0190 | 0.1126 | - | 0.1177 |
| before WWTP | B14 | 0.6091 | 0.3154 | 2.0977 | 0.1019 | 0.6341 | 0.0124 | 0.0653 | - | 0.1471 |
|  | B24 | 0.5103 | 0.2846 | 1.9624 | 0.1130 | 0.5968 | 0.0121 | 0.0460 | - | 0.1260 |
| CB5 effluent | B5 | - | - | 0.0969 | - | 0.0041 | - | 0.0888 | - | 0.2196 |
|  | B18 | - | - | 0.0061 | - | - | - | - | - | 0.0122 |
|  | B28 | - | - | 0.1110 | - | - | - | 0.0748 | - | 0.0998 |
| Creek Molen- | B6 | 0.8767 | 0.1383 | 0.3058 | - | 0.2076 | 0.0173 | 0.0233 | - | 0.1023 |
| beek before | B19 | 0.9681 | 0.2735 | 0.7956 | 0.0021 | 0.3115 | 0.0098 | 0.1796 | 0.0199 | 0.1468 |
| Plant | B29 | 0.6022 | 0.1934 | 0.4907 | 0.0079 | 0.2712 | 0.0324 | 0.0616 | - | 0.1105 |
| Creek Molen- | B7 | 0.4222 | 0.0477 | 0.1472 | 0.0054 | 0.0985 | 0.0154 | 0.0154 | - | 0.0538 |
| beek after | B20 | 0.8290 | 0.2970 | 0.5608 | - | 0.1764 | 0.0155 | 0.1267 | - | 0.1576 |
| Plant | B30 | 0.3158 | 0.1299 | 0.1889 | - | 0.0710 | 0.0077 | 0.0406 | - | 0.0761 |
| WWTP | B8 | 1.6533 | 0.1740 | 0.4757 | 0.0001 | 2.1662 | 0.0073 | 0.0092 | - | 0.3656 |
| Waregem | B15 | 1.1173 | 0.1502 | 0.5520 | 0.0079 | 1.7215 | 0.0098 | 0.0144 | - | 0.2754 |
|  | B25 | 1.7255 | 0.2148 | 0.8292 | 0.0003 | 1.9427 | 0.0037 | 0.0101 | 0.0154 | 0.3228 |
| Creek Gaver- | B9 | 1.3804 | 1.1023 | 8.4771 | 0.0435 | 0.8719 | 0.0626 | 0.0141 | - | 0.0547 |
| beek before | B16 | 4.4066 | 0.7466 | 5.1309 | 0.0155 | 1.5270 | 0.0331 | 0.0117 | - | 0.0941 |
| Plant | B26 | 5.5661 | 1.0849 | 5.9612 | 0.0134 | 1.4228 | 0.0392 | 0.0149 | - | 0.1018 |
| Creek Gaver- | B10 | 2.6393 | 0.5546 | 5.4243 | 0.0315 | 2.0980 | 0.0382 | 0.0140 | - | 0.3776 |
| beek after | B17 | 1.9610 | 0.2855 | 1.9646 | 0.0038 | 1.6211 | 0.0075 | 0.0062 | 0.0164 | 0.2727 |
| Plant | B27 | 2.0335 | 0.3342 | 2.2197 | 0.0108 | 1.5761 | 0.0161 | 0.0133 | - | 0.2388 |

Table continued.

|  |  | Disperse Blue 14 | Chlorven-finphos | Chlorpyrifos | Carbaryl |
| --- | --- | --- | --- | --- | --- |
| CB4 | B1 | - | - | - | 0.0361 |
| Plant effluent | B11 | - | 0.1000 | - | - |
| WWTP | B21 | - | - | - | 0.0075 |
| WWTP | B12 | - | - | 0.0042 | - |
| Oudenaarde | B22 | - | - | - | - |
| River Schelde | B3 | - | - | - | - |
| after WWTP | B13 | - | - | - | - |
|  | B23 | - | 0.0094 | - | 0.0016 |
| River Schelde | B4 | - | - | - | - |
| before WWTP | B14 | - | - | 0.0030 | - |
|  | B24 | - | - | - | - |
| CB5 effluent | B5 | - | - | - | - |
|  | B18 | - | - | - | - |
|  | B28 | - | 0.0928 | - | 0.0473 |
| Creek Molen- | B6 | - | - | - | - |
| beek before | B19 | - | - | - | - |
| Plant | B29 | - | - | - | 0.1328 |
| Creek Molen- | B7 | - | - | - | 0.0638 |
| beek after | B20 | - | - | - | 0.0590 |
| Plant | B30 | - | 0.0090 | - | 0.0438 |
| WWTP | B8 | - | - | - | - |
| Waregem | B15 | - | - | - | 0.0470 |
|  | B25 | - | - | 0.0035 | - |
| Creek Gaver- | B9 | - | 0.0046 | 0.0031 | - |
| beek before | B16 | 0.0119 | 0.0043 | 0.0050 | - |
| Plant | B26 | - | 0.0045 | 0.0035 | - |
| Creek Gaver- | B10 | - | - | - | - |
| beek after | B17 | 0.0128 | 0.0046 | - | - |
| Plant | B27 | - | - | 0.0009 | 0.0204 |

Table continued.

|  |  | Nitrophenol | 2,4-Dinitro-phenol | Benzene-sulfonate | 4-Chloro-bzsulfonate | p-Toluene-sulfonate | 2-Amino-1-nasulfonate | NS | Anthra-quinon-1,5-disulfonate | 1,5-Na-disulfonate | Anthra-quinon-2-sulfonate | C12-LAS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB4 | B1 | 0.0491 | 0.0039 | - | 0.0119 | - | - | 0.3710 | - | - | - | 0.6629 |
| Plant effluent | B11 | 0.1754 | 0.0099 | 0.1256 | 0.0424 | - | 0.1326 | 1.1849 | - | - | 0.0496 | 4.3716 |
| WWTP | B21 | 0.1328 | 0.0098 | 0.0992 | 0.0415 | - | - | 0.9983 | - | - | 0.0591 | 18.969 |
| WWTP | B12 | - | 0.0761 | - | - | - | 0.0902 | 1.3478 | - | 0.0147 | 0.0042 | ? |
| Oudenaarde | B22 | 0.0311 | 0.0698 | - | 0.0041 | - | 0.0668 | 0.3841 | - | - | 0.0056 | 0.4940 |
| River Schelde | B3 | 0.0424 | 0.0317 | 0.0049 | 0.0019 | 0.0071 | - | 0.6586 | - | - | 0.0030 | 3.1295 |
| after WWTP | B13 | - | 0.0184 | 0.0078 | 0.0020 | 0.0293 | - | 0.6530 | 0.0056 | 0.0602 | 0.0040 | 0.1635 |
|  | B23 | 0.0588 | 0.0470 | - | - | - | 0.0238 | 0.8238 | - | - | 0.0045 | 0.0373 |
| River Schelde | B4 | 0.0448 | 0.0251 | 0.0013 | 0.0024 | - | - | 0.7022 | - | - | 0.0033 | 30.637 |
| before WWTP | B14 | 0.0504 | 0.0231 | - | - | 0.0298 | 0.0171 | 0.7184 | - | - | 0.0052 | 0.2393 |
|  | B24 | 0.0323 | 0.0230 | - | 0.0023 | - | - | 1.6142 | 0.0007 | - | 0.0036 | 4.9012 |
| CB5 effluent | B5 | 0.0741 | 0.0213 | 0.0645 | 0.0389 | 0.0994 | 0.1566 | 180.76 | - | - | 0.6313 | 158.92 |
|  | B18 | 0.0069 | 0.0102 | - | 0.0039 | 0.0528 | 0.0086 | 3.0472 | - | - | 0.0025 | 0.4458 |
|  | B28 | 0.3049 | 0.0259 | 0.0399 | - | 0.0664 | - | 13.744 | - | - | 0.0478 | 117.29 |
| Creek Molen- | B6 | 0.0279 | 0.0158 | - | 0.0064 | 0.1252 | 0.2160 | 0.3218 | - | - | 0.2069 | 56.001 |
| beek before | B19 | 0.0797 | 0.0755 | - | 0.0029 | - | - | 0.2817 | 0.0013 | - | 0.1265 | 18.645 |
| Plant | B29 | 0.0606 | 0.0412 | - | 0.0043 | - | 0.0965 | 0.1732 | - | - | 0.1453 | 26.569 |
| Creek Molen- | B7 | 0.0196 | 0.0041 | - | 0.0040 | 0.0240 | 0.0703 | 58.961 | - | - | 0.2464 | 135.49 |
| beek after | B20 | 0.1131 | 0.0982 | 0.1273 | 0.0072 | 1.7330 | 0.0476 | 90.408 | - | - | 0.1122 | 56.589 |
| Plant | B30 | 0.0699 | 0.0323 | - | 0.0051 | 0.0564 | 0.0272 | 45.045 | - | - | 0.0659 | 89.848 |
| WWTP | B8 | - | 0.0049 | - | 0.0020 | - | - | 0.0460 | - | - | 0.0041 | 0.1252 |
| Waregem | B15 | 0.0286 | 0.0380 | 0.0085 | 0.0039 | - | - | 0.2360 | - | - | 0.0063 | 1.2192 |
|  | B25 | - | 0.0377 | - | - | - | - | 0.1843 | - | - | 0.0062 | 0.4050 |
| Creek Gaver- | B9 | 0.0433 | 0.0551 | 0.0032 | 0.0027 | - | - | 2.1722 | - | - | 0.0058 | 11.892 |
| beek before | B16 | 0.0223 | 0.2123 | - | 0.0018 | - | - | 9.1902 | - | - | 0.0042 | 8.6719 |
| Plant | B26 | 0.0322 | 0.2017 | - | 0.0019 | - | - | 9.3379 | - | - | 0.0041 | 28.121 |
| Creek Gaver- | B10 | 0.0322 | 0.0233 | - | 0.0022 | - | - | 0.4574 | - | - | 0.0055 | 1.6164 |
| beek after | B17 | 0.0901 | 0.0761 | - | 0.0020 | - | - | 1.0396 | - | - | 0.0058 | 8.4568 |
| Plant | B27 | 0.0796 | 0.0782 | - | 0.0041 | - | 0.0239 | 1.6404 | - | - | 0.0062 | 13.150 |

Table continued.

|  |  | MCPA | 2,4-D | Mecoprop | Bentazone | Dichlorprop | 2,4,5-T | Acid Red 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CB4 | B1 | 0.0399 | - | - | 0.0188 | - | - | 0.0809 |
| Plant effluent | B11 | 0.1276 | - | 0.1435 | 0.0443 | - | 0.0652 | - |
| WWTP | B21 | 0.1015 | - | - | 0.0397 | - | - | - |
| WWTP | B12 | 0.3147 | 0.2327 | 0.1159 | 0.0914 | - | - | 0.0234 |
| Oudenaarde | B22 | 0.2737 | 0.2506 | 0.1251 | 0.0932 | - | - | - |
| River Schelde | B3 | 0.2907 | 0.1686 | 0.0703 | 0.2196 | 0.1009 | 0.0024 | - |
| after WWTP | B13 | 0.1900 | 0.0631 | 0.0438 | 0.1891 | 0.0744 | 0.0022 | - |
|  | B23 | 0.1635 | 0.0809 | 0.0499 | 0.1919 | 0.0864 | 0.0070 | - |
| River Schelde | B4 | 0.2472 | 0.1458 | 0.0681 | 0.2864 | 0.0996 | 0.0022 | - |
| before WWTP | B14 | 0.1867 | 0.0741 | 0.0821 | 0.2182 | 0.0983 | 0.0083 | 0.0291 |
|  | B24 | 0.1565 | 0.0618 | 0.0461 | 0.1485 | 0.0670 | - | 0.0127 |
| CB5 effluent | B5 | 0.0823 | 0.0862 | 0.1412 | 0.0626 | 0.0555 | 0.0697 | 0.0954 |
|  | B18 | - | - | - | 0.0014 | 0.0047 | - | - |
|  | B28 | 0.0832 | 0.0652 | - | 0.0169 | - | - | - |
| Creek Molen- | B6 | 0.0965 | 0.0912 | 0.0676 | 0.0116 | 0.0071 | 0.2032 | 0.0434 |
| beek before | B19 | 0.0751 | 0.0329 | 0.0639 | 0.0127 | 0.0218 | 0.1129 | - |
| Plant | B29 | 0.0919 | 0.0428 | 0.0479 | 0.0127 | 0.0156 | 0.1489 | - |
| Creek Molen- | B7 | 0.0339 | 0.0316 | 0.0329 | 0.0043 | 0.0053 | 0.0534 | 0.0476 |
| beek after | B20 | 0.0741 | 0.0435 | 0.0820 | 0.0046 | 0.0212 | 0.0904 | 0.0364 |
| Plant | B30 | 0.0384 | 0.0197 | 0.0299 | 0.0082 | 0.0095 | 0.0404 | 0.0543 |
| WWTP | B8 | 0.0012 | 0.0056 | 0.0411 | 0.0523 | - | - | 4.0302 |
| Waregem | B15 | 0.0358 | 0.0434 | 0.0549 | 0.0484 | 0.0143 | - | 1.5082 |
|  | B25 | 0.0230 | 0.0342 | 0.0483 | 0.0561 | 0.0145 | - | 1.2574 |
| Creek Gaver- | B9 | 4.3845 | 0.1661 | 0.0813 | 1.2174 | 0.1184 | 0.0021 | - |
| beek before | B16 | 0.4013 | 0.0819 | 0.0633 | 0.2393 | 0.0328 | - | 0.0671 |
| Plant | B26 | 1.0214 | 0.2738 | 0.0560 | 0.3285 | 0.0389 | - | - |
| Creek Gaver- | B10 | 1.3432 | 0.0489 | 0.0583 | 0.4122 | 0.0526 | - | 0.9382 |
| beek after | B17 | 0.1007 | 0.0471 | 0.0516 | 0.0815 | 0.0145 | - | 1.5562 |
| Plant | B27 | 0.1630 | 0.0802 | 0.0474 | 0.1023 | 0.0283 | - | 1.1931 |

Annex 12. Sampling campaign in N-Italy (I). 19 November 2003.

| Sample Name | Place/City | Time | pH | Comment |
| --- | --- | --- | --- | --- |
| I1 | WWTP effluent Fino Mornasco | 10.20 | 7.2 | Colored |
| I2 | River Seveso after WWTP F.M. | 10.25 | 7.6 | Colored |
| I3 | River Seveso before WWTP F.M. | 10.30 | 7.9 |  |
| I4 | WWTP effluent Bulgarograsso | 11.40 | 7.3 | No water in receiving river Lura |
| I5 | River Seveso in Cusano Milanino | 13.20 | 8.0 | River Seveso before Milano |
| I6 | River Lambro before WWTP in Monza / Brugherio | 14.50 | 8.2 | WWTP at CNR-IRSA |
| I7 | River Lambro after WWTP in San Maurizio al Lambro | 15.15 | 8.0 |  |
| I8 | River Lambro in Milano, Parco Lambro | 15.40 | 8.1 |  |

Annex 13. Sampling campaign in N-Italy. Polar organic compounds. [g/L]

|  | LOD | I1 | I2 | I3 | I4 | I5 | I6 | I7 | I8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2,4-Dinitrophenol |  | 0.0443 | 0.0342 | 0.0099 | 0.0229 | 0.0481 | 0.0875 | 0.0353 | 0.0393 |
| Nitrophenol |  | 0.1634 | 0.1508 | 0.0765 | 0.1483 | 0.0582 | 0.1333 | 0.0369 | 0.0620 |
| Bisphenol A |  | 0.0050 | - | 0.0357 | - | 0.1750 | 0.0550 | 0.0495 | 0.0740 |
| 4-Nonylphenol |  | 0.3659 | 0.4555 | 0.4975 | 0.5324 | 0.7022 | 0.4784 | 0.5366 | 0.6499 |
| 4-Octylphenol |  | 0.0135 | 0.0155 | 0.0877 | 0.0112 | 0.0122 | 0.1108 | 0.0191 | 0.0201 |
| NPE1C | 0.0020 | 2.7780 | 2.1594 | 0.2747 | 4.5411 | 1.5999 | 1.1840 | 2.2151 | 1.9171 |
| NPE2C |  | 0.3593 | 0.2593 | 0.0255 | 0.9093 | 0.3722 | 0.1053 | 1.3378 | 0.7376 |
| NPE3C |  | 1.3625 | 1.0397 | 0.0173 | 2.6434 | 0.8001 | 0.2377 | 0.7619 | 0.4643 |
| OPE1C | 0.0034 | 0.0015 | - | 0.0078 | 0.0011 | - | 0.0011 | 0.0012 | - |
| OPE2C |  | 0.0430 | 0.0335 | 0.0054 | 0.1259 | 0.0972 | 0.0084 | 0.4243 | 0.2695 |
| Benzafibrate |  | 0.0243 | 0.0182 | 0.0035 | 0.0152 | 0.0866 | 0.0245 | 0.1354 | 0.1634 |
| Ibuprofen |  | 0.0000 | 0.0019 | 0.0015 | 0.0000 | 0.0277 | 0.0092 | 0.0198 | 0.0275 |
| Diclofenac |  | 0.0000 | 0.0093 | 0.0075 | 0.0000 | 0.4663 | 0.0263 | 0.1257 | 0.1005 |
| Gemfibrozil |  | 0.0138 | 0.0114 | 0.0028 | 0.0083 | 0.0888 | 0.0179 | 0.1103 | 0.0901 |
| Carbamazepine |  | - | - | - | - | 0.0390 | 0.0074 | 0.0522 | 0.0382 |
| Uniblue A |  | - | - | - | - | - | - | 0.0153 | 0.0069 |
| Sulforhodamine G |  | - | - | - | - | 0.0642 | - | - | - |
| Sulforhodamine B | 0.0026 | 0.1401 | 0.1053 | 0.0369 | 0.2435 | 0.7516 | 0.0025 | 0.0940 | 0.0804 |
| Acid Orange 8 | 0.0042 | - | - | 0.0037 | - | 0.0606 | - | - | - |
| C12-LAS | 0.0021 | 1.3634 | 2.8485 | 9.8858 | 2.0118 | 16.387 | 10.086 | 5.4397 | 29.181 |
| PCP |  | 0.0059 | 0.0049 | - | 0.0062 | 0.0246 | - | 0.0176 | 0.0056 |
| NPE1O |  | 0.6789 | 0.7849 | 0.3134 | 1.3372 | - | 0.2993 | 1.2734 | 0.5969 |
| NPE2O | 0.0088 | 0.3297 | 0.3114 | 0.3424 | 0.9852 | 0.8265 | 0.5430 | 3.6336 | 2.1865 |
| NPE3O |  | 0.3797 | 0.3960 | 0.4717 | 0.8122 | 0.9484 | 0.3900 | 3.2604 | 2.7949 |
| OPE2O | 0.0023 | 0.0017 | 0.0010 | 0.0027 | 0.0022 | 0.0504 | 0.0187 | 0.0926 | 0.0451 |
| Simazine |  | - | - | 0.0090 | - | 0.0123 | 0.0116 | 0.0188 | 0.0097 |
| Atrazine |  | - | - | 0.0035 | - | 0.0074 | 0.0122 | 0.0316 | 0.0102 |
| Terbutylazine |  | 0.0048 | 0.0012 | - | 0.0094 | 0.0161 | 0.0589 | 0.0264 | 0.0254 |
| Linuron |  | - | - | - | - | - | - | 0.2455 | 0.1649 |

Annex 14. Concentrations of EDCs in the Belgian (B) and Italian (I) water samples. Concentrations in g/L, (-) means below MDL.

| B1 | Textile company effluent | B4 | River Schelde, after WWTP | B7 | Creek Molenbeek after plant |
| --- | --- | --- | --- | --- | --- |
| B2 | WWTP effluent (River Schelde) | B5 | Textile plant effluent | B8 | WWTP effluent (Creek Gaverbeek) |
| B3 | River Schelde, before WWTP | B6 | Creek Molenbeek before plant | B9 | Creek Gaverbeek before plant |
| B10 | Creek Gaverbeek after plant |  |  |  |  |
| I1 | WWTP effluent Fino Mornasco (F.M.) | I4 | WWTP effluent Bulgarograsso | I7 | River Lambro after WWTP in San Maurizio |
| I2 | River Seveso after WWTP F.M. | I5 | River Seveso in Cusano Milanino |  | al Lambro |
| I3 | River Seveso before WWTP F.M. | I6 | River Lambro before WWTP in Monza | I8 | River Lambro in Milan, Parco Lambro |

|  | BPA | NP | OP | NPE1C | NPE2C | NPE3C | OPE1C | OPE2C | PCP | NPE1O | NPE2O | NPE3O | OPE2O |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1 | - | 0.26 | - | 0.23 | 0.02 | 0.10 | - | 0.007 | 0.013 | - | 0.05 | 0.18 | 0.006 |
| B2 | - | 0.73 | - | 2.4 | 0.77 | 0.97 | - | 0.18 | 0.031 | 0.19 | 0.38 | 0.52 | 0.044 |
| B3 | 0.038 | 2.5 | - | 1.7 | 0.20 | 0.15 | 0.002 | 0.030 | 0.002 | 0.20 | 0.32 | 0.43 | 0.013 |
| B4 | 0.055 | 2.2 | - | 1.5 | 0.14 | 0.12 | - | 0.022 | 0.003 | 0.21 | 0.17 | 0.40 | 0.005 |
| B5 | 0.006 | 0.25 | - | 0.01 | 0.001 | - | - | - | - | - | 2.1 | 2.9 | 0.043 |
| B6 | 0.028 | 0.99 | - | 1.9 | 0.62 | 0.29 | - | 0.14 | 0.078 | 0.34 | 1.2 | 0.86 | 0.026 |
| B7 | 0.016 | 0.32 | - | 0.41 | 0.15 | 0.06 | - | 0.036 | 0.027 | - | 0.28 | 0.89 | 0.018 |
| B8 | - | 0.58 | - | 0.52 | 0.32 | 1.0 | 0.001 | 0.017 | 0.045 | - | 0.02 | 0.08 | 0.007 |
| B9 | 0.028 | 1.2 | - | 1.1 | 0.22 | 0.08 | - | 0.012 | 0.017 | 0.37 | 0.30 | 1.1 | 0.020 |
| B10 | 0.003 | 0.76 | - | 0.72 | 0.30 | 0.79 | - | 0.019 | 0.036 | - | 0.06 | 0.15 | 0.009 |
| I1 | 0.005 | 0.37 | 0.014 | 2.8 | 0.36 | 1.4 | 0.002 | 0.043 | 0.006 | 0.68 | 0.33 | 0.38 | 0.002 |
| I2 | - | 0.46 | 0.016 | 2.2 | 0.26 | 1.0 | - | 0.034 | 0.005 | 0.79 | 0.31 | 0.40 | 0.001 |
| I3 | 0.036 | 0.50 | 0.088 | 0.28 | 0.03 | 0.02 | 0.008 | 0.005 | - | 0.31 | 0.34 | 0.47 | 0.003 |
| I4 | - | 0.53 | 0.011 | 4.5 | 0.91 | 2.6 | 0.001 | 0.13 | 0.006 | 1.3 | 1.0 | 0.81 | 0.002 |
| I5 | 0.175 | 0.70 | 0.012 | 1.6 | 0.37 | 0.80 | - | 0.097 | 0.025 | - | 0.83 | 0.95 | 0.050 |
| I6 | 0.055 | 0.48 | 0.111 | 1.2 | 0.11 | 0.24 | 0.001 | 0.008 | - | 0.30 | 0.54 | 0.39 | 0.019 |
| I7 | 0.050 | 0.54 | 0.019 | 2.2 | 1.3 | 0.76 | 0.001 | 0.42 | 0.018 | 1.3 | 3.6 | 3.3 | 0.093 |
| I8 | 0.074 | 0.65 | 0.020 | 1.9 | 0.74 | 0.46 | - | 0.27 | 0.006 | 0.60 | 2.2 | 2.8 | 0.045 |

Annex 15. Concentrations of NPEOs in the Italian water samples (g/L).

| I1 | WWTP effluent Fino Mornasco (F.M.) | I4 | WWTP effluent Bulgarograsso | I7 | River Lambro after WWTP in San Maurizio |
| --- | --- | --- | --- | --- | --- |
| I2 | River Seveso after WWTP F.M. | I5 | River Seveso in Cusano Milanino |  | al Lambro |
| I3 | River Seveso before WWTP F.M. | I6 | River Lambro before WWTP in Monza | I8 | River Lambro in Milan, Parco Lambro |

|  | I1 | I2 | I3 | I4 | I5 | I6 | I7 | I8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NPE2O | 0.33 | 0.31 | 0.34 | 0.99 | 0.83 | 0.54 | 3.6 | 2.8 |
| NPE3O | 0.38 | 0.40 | 0.47 | 0.81 | 0.95 | 0.39 | 3.3 | 2.8 |
| NPE4O | 0.088 | 0.12 | 0.31 | 0.18 | 0.47 | 0.20 | 0.56 | 1.5 |
| NPE5O | 0.037 | 0.066 | 0.24 | 0.068 | 0.27 | 0.10 | 0.14 | 1.3 |
| NPE6O | 0.023 | 0.053 | 0.23 | 0.040 | 0.22 | 0.08 | 0.039 | 1.2 |
| NPE7O | 0.018 | 0.045 | 0.21 | 0.028 | 0.19 | 0.050 | 0.024 | 1.2 |
| NPE8O | 0.013 | 0.041 | 0.20 | 0.022 | 0.19 | 0.040 | 0.019 | 1.2 |
| NPE9O | 0.009 | 0.035 | 0.18 | 0.017 | 0.17 | 0.030 | 0.013 | 1.1 |
| NPE10O | 0.008 | 0.033 | 0.17 | 0.014 | 0.17 | 0.026 | 0.010 | 1.1 |
| NPE11O | 0.008 | 0.031 | 0.16 | 0.012 | 0.17 | 0.023 | 0.010 | 1.1 |
| NPE12O | 0.007 | 0.031 | 0.16 | 0.011 | 0.18 | 0.022 | 0.008 | 1.1 |
| NPE13O | 0.007 | 0.028 | 0.15 | 0.011 | 0.19 | 0.021 | 0.009 | 1.1 |
| NPE14O | 0.008 | 0.031 | 0.14 | 0.016 | 0.20 | 0.020 | 0.009 | 1.1 |
| NPE15O | 0.012 | 0.027 | 0.14 | 0.013 | 0.23 | 0.022 | 0.012 | 1.2 |
| NPE16O | 0.011 | 0.029 | 0.16 | 0.012 | 0.26 | 0.023 | 0.010 | 1.3 |
| NPE17O | 0.009 | 0.029 | 0.16 | 0.022 | 0.34 | 0.025 | 0.014 | 1.5 |
| Sum | 1.0 | 1.3 | 3.4 | 2.3 | 5.0 | 1.6 | 7.8 | 22.6 |

Van den Belt K., Wester P.W., Van de Ven L., Verheyen R. and Witters H. (2002). Effects of ethynylestradiol on the reproductive physiology in zebrafish (Danio rerio): time-dependency and reversibility. Environm. Toxicol. Chem. , 21 (4), 767-775

- Van den Belt K., Verheyen R. and Witters H. (2003). Comparison of vitellogenin responses in zebrafish and rainbow trout following exposure to environmental estrogens. Ecotoxicology and Environmental Safety 56 (2), 271-281